3GPP TSG-WG SA2 Meeting #163	S2-2406181

Jeju, South Korea, May27th - 31th, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3, Update of Solution#24 to answer questions from NWM

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: It is proposed to update Sol#24 to answer questions from NWM.

# 1. Discussion

Based on NWM discussion, some comments point out that assigning port number has assumption and impact on EAS. From our investigation, it seems with the clarification on the usage of the tunnel between the L-PSA and EAS, and tunnel between PSA and AS, it can also achieve same target with less impact on the EAS.

It is proposed to update the option 1 to indicate that a GRE tunnel can be established between the PSA and AS for traffic routing, and IP replacement is not needed.

As described in TS 29.561[14], GRE protocol can be used for N6 interface. As described in RFC 2890[15], Key Field is specified in GRE header to define a logical traffic flow between encapsulator and decapsulator. The L-PSA and EAS can set the Key Field of the traffic to different value per different PDU session, so that the L-PSA can link traffic between EAS and AS to PDU session based on the Key Field in the GRE header.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS)".

[5]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[6]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[7]	IETF RFC 792: "Internet Control Message Protocol".

[8]	IETF RFC 4443: "Internet Control Message Protocol (ICMPv6) for the Internet Protocol Version 6 (IPv6) Specification".

[9]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[10]	IETF RFC 4656: "A One-way Active Measurement Protocol (OWAMP)".

[11]	IETF RFC 5357: "A Two-Way Active Measurement Protocol (TWAMP)".

[12]	IETF RFC 8762: "Simple Two-Way Active Measurement Protocol".

[13]	IETF RFC 9484: "Proxying IP in HTTP".

[14]	3GPP TS 29.561: "Interworking between 5G Network and external Data Networks".

[15]	IETF RFC  2890: "Key and Sequence Number Extensions to GRE".

# * * * * Second change * * * *

6.24	Solution #24: Support traffic routing between local-DN and central-DN via the existing UP path of the PDU session and IP replacement

6.24.1	Key Issue mapping

This solution is proposed to solve KI#3. It is the merged solution on using the PDU session to convey the traffic between local DN to central DN the existing UP path of the PDU Session.

6.24.2	General Description

This solution resolves KI#3 about traffic routing between L-DN and central DN based on the scenario that the application traffic is required to be processed at different locations.

Based on AF traffic influence request information, the 5GC determines to enable the traffic routing between L-DN and central DN via the tunnel of PDU session.

-	For the UL traffic, the UE generates traffic and send it to the EAS (i.e. target IP address is EAS IP address), the traffic is routed to EAS to be processed firstly. Then after processed by EAS, the EAS generates the traffic and send it to the AS (i.e. the cloud server) for further processing.

-	For the DL traffic, the AS (cloud server) generates traffic based on the received UL traffic and send it back to the EAS.

-	For UL/DL traffic between EAS and AS, L-PSA also perform IP replacement (if required).

For the traffic sent from EAS to cloud AS, there are three options (option 1: sol#18; option 2: sol#17; option 3:sol#19) on how to handle the user plane traffic at L-PSA, including the IP replacement as described in Table 6.24.2-1, and traffic rules configured at the UL CL/BP as described in Table 6.24.2-2. UL traffic is the traffic from EAS to AS and DL traffic is the traffic from AS to EAS.

Table 6.24.2-1: Traffic between local DN and central DN for each option

| Options | UL Traffic generated by EAS | How L-PSA associate the UL traffic with PDU session | How L-PSA performs IP replacement for UL traffic | DL Traffic generated by AS | How PSA associate the DL traffic with PDU session | How L-PSA performs IP replacement for DL traffic |
| --- | --- | --- | --- | --- | --- | --- |
| Option 1 (sol#18) | Source IP: EAS IP Target IP: AS IP | Based on IP 5-tuple or GRE header (NOTE 3) | Replace the source IP to UE IP2. (NOTE 4, NOTE 7) | Source IP: AS IP Target IP: UE IP 2 or UE IP (NOTE 7) | Based on UE IP 2 (target IP address) or GRE header | Replace the target IP to EAS IP (NOTE 7) |
| Option 2 (sol#17) | Source IP: UE IP Target IP: EAS IP or AS IP | Based on UE IP (source IP address) | None. | Source IP: AS IP Target IP: UE IP | Based on UE IP (target IP address) | None. |
| Option 3 (sol#19) (NOTE 1) | Source IP: EAS IP 2 (NOTE 2) Target IP: UE IP | Based on UE IP (target IP address) | Replace the Source IP to UE IP address, and Target IP to AS IP address. | Source IP: AS IP 2 Target IP: UE IP | Based on UE IP (target IP address) | Replace the target IP to EAS IP. Replace the source IP to AS IP. |
| NOTE 1: For option 3, to align with the existing mechanism, it is suggested not to perform IP replacement at ULCL/BP. NOTE 2: EAS IP 2 is an IP address assigned by EAS, and is different from the IP address EAS used to communicate with UE. NOTE 3: The IP 5-tuple is the IP 5-tuple of traffic between EAS and AS (i.e. EAS IP address, port number and AS IP address, port number) NOTE 4: UE IP 2 is an IP address assigned by PSA, and is different from the UE IP included in the UL traffic. NOTE 5: It is assumed that there is a tunnel configured between the L-PSA and EAS. NOTE 6: AS IP 2 is an IP address assigned by AS, and is different from the IP address of AS used to communicate with UE. NOTE 7: If the related information is conveyed via the GRE header, then the IP replacement and allocation of UE IP2 address is optional. |  |  |  |  |  |  |

Table 6.24.2-2: Traffic routing rule configured at ULCL/BP for each option

| Options | Traffic from L-PSA | Traffic from PSA |
| --- | --- | --- |
| Option 1 (sol#18) | Target IP is UE IP address: Route to UE (RAN). Others: Route to PSA. | Target IP is UE IP address: Route to UE (RAN). Others: Route to L-PSA. |
| Option 2 (sol#17) | Target IP is UE IP address: Route to UE (RAN). Others: Route to PSA. | Source IP is AS IP: Route to L-PSA for specific source AS IP Others: Route to UE (RAN). |
| Option 3 (sol#19) | Target IP is UE IP address: Route to UE (RAN). Others: Route to PSA. | Source IP is AS IP 2: Route to L-PSA Others: Route to UE (RAN). |

NOTE 1:	Option 2 and 3 assume the EAS can do IP replacement for the UL and DL traffic.

NOTE 2:	Option 3 assumes the AS can assign two IP addresses, one is for directly communicate with UE and the other is for communicate with EAS.

NOTE 3:	Option 3 assumes EAS IP 2 is preconfigured in 5GC for traffic detection.

6.24.3	Procedures

2. Packet detection and optional IP replacement configuration

2. Packet detection and optional IP replacement configuration

1. AF request for local traffic routing

1. AF request for local traffic routing

UE

SMF

PCF

ULCL/BP

L-PSA

PSA

EAS

AF/NEF

AS

(cloud server)

3. traffic routing rule configuration

5.Traffic from L-DN to central DN

6.Traffic from central DN to L-DN

4. Notify AF the user plane has been configured

0b. Traffic from UE

7.Traffic from EAS to UE

0a. PDU session establishment

UE

SMF

PCF

ULCL/BP

L-PSA

PSA

EAS

AF/NEF

AS

(cloud server)

3. traffic routing rule configuration

5.Traffic from L-DN to central DN

6.Traffic from central DN to L-DN

4. Notify AF the user plane has been configured

0b. Traffic from UE

7.Traffic from EAS to UE

0a. PDU session establishment

Figure 6.24.2-1: High level procedure for supporting traffic routing between local DN and central DN via PDU session

Traffic from UE to EAS via the existing procedure

0.	The UE establishes a PDU Session as described in clause 4.3.2.2.1 of TS 23.502 [3]. The UE does the DNS query and finds the EAS. The UL CL/BP is inserted per existing procedure as defined in clause 6.2.3 of TS 23.548 [5]. UE send the traffic to the EAS, i.e. the target IP address is EAS IP address, and UL CL/BP route the traffic to L-PSA.

Traffic from EAS to central AS (i.e. Cloud server) is routed via the tunnel of PDU session

1.	The AF requests the SMF to perform traffic routing between local DN and central DN via the existing AF traffic influence procedure.

Following information are included in the information sent from AF to PCF: indication for traffic routing between local DN and central DN, traffic flow description for UL and DL (e.g. IP 5-tuples), target PDU session (e.g. associated UE IP address), QoS requirement of the indicated traffic flow.

Editor's note:	Whether other IEs should also be included in the AF request is FFS.

2.	Based on AF request in step 1, the SMF configures UPF(s) (i.e. L-PSA and/or PSA) to detect traffic related to one dedicated PDU session (e.g. based on traffic flow), and performs IP replacement (if needed) at L-PSA. The information for SMF to instruct to L-PSA/PSA is per the information described in Table 6.24.2-1. This step includes the following handling:

If traffic is link to PDU session based on IP 5-tuple and IP replacement is performed:

For UL traffic between local DN and central DN sent by EAS:

2a.	The SMF configures the L-PSA to detect the indicated traffic flow and associate the indicated the traffic flow with the PDU session per the received traffic flow description (i.e. IP 5-tuple) and target PDU session information.

2b.	The SMF configures the L-PSA to perform IP replacement for UL traffic.

For DL traffic between local DN and central DN sent by AS:

2c.	The SMF configures the PSA to detect the indicated traffic flow and associate the indicated the traffic flow with the PDU session per the received traffic flow description and target PDU session information.

2d.	The SMF configures L-PSA performs IP replacement for DL traffic.

If traffic is link to PDU session based on GRE header:

For UL traffic between local DN and central DN sent by EAS:

2a.	The SMF configures the L-PSA to detect the indicated traffic flow and associate the indicated the traffic flow with the PDU session per the received traffic flow description (i.e. GRE header of the GRE tunnel) and target PDU session information.

NOTE 0: The L-PSA sets the keys in GRE header to UE IP address additionally. Thus, the EAS also sends traffic between EAS and AS to L-PSA with the same key value, e.g. UE IP address, in GRE header. and the L-PSA can detect which PDU session the traffic related to base on the key IE in the GRE header.

For DL traffic between local DN and central DN sent by AS:

2c.	The SMF configures the PSA to detect the indicated traffic flow and associate the indicated the traffic flow with the PDU session per the received traffic flow description and target PDU session information.

NOTE 0a: Similar with UL traffic, the PSA and AS can send traffic between EAS and AS with the same key value (e.g. UE IP) to link traffic with PDU session. Or IP replacement can be performed at L-PSA to enable traffic routing between PSA and AS.

NOTE 1:	If UE IP 2 is used as described in Table 6.24.2-1, SMF also needs to request another UE IP address from PSA before step 2b.

3.	The SMF configures the traffic routing rule on ULCL/BP to route traffic between L-DN and central DN. The information for SMF to instruct to UL CL/BP is detailed in Table 6.24.2-2.

4.	The SMF notifies AF the traffic routing path between L-DN and central DN has been configured via PCF and/or NEF as described in clause 4.3.6.3 of TS 23.502 [3].

5.	Traffic from L-DN (i.e. after processed by EAS) is sent to central DN via the tunnel of PDU session (i.e. via L-PSA, ULCL/BP and PSA).

The L-PSA determines which PDU session to convey the traffic per N4 rule configured in step 2. And the UL-CL/BP routes the traffic to PSA per the N4 rule configured in step 3.

6.	Traffic from central DN (i.e. after processed by cloud server) sent to L-DN via the tunnel of PDU session (i.e. via PSA, ULCL/BP and L-PSA).

The PSA determines which PDU session to convey the traffic per the N4 rule configured in step 2. And UL-CL/BP route the traffic to L-PSA per the N4 rule configured in step 3.

7.	Traffic sent from EAS to UE.

The L-PSA determines which PDU session to convey the traffic per UE IP address. And UL-CL/BP route the traffic to RAN per the N4 rule configured in step 3.

NOTE 2:	All the traffic is within the PDU Session, thus operators can identify it with SDF template in a PCC rule and apply appropriate QoS parameter as described in clause 6.3 TS 23.503 [4]. The PCF can generate QoS parameter considering the QoS requirement required by AF.

NOTE 3:	All the traffic is within the PDU Session, thus operators can identify it with SDF template in a PCC rule and apply appropriate charging key and charging method (e.g. neither of online or offline charging) as described in clause 6.6 TS 23.503 [4]. There can also be Sponsor Identifier if it is charging for 3rd party as described in clause 6.6 TS 23.503 [4].

6.24.4	Impacts on services, entities and interfaces

SMF (applies for all three options):

-	Generate traffic routing rule on ULCL/BP and PDR, FAR on L-PSA/PSA based on AF influence as described above.

-	Inform AF the routing path has been configured.

NOTE:	Details of the traffic routing rule, PDR, FAR are different for three options and described in UPF impacts.

L-PSA:

-	detect traffic between local DN and central DN (based on IP 5-tuple for option1, based on UE IP for option 2,3) and route the traffic to ULCL/BP.

-	For option 1,3: Perform IP replacement (if needed) for traffic between local DN and central DN.

ULCL/BP:

-	Route traffic from EAS to AS based on Table 6.24.2-2.

PSA:

-	For option 1: assign UE IP 2 for IP replacement (if IP replacement option is used).

AF (applies for all three options):

-	inform 5GC to configure the traffic between local DN and central DN.