3GPP TSG-WG SA2 Meeting #163	S2-2406229

Jeju, South Korea, 27 - 31 May, 2024

Source:	Ericsson

Title:	KI#3 Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Provides evaluation of Solutions for KI#3 and the associated conclusion

# 1. Introduction

The KI#3 is formulated as below:

This key issue aims at addressing the following points:

-	Study whether, how, and what PDU Set QoS information can be used for DSCP marking on the outer header of downlink packets of the PDU Set over N3/N9 in the transport network (i.e. to enable differentiated handling of transport packets carrying PDU Sets within QoS Flow).

As can be seen from the above, the ‘whether’, i.e., the clear justification and not merely the ‘how’ must be considered in case a normative work shall be considered.

# 2. Text Proposal

It is proposed to introduce the following changes vs. TR 23.700-70.

# * * * * First change all new text* * * *

# 7	Overall Evaluation

Editor's note:	This clause provides evaluations of different solutions.

## 7.3	Evaluations for Key Issue #3

PDU Set Information based DSCP markings per QoS Flow, solution 13 and 27:

In these solutions the use of PDU Set Importance/Information deviates from the Rel-18 baseline where PDU Set Importance can/may be considered by NG-RAN for dropping of PDU Sets within a QoS Flow in two ways:

- The differentiation is proposed to be considered on PDU basis instead of PDU Set basis;

- PDUs from different QoS Flows will be subject to potential dropping in a non-deterministic manner. Non-deterministic as it cannot be assumed that importance or other PDU Set Information used for DSCP marking set by different ASs is (a) coordinated and (b) has the same meaning wrt QoE for different applications. Also, there may be AS that do not set importance at all and would be potential victim to that approach.

The solutions do not describe benefits nor do they provide the issues they addresses, hence they miss the ‘whether’ aspect of the KI#3.

Evaluation result: Based on the evaluation above we do not support inclusion of these solutions in the normative work.

# * * * * Next changes * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusions for Key Issue #3

No normative work is needed for KI#3.

# * * * * End of changes * * * *