3GPP TSG-SA WG2 Meeting #163	S2-2406289

27–31 May 2024, Jeju, South Korea

Source:	NEC

Title:	KI#2 Conclusions update

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: The contribution proposes updates to KI#2 "Subscription and policy control to support energy efficiency and energy saving as service criteria".

1. Introduction

This contribution proposes updates to the interim conclusions of KI#2 "Subscription and policy control to support energy efficiency and energy saving as service criteria" based on  NWM discussion output.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-66 v0.6.0.

* * * Start of Change * * * *

## 8.2	Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

For KI#2, the following enhancements on subscription and policy control are agreed as interim conclusions to support energy efficiency and energy saving as service criteria in the 5GS:

-	The following are supported:

-	Energy saving authorization information to allow the network to perform energy saving for the UE.

-	Energy related credit per UE and per Network Slice, optionally with validity conditions (e.g. validity time and/or area).

-	The energy credit information is stored as part of subscription information in the UDM/UDR.

Editor's note:	It is FFS whether the energy saving authorization information per UE is needed.

-	The PCF may take into account the following energy related information including subscription data when making policy decisions on the AM, SM and UE policies:

-	energy related credits (e.g., energy consumption limits) per UE and per network slice provided by the UDM/UDR;

-	validity information (e.g. validity time and/or area) associated with the energy credits;

-	energy related analytics provided by the NWDAF;

-	other energy related information, e.g. energy consumption, energy efficiency;

Editor's note:	It is FFS what information in the AM, SM and UE policies can be determined and provided/updated for enforcement based on energy related information or subscription data.

Editor's note:

Editor's note:	Whether energy related information (e.g. energy consumption) per Application ID, UE, PDU Session or QoS flow can be considered by the PCF for policy control is FFS, which depends on whether the information can be obtained as concluded in KI#1.

Editor's note:	Whether the interaction with CHF for policy enforcement and charging is FFS.

* * * End of Change * * * *