SA WG2 Meeting #163	S2-2406371

Jeju, Korea, May 27 – 31, 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	KI#1 conclusion update

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper proposes to update conclusion for key issue #1.

Discussion

This paper proposes to update conclusion for key issue #1.

Proposal

It is proposed to agree the following changes into TR 23.700-32.

* * * * Start of 1st Change * * * *

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

The following bullets are the interim conclusion principles for KI#1:

-	The User Identifier format is NAI.

-	The User Identity Profile may contain:

-	one or more User Identifiers;

-	a list of linked subscriptions.

-	Service specific settings.

-	QoS information (5QI and ARP)

-	Allowed DNNs

-	Allowed operator services (e.g. Voice, SMS, etc)

-	The User Identity Profile is stored in the UDR.

-	The User Identity Profile can be provisioned by the AF using NEF parameter provisioning procedure.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	The User Identifier is activated based on Registration Request message including User Identifier provided by the UE. When the AMF received User Identifier, the AMF provides the User Identifier to the UDM. The UDM enforces only one User Identifier is activated at a given time.

-	When a User Identifier is activated, the AMF ensures that a UE's ongoing services are terminated/disabled

-	by triggering PDU Session release procedure.

-	by triggering Deregistration procedure for SMS over NAS (i.e. deactivation of the UE's SMS service).

-	When a User Identifier is deactivated, the AMF ensures that a user's ongoing services are terminated/disabled

-	by triggering PDU Session release procedure.

-	by triggering Deregistration procedure for SMS over NAS (i.e. deactivation of the user's SMS service).

-	When the UE with active User Identifier establishes PDU Session, the AMF provides activated User Identifier to the SMF and SMF provides the User Identifier to the PCF.

-	The PCF may generate PCC rules for activated User Identifier by retrieving User Identifier Profile from the UDR.

* * * * End of Changes * * * *