SA WG2 Meeting #163	S2-2406524

# Jeju, South Korea, May 27th – May 31st, 2024	 		(revision of S2-240)

Source: 	Nokia

Title: 	Conclusions for KI#1.2

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper provides some conclusions for KI#1.2.

# 1	Discussion

This paper proposes conclusions on KI#1.2

# 2 Proposal

It is proposed to update TR 23.700-54 as follows:

* * * * First Change * * * *

## 8.1	Conclusions for DualSteer

### 8.1.x2	 Conclusions for KI#1.2

Below principle are agreed for KI#1.2:

1.	Each UE/SUPI performs separate registration procedure and includes "Reg-ID" assigned by the DualSteer layer in Registration request.

2.	The "Reg-ID" is further forwarded by the AMF to the UDM. The presence of "Reg-ID" in Registration request from the UE also acts as an indication that the UE is used with a DualSteer device.

3.	If the UE has DualSteer subscription then the UDM also provides the additional subscription information as concluded for KI#1.1, in Access and Mobility Subscription data and SMF Selection subscription data to the AMF.

4.	If the network supports DualSteer feature then the AMF sends a support indication to the UE in Registration Accept.

* * * * End of Changes * * * *