SA WG2 Meeting #163	S2-2406525

# Jeju, South Korea, May 27th – May 31st, 2024	 		(revision of S2-240)

Source: 	Nokia

Title: 	Conclusions for KI#1.3

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper provides some conclusions for KI#1.3.

# 1	Discussion

This paper proposes conclusions on KI#1.3

# 2 Proposal

It is proposed to update TR 23.700-54 as follows:

* * * * First Change * * * *

## 8.1	Conclusions for DualSteer

### 8.1.x3	 Conclusions for KI#1.3

Below principle are agreed for KI#1.3:

1.	Two separate PDU Session establishments are performed for the two SUPIs of the DualSteer device and each PDU Session uses its own PDU Session ID.

2.	During PDU Session establishment the UE includes below additional information:

-	A unique identifier assigned or associated with the UE/SUPI (e.g. "Reg-ID" in Solution #1.3)

-	whether switching is required for the requested PDU Session

-	whether the DualSteer device supports simultaneous transmission

-	During the PDU Session establishment of the second SUPI, the PDU Session ID of the first SUPI is also included as the Linked PDU Session ID.

-	Linked SUPI information is not provided in the PDU Session establishment.

3.	DualSteer PDU Session should use below principles, some of which are based on principles of ATSSS:

-	DualSteer rules are provided to UE similar to ATSSS rules.

-	Two steering modes are supported: Active-Standby and Smallest Delay. Smallest Delay steering mode is supported only for DualSteer devices that support simultaneous transmission.

-	PMF measurements are used for Smallest Delay Steering mode.

-	Same anchor SMF and UPF are selected for managing the PDU Session from the two SUPIs of the DualSteer device.

-	Same PCF is selected by the SMF for SM Policy association for the two PDU Sessions of the two SUPIs of the DualSteer device.

* * * * End of Changes * * * *