3GPP TSG-SA WG2 Meeting #163	S2-2406530

Jeju, Korea, May 27 – May 31, 2024

Source: 	Thales(rapporteur)

Title: 	Introduction for conclusions on the architecture principles following the NWM discussions, for KI3

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes Introduction for conclusions on the architecture principles following the NWM discussions, for KI3.

# 1 Discussion

This paper summarizes architecture principles that can be deduced after the 2 rounds of NWM discussion (see https://nwm-trial.etsi.org/#/documents/8860).

Method used is to directly deduce conclusions from observations made, observations being made following choices of the majority (i.e.: the greatest number) of answers.

# 2 Proposal

It is proposed to agree below proposed changes to 23.700-29.

*** First Change (all new text) ***

# 8	Conclusions

## 8.X Introduction for conclusions on the architecture principles

This chapter summarizes architecture principles that can be deduced after the 2rounds of NWM discussion (see https://nwm-trial.etsi.org/#/documents/8860).

Method used is to directly deduce conclusions from observations made, observations being made following choices of the majority (i.e.: the greatest number) of answers.

### 8.X.y Introduction for conclusions on the architecture principles for KI#3

It is proposed to agree on the following principals:

KI3_P1: R19 shall supports UEs served with different interconnected satellites.

KI3_P2: R19 shall focus on IMS voice and video.

KI3_P3: The two parties do not need to belong to the same HPLMN, as long as IMS are interconnected and UEs are served with different interconnected satellites

KI3_P4: P-ANI, user location (access network information) fetched from PCF, N6 breakout point information received from UPF->SMF->PCF can be used to determine possibility of UE-Sat-UE communication.

KI3_P5: Principals described in Sol#43 of the TR to address change of serving satellite.

KI3_P6: IMS AGW on board is not mandatory but should be mandated to be onboard for LI purpose.

KI3_P7: SA3-LI evaluation is required.

KI3_P8: SA2 shall propose a solution with IMS AGW on board.

KI3_P9: Satellite ID need to be exchanged between IMS networks

KI3_P10: UEs end point addresses (N6 termination point, @IP) may also need to be exchanged between IMS networks

KI3_P11: No deployment constraints (on SMF, PCF, P-CSCF) is preferable.

KI3_P12: (relative) Majority of answers are in favour of Opt2, where Opt2 is having full feature in this release, with minimum restrictions.

KI3_P13: Following previous principals, it is proposed to select the architecture principals of the Sol#43 as described in the TR.

*** End Of Change ***