3GPP TSG-SA WG2 Meeting #163	S2-2406545

Jeju, Korea, May 27 – 31, 2024

Source:	Huawei, HiSilicon

Title:	Update Conclusion for KI#1

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract: Conclusion for KI#1 is updated.

1. Introduction/Discussion

The conclusion for KI#1 is proposed to be updated as follows:

- 	Add DNN/S-NSSAI as part of User Identity Profile, considering that the User ID identifies the human user who uses the UE, and the 5GC supports the different service control for the user, and each of the users may use different DNN/S-NSSAI.

- 	Add QoS and Charging policy per DNN/S-NSSAI combination as part of User Identity Profile, which is similar as how QoS and Charging policy are defined for the UE/SUPI.

- 	Add Use ID credential per DNN/S-NSSAI to the User Identity Profile, considering that different security level may be required when the user accesses different DNN/S-NSSAI. Details needs to be further studied in SA3.

- 	Add the subscribed User Identifier list and the activated User Identifier information to the UE subscription data, as proposed in solutions #5, #17, #23 and #24.

2. Text Proposal

It is proposed to capture the following texts vs. TR 23.700-32.

# * * * * First change * * * *

8	Conclusions

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

The following bullets are the interim conclusion principles for KI#1:

Editor’s note: 	The above conclusion principles are only tentatively agreed.

-	The User Identifier format is NAI.

-	The User Identity Profile stored in UDR may contain:

-	one User Identifier;

-	a list of linked subscriptions;

-	subscribed DNN/S-NSSAI and corresponding QoS and charging policy, optionally it may also include the credential for this User Identity per DNN/S-NSSAI combination.

Editor's note:	The User Identity authentication based on the credential stored in User Identity Profile needs further study in SA3.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	The UE subscription data is extended to include:

-  The subscribed User Identifier list which includes the User Identifiers allowed to access the 5GC via this UE, and

-	optionally activated User Identifier information to indicate the related User Identifier is currently activated in this UE.

# * * * * End of changes * * * *