3GPP TSG-SA WG2 Meeting #163	S2-2406570

27 May - 31 May 2024, Jeju, South Korea	(revision of S2-240xxxx)

Source:	CATT

Title:	Conclusion for KI#1: Interim Conclusion for the Alternative PDU Set QoS

Document for:	Discussion/Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: this contribution proposes the interim conclusion for the KI#1 alternative PDU Set QoS.

1.	Introduction

This contribution proposes the interim conclusion for the KI#1 alternative PDU Set QoS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First Change (all new text)* * * *

8.1	Conclusions for Key Issue #1

8.1.1	Interim Conclusion for Alternative PDU Set QoS

It is concluded that:

- An alternative PDU Set QoS profile can be associated with an alternative QoS Profile that is provided to the NG-RAN by the 5GC;

- An alternative PDU Set QoS profile can include the PDU Set QoS parameters as described in clause 5.7.7 of TS23.501[2];

- Different alternative PDU Set QoS profiles have different values of  the PDU Set QoS parameters;

- An alternative PDU Set QoS profile is associated with a QoS/Alt-QoS indicator;

- The UPF marks the corresponding QoS/Alt-QoS indicator in the GTP-U HE based on the N4 PDR from the SMF;

- The NG-RAN selects an associated alternative PDU Set QoS profile for the DL PDU Set packets based on the received QoS/Alt-QoS indicator in the GTP-U HE of the DL PDU Set packets.

- SMF provides the alternative PDU Set QoS profiles and associated QoS/Alt-QoS indicators to the NG-RAN based on the PCC Rules from the PCF.

- SMF provides the PDRs and associated QoS/Alt-QoS indicators to the UPF.

- PCF generates the PCC Rules which include the alternative PDU Set QoS and associated alternative QoS and associated QoS/Alt-QoS indicator based on the AF request on QoS or based on its local configuration.

- If QoS notification control is enabled for alternative QoS profiles, the NG-RAN can select different alternative QoS profiles as described in clause 5.7.2.4 of TS23.501 [2] but cannot select different alternative PDU Set QoS profiles for the packets with the same QoS/Alt-QoS indicator.

- The NG-RAN can provide the QoS notification control report with the selected alternative QoS profile and an alternative PDU Set QoS profile to the AF via CP or UP.

# * * * * End of change* * * *