3GPP TSG-WG SA2 Meeting #163	S2-2406651

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2404899)

Source:	Lenovo

Title:	KI#3, KI#1, New Solution: AIoT communication for connectivity topology 2 and AIoT data transmission in the UP of PDU Session

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This paper proposes a new solution to KI#1 and KI#3 describing the indirect AIoT communication via UE as an intermediate node (i.e. topology 2) and AIoT data transmission in the UP of a PDU Session.

# 1. Introduction/Discussion

The KI#1 includes the among others the following aspect to be studied:

-	System architecture identified along with the solutions for KI#2 and KI#3.

-	Validation of the Ambient IoT Device identifier;

-	Whether and how to secure device operations and services for an Ambient IoT Device or a group of Ambient IoT Devices;

Further, the Key Issue #3 includes the following aspect to be studied:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	Including whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

There are 2 main communication topologies assumed in the Ambient AIoT study, namely:

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

This solution focuses on the Topology 2. It is proposed that a UE, which is capable of AIoT transmissions towards AIoT devices, indicates its capability during the registration with the network.

This solution proposes a solution for the AIoT information transfer between the AIoT application server and the AIoT devices via the control plane of the 5GS. A new NF called Ambient AIoT gateway (AIoT-GW) is introduced.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  | X |  |
| #2 |  | X |  |
| #3 | X | X | X |
| #4 | X | X | X |
| #5 |  | X | X |
| #6 | X | X | X |
| #7 | X | X |  |
| #8 | X | X |  |
| #9 | X | X | X |
| #10 | X | X |  |
| #11 |  |  | X |
| #12 | X | X | X |
| #X | X |  | X |

# * * * * Second change * * * *

## 6.x	Solution #X: AIoT communication for connectivity topology 2 and AIoT data transmission in the UP of PDU Session

### 6.x.1	Key Issue mapping

This solution addresses the following requirements:

-	from Key Issue #1 the requirement:

-	System architecture identified along with the solutions for KI#2 and KI#3.

-	from Key Issue #3 the requirements:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	Including whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

### 6.x.2	Functional Description

#### 6.x.2.1	General

This solution focuses on the Topology 2. A UE, which is capable of AIoT transmissions towards AIoT devices, is called "UE AIoT Reader". The UE AIoT Reader indicates its AIoT capability during the registration with the network.

This solution proposes that the AIoT data/information transfer between the AIoT application server (AIoT AS) and the AIoT devices is executed via the User Plane (e.g. UP) of a PDU Session. A new NF called Ambient AIoT function (AIoTF) is introduced which may be located the Control Plane (CP) and/or in the UP (e.g. on the N6 LAN). The assumed architecture is shown in Figure 6.X.2.1-1.

Figure 6.X.2.1-1:  Assumed architecture for AIoT services using topology 2 and and AIoT data transmission in the UP of a PDU Session

The following principles are proposed:

-	During the UE AIoT Reader registration procedure:

-	the UE AIoT Reader indicates its AIoT capabilities to the AMF and the AMF registers the UE AIoT Reader with the AIoTF.

-	The AMF or AIoTF-CP may configure the UE AIoT Reader to use the U-plane for the AIoT data transmission.

-	The UE AIoT Reader establishes a PDU Session to transmit and receive AIoT data:

-	An AIoT application client resides at the UE AIoT Reader. The AIoT client performs 1) reception and transmission of AIoT data from/to AIoT-AS via the 5GS and 2) transmission and reception of AIoT data/signalling to/from the AIoT device.

-	The SMF may determine the AIoT configuration data for the PDU Session either from the SM Subscription data or from the SM policy control information received from the PCF.

-	The SMF configures the UPF and the UE AIoT Reader with the appropriate U-plane information to receive and transmit AIoT data from/to the AIoT application server.

-	An AIoTF in the U-plane can be introduced (e.g. called "AIoTF-UP") to perform inspection of the AIoT data and/or perform verification of the AIoT devices.

#### 6.x.2.1	Protocol Stack

#### The Figure 6.X.2.1-1 shows the protocol stack for AIoT communication between the AIoT device and AIoTF-UP. The AIoT data is transmitted over the user plane of a PDU Session established by the UE AIoT Reader.

Figure 6.X.2-1:  Protocol Stack for AF Based Solution (over user plane)

The following is applicable:

-	App layer: the command layer protocol between AIoT devices and AAIoT-AS includes the commands for "inventory" service and the "command" service (e.g. read, write, execute, etc.).

-	UE AIoT layer: this the layer between the AIoTF-UP and the UE Reader and it can be implemented as an IP protocol outside the scope of SA WG2. The UE AIoT layer encapsulates the AIoT device NAS PDUs.

-	AIoT device NAS: the protocol between the AIoTF-UP and AIoT device for authentication or verification of the AIoT device and also carrying the App layer data.

-	Uu AS layer: the same as the current Uu interface.

NOTE 1:	The AIoT device NAS protocol is assumed to be in the scope of SA WG2 (Stage 2 aspects) and CT WG1 (Stage 3 aspects).

### 6.x.3	Procedures

Two procedures are shown in Figure 6.X.3-1 to explain the concept of this solution:

-	the Registraion procedure of the UE AIoT Reader with the network and the AIoTF; and

-	the PDU Session establsihment procedure for the transmission of the AIoT data/signalling via the U-plane.

Figure 6.X.3-1:  Signalling flow of the AIoT session establishment between the UE AIoT Reader and the AIoTF-UP or AIoT-AS in the U-plane

The detailed description of the steps is provided as follows:

0a.	The UE AIoT Reader may implement an AIoT client functionality which is responsible for: 1) reception and transmission of AIoT data/signalling via the U-plane (e.g. from AIoTF-UP or AIoT-AS), and 2) transmission and reception of AIoT data/signalling to/from the AIoT devices.

0b.	The UDM/UDR stores:

-	UE AIoT Reader subscription data which may include the UE authorisation to act as AIoT intermediate node, a list of AIoT service(s) for which is allowed to transmit AIoT data/signalling to the AIoT devices, etc.

-	AIoT application subscription data contains the configuration information for the AIoT application including credentials.

0c.	The AIoT-AF may use parameter provisioning service to confiure or update the AIoT U-plane configuration for a specific AIoT service (idenitified by AIoT service ID).

1.	The UE sends a Registration Request message which includes 1) indication that the UE is capable of AIoT communication serving as intermediate node, 2) whether the UE can receive and transmit AIoT data from/to the network via CP or UP communication path, 3) AIoT capabilities (e.g. supported frequency bands, range of communication or AIoT radio coverage area, one or more power modes.

2.	The AMF can retrieve the UE’s subscription data from the UDM/UDR. The UDM/UDR may indicate whether the UE is enabled/disabled to serve as AIoT intermediate node, a list of AIoT service(s) for which is allowed to transmit AIoT data/signalling to the AIoT devices, AIoTF selection information, etc.

3.	The AMF requests AM policy association establishment with the PCF. The PCF may configure AM policy for the UE specific for the operation as UE AIoT Reader.

4.	The AMF sends a Registration Accept message (or UE Configuration Update Command, UCU) which includes an indication whether the operation as UE AIoT Reader is allowed and which AIoT services are allowed to be served.

5a.	The UE AIoT Reader initiates PDU Session establishment. The AMF may determine either based on the combination S-NSSAI/DNN or based on explicit indicaiton, that the PDU Session is for AIoT data.

5b.	The AMF selects an appropriate SMF. If the AMF has AIoT configuration information, the AMF sends it to the SMF.

6.	The SMF retrieves SM subscription data from the UDM, which may inlcude information about the user plane configuration (e.g. AIoT-UP address or AIoT-AS address).

7.	The SMF initiates the establishemnt of SM policy association to the PCF. The PCF may be preconfigured, or retrieve PDU Session subscription information from the UDR, or receive AF steering information in order to determine SM policy whether AIoT-UP address should be condigured in the U-plane.

8.	Optionally, there may be a signalling exchange between the SMF and AIoT-UP entity to configure the user plane for the AIoT data transmission.

9.	The SMF establishes N4 session with the selected UPF.

10.	The SMF sends N2 SM configuration information to the RAN node and N1 SM container with configuration information to the UE.

11.	The UE AIoT Reader may initiate application layer signalling to register with the AIoT-AS or AIoT-UP. During this step the UE and the AIoTF-UP / AIoT-AS may exchange an AIoT session identifier (e.g. AIoT Service ID, UL port ID and DL port ID, etc.) which is used for the correct routing of the AIoT data/signalling in the UE AIoT Reader to one or more AIoT devices.

12.	End-to-end communicatio:

-	In the DL direction: the AIoT-UP transmits AIoT data which is received in the UE AIoT Reader (e.g. in an AIoT client functionality). The AIoT client functionality transmits the AIoT data to the AIoT device(s).

-	In the UL direction: the UE AIoT Reader (e.g. in an AIoT client functionality) receives AIoT data from the AIoT device. The UE AIoT Reader transmits the AIoT data the AIoT-UP.

### 6.x.4	Impacts on existing services, entities and interfaces

The following NFs and interfaces are impacted:

-	AIoTF-UP: optionally, a new function on the N6 interface can be introduced.

-	UE AIoT Reader: 1) supporting AIoT capabilities to communication with the AIoT devices and 2) capabilities to receive configuration information from the SMF to initiate application layer communication with the AIoTF-UP or AIoT-AS.

-	PCF or UDM/UDR: store AIoT configuration information to configure PDU Session for the communicaiton between the UE AIoT Reader and AIoTF-UP / AIoT-AS.

# * * * * End of changes * * * *