3GPP TSG-WG SA2 Meeting #163	S2-2406666

Jeju, Korea, May 27th – 31st, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion of KI #1

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_ Ph3 / Rel-19

Abstract: This pCR add the conclusion for KI#1 in TR 23.700-59

- Introduction

This paper proposes the conclusion of KI #1.

- Discussion

KI #1 studies enhancement of NEF services, which includes the following aspects:

-	whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting i.e.

-	Pre-mission flight planning and in-mission flight monitoring for UAVs.

-	C2 communication reliability.

-	The scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

Eight solutions are agreed for KI #1:

-	Solution #1, #2 and #3: They propose to support service exposure and interactions for flight planning/monitoring, including movement behaviour analytics, flight route deviation determination and QoS sustainability analytics.

-	Solution #10, #11: They propose to enable 5GC support for multiple USS. One is for considering possible UAV border-crossing points, and the other is for updating the serving USS information based on UAV location.

-	Solution #12, #13 and 14: They propose to support C2 communication reliability. Sol#12 and #14 are for dual C2 communication support for E2E reliability. Sol#13 reuses the existing NWDAF analytics for C2 communication QoS analysis.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-77.

# * * * * First change * * * *

8.X	Conclusions of Key Issue #1

The following aspects are concluded as principles for the normative work:

- Flight planning/monitoring:

- The USS/UTM may request the exposure information from 5GC for pre-mission flight planning and in-mission flight monitoring for UAVs, including:

- Request for QoS Sustainability Analytics corresponding to UAV flight path.

- Request for UE Movement Analytics for UAV pre-mission flight planning.

- Request the deviation information between the UAV current location and the planned UAV flight path (indicated by waypoints).

- In order to request 5G network for pre-mission flight planning and in-mission flight monitoring for UAVs, USS/UTM shall provide UAV flight path to the 5G network.

- Multiple USS:

- It is assumed multiple USSes can communicate with each other, through interfaces not defined by SA2.

- Serving USS subscribes to network for UAV location via NEF (UAS NF), and determines USS-relocation based on the notified UAV location and its flight path.

- Relocation of USS requires an authentication and authorization (UUAA) procedure to be performed between the UAV and the new USS.

- C2 communication reliability:

- The USS can add redundant information in C2 aviation payload and C2 authorization payload to UAV.

- The 5GS can support the UAV to set up C2 dual transmission.

- The UAV can set up redundant transmission to support C2 dual connectivity for End-to-End reliability.

# * * * * End of changes * * * *