3GPP TSG-SA WG2#163	S2-2406671

Jeju, Korea, 27-31 May 2024	(was S2-240xxxx)

Source:	Samsung, Nokia

Title:	Conclusions of KI#5

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2/Rel-19

Abstract of the contribution: This paper proposed conclusions on KI#5: Handling of PS data off exemption for services over IMS DC.

# 1.	Discussion

Based on LS Reply from SA1 (S1- 240301) and updated Stage1 requirement is TS 22.011, it is clear that the services over IMS DC is exempted as a whole or not. But when it is exempted then operator can control the usage of application over IMS DC. Based on this, the EN in the KI#5 Description can be removed and resolved by adding a Note in conclusion.

Solution #28 and 29 are documented for handling PS data off exemption for services over IMS DC. Following principles are derived from the above two solutions and proposed as conclusions for KI#5.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-77:

# >>>>BEGINNING OF CHANGES<<<<

## Key Issue #5: Handling of PS data off exemption for services over IMS DC

### 5.5.1	Description

This key issue studies how IMS architecture, procedures need to be enhanced to support PS Data off for services over IMS DC. In particular, this key issue will study:

-	Whether and how to enhance the current 3GPP PS Data Off Exempt service information (e.g. list of 3GPP PS Data Off Exempt services) to support 3GPP PS Data Off for services over IMS data channel.

# >>>>NEXT CHANGE<<<<

## 8.X	Conclusion of KI#5

For KI#5 on "Handling of PS data off exemption for services over IMS DC" the following conclusions are agreed:

1.	"Services over IMS Data Channel" can be configured as a service in the list of 3GPP PS Data Off Exempted Services, i.e. either "Services over IMS DC" is exempted from PS Data Off or is not exempted.

NOTE:	Whether individual DC applications can be downloaded and used by the UE if IMS DC is not exempted from PS data off, is up to implementation.

2.	If IMS DC is not configured as an exempted service and PS Data Off is activated in the UE following applies:

UE enforcement:

- If there is standalone IMS DC session then UE shall terminate standalone IMS DC session.

- If there is IMS DC session including audio and video media, UE shall keep only audio and video media and release IMS DC (both bootstrap DC and application DC).

Network enforcement:

- If there is IMS DC session with audio and video media, IMS AS shall ensure that only audio and video media is permitted.

3.	 If IMS DC is configured as exempted service, none of the services over IMS DC  are impacted when PS Data Off is activated.

# >>>>END OF CHANGES<<<<