3GPP TSG-WG SA2 Meeting #163	S2-2406724

Jeju, Korea, May 26th – 31th, 2024	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#1 conclusion proposal

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: This paper proposes conclusion on KI#1in TR 23.700-63.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-63.

# * * * * First change * * * *

8.1	Interim Conclusions for Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clause 4.17.1 and clause 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	The following UPF functionalities are added in UPF profile stored in NRF:

-	The functionality of NAT information exposure;

-	Packet Inspection functionality (to differentiate between IP or MAC filter based packet detection, and the packet detection based on other means, e.g. layer 7 DPI);

-  Defining operator configurable parameter in the N4 capabilities and UPF NF profile which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities. The format of the parameter is to be determined by stage 3.

-	for example, the operator may configure values for this parameter to represent customized configuration (e.g, hardware accelerators (for example GPU, DPU etc.), Firewall, DDos Protection, etc.)

-	this parameter may also be included in the UE SM subscription which can represent some user plane functionalities that are required and/or desired for the PDU session, for example parent control. In this case this parameter is only valid for non-roaming case.(depending on SoH)

# * * * * End of changes * * * *