3GPP TSG-WG SA2 Meeting #163	S2-2406741

# Jeju, Korea, May 27- May 31, 2024

Source: 	Samsung

Title:	Conclusions for KI#2

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose Conclusions for KI#2.

# 1. Discussion

This proposal proposes the conclusions for KI#2 as baseline for normative work. Mainly it proposes the following aspects:

- Consumer NF which directly subscribes to UPF using UE IP address are extended to include other NFs

- Consumer NF targeting a specific GPSI/SUPI can subscribe directly only if they have information of serving UPF, otherwise the Rel-18 mechanism is used.

- Adding new UPF Event to expose UE’s NATed Public IP address

# 2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE: All text new

## 8.X	Key Issue #2: Enhancements on UPF information exposure

The following aspects are concluded as principles for normative work:

-	Consumers of Nupf_EventExposure Subscribe operation targeting a UE IP address are extended to include AF/NEF.

NOTE:	Restriction of subscribing particular Event ID(s) (like QoS Monitoring Event) indirectly via SMF remains as defined in Rel-18.

-    For subscribing Event ID(s) targeting specific GPSI/SUPI, if the consumer NF (e.g. NEF/AF) have the information of the serving UPF (e.g. UPF ID) for the UE’s PDU Session, the consumer NF can subscribe to UPF Event Exposure service directly via UPF. For subscribing Event ID(s) targeting specific GPSI/SUPI, if the consumer NF (e.g. NEF/AF) do not have the information of the serving UPF (e.g. UPF ID) for the UE’s PDU Session, the consumer NF subscribes to UPF Event Exposure services using the Rel-18 mechanism.

NOTE:	Restriction of subscribing particular Event ID(s) (like QoS Monitoring Event) indirectly via SMF remains as defined in Rel-18.

- UPF Event exposure service is extended to include to following Event ID

  - Event ID : UEPublicIPAddrMonitoring

  - Target : UE private IP address

  - Event Filter : Destination IP address, port number

  - Output : UE public IP address, port number

NOTE:	Any further inputs required for subscribing this Event ID can be decided during normative phase.

END OF CHANGES