SA WG2 Meeting #163	S2-2406764

Jeju, Korea, May 27 – May 31, 2024	(was S2-24XXXXX)

Source:	vivo

Title:	KI#1, Update to sol#22 to clarify the QoS notification detail

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This paper proposes update to solution#22.

1	Proposal

It is proposed to include the following changes in TR 23.700-70.

START OF CHANGES

6.22	Solution #22: The UL and DL PDU Set handling

6.22.1	Key Issue mapping

This solution addresses key issue #1.

6.22.2	Description

As approved in RAN R2-2312136, the PDU Set QoS handling depends on UE’s indication via UAI: when PDU Set identification is possible for a QoS flow, this is indicated to the gNB by the UE.

In the uplink, the UE needs to be able to identify PDU Sets and Data Bursts dynamically, including PSI. How this is done is left up to UE implementation but when possible for a QoS flow, this is indicated to the gNB via UE Assistance Information.

According to the existing procedure at RAN side, the NG RAN can understand whether UL PDU Set handling is possible or not for a QoS flow, but the CN is not aware of the UE supporting UL PDU Set handling or not. E.g. the CN may still send the UL PDU Set QoS parameters to the RAN even though the UL PDU Set handling is not possible. But the UL PDU Set handling is not applicable in this case and the CN has no idea about this situation.

If the AF has subscribed the QoS notification, it is beneficial and needed for CN to understand the UL PDU Set QoS is not fulfilled due to the UL PDU Set handling is not possible and notify the AF.

The principles of this solution are:

-	Alternative1:

-	The NG RAN notifies to the SMF that UL PDU Set handling is possible for a QoS flow when the NG RAN understand the UL PDU Set handling is possible for a QoS flow.

-	The SMF may provide UL PDU Set QoS to the NG RAN upon knowing the UL PDU Set handling is possible for a QoS flow.

NOTE:   Before knowing that UL PDU Set handling is possible for a QoS flow, the SMF doesn’t need to send the UL PDU Set QoS parameter to the NG RAN.

Editor's note:	Alternative1 assumes the UE can indicate UAI to the RAN before RAN receives UL PDU Set QoS. It needs to evaluate whether it is realistic.

-	Alternative2:

-	The SMF provides UL PDU Set QoS to the NG RAN without knowing the UL PDU Set handling is possible for a QoS flow.

-	Upon reception of the UL PDU Set QoS, if the UL PDU Set handling is possible or not possible for a QoS flow, the NG RAN notifies to the SMF the UL PDU Set handling is possible or not possible for a QoS flow. if DL PDU Set handling is possible for a QoS flow, the NG RAN notifies it to the SMF as well.

-	In order to support QoS Notification Control “GFBR cannot be guaranteed or can be guaranteed again” (see clause 5.7.2.4 in 23.501[2]), for a GBR QoS flow with PDU Set QoS parameters, the NG RAN determines the QoS notification based on the PDU Set handling or not in both directions.

a)	If the PDU Set handling is supported in UL and/or DL, in the corresponding direction, instead of PDB and PER, the PSDB and PSER are used to determine QoS notification.

b)	If the PDU Set handling is not supported in UL and/or DL, in the corresponding direction, the received PDU Set QoS parameters are ignored and the PDB and PER are used to determine QoS notification.

-	Based on the received the QoS notification and PDU Set handling support or not indications in UL and DL, the 5GC and AF understands a set of QoS parameters of the QoS profile is fulfilled or not.

a)	If the PDU Set handling is supported in UL and/or DL, the PDB and PER in the corresponding direction are not considered in the QoS parameter set.

b)	If the PDU Set handling is not supported in UL and/or DL, the PSDB and PSER in the corresponding direction are not considered in the QoS parameter set.

NOTE:   If the AF has subscribed “QoS targets can no longer (or can again) be fulfilled”, the CN can notify the received notification and UL/DL indications of PDU Set handling support or not to the AF.

NEXT CHANGES

6.22.4	Impacts on services, entities and interfaces

Alternative1

NG RAN:

-	When the NG RAN knows the UL PDU Set handling is possible for a QoS flow, the NG RAN notifies UL PDU Set handling is possible for a QoS flow to the SMF.

SMF:

-	Upon knowing the PDU Set identification is possible for a QoS flow, provide UL PDU Set QoS to the NG RAN.

NOTE:   Before knowing that UL PDU Set handling is possible for a QoS flow, the SMF doesn’t need to send the UL PDU Set QoS parameter to the NG RAN in this alternative.

UE:

-	No impacts.

Alternative2

NG RAN:

-	Upon reception of the UL PDU Set QoS, if the UL PDU Set handling is not possible for a QoS flow, the NG RAN notifies to the SMF the UL PDU Set is not fulfilled due to UL PDU Set handling is not possible for a QoS flow.

-  Upon reception of the DL PDU Set QoS parameters, the NG RAN indicates to 5GC the PDU Set handling supporting indication in DL.

-  Determine QoS notification based on the PDU Set handling support or not in UL and/or DL if the QoS profile include the UL and/or DL PDU Set QoS.

CN:

-	Notifies to the above QoS notification and PDU Set handling support or not indications in UL and DL to AF if the has subscribed the notification.

-	Based on the received the QoS notification and PDU Set handling support or not indications in UL and DL, the 5GC understands a set of QoS parameters of the QoS profile is fulfilled or not.

a)	If the PDU Set handling is supported in UL and/or DL, the PDB and PER in the corresponding direction are not considered in the QoS parameter set.

b)	If the PDU Set handling is not supported in UL and/or DL, the PSDB and PSER in the corresponding direction are not considered in the QoS parameter set.

UE:

-	No impacts.

END OF CHANGES