SA WG2 Meeting #163	S2-2406767

27 May – 31 May, 2024, Jeju, Korea                           (Revision of S2-2404905)

Source:	InterDigital Inc.

Title:	Key Issue #3, Conclusion Update

Document for:	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes key issue #3 conclusion update.

# 1	Discussion

This p-CR makes 2 updates to the Key Issue #3 conclusion:

Update #1:

As discussed in S2-2406768, the energy consumption that is associated with an NF is highly dependent on what service operations of the NF are invoked. When an NF performs NF Selection, it will likely prefer an NF that consumes relatively little energy for the service operations that are likely to be invoked.

This conclusion update proposes to add a note that says that the granularity (e.g. NF Instance granularity, Service Operation granularity, etc.) of the energy information in the NF profile can be determined during the normative phase.

Update #2:

In the NWM survey, there was consensus on whether energy related information can be static or dynamic. Whether a piece of information is static or dynamic should be decided on a case-by-case basis. It seems too restrictive to say that ALL information must be static or that all information must be dynamic. Thus, this update proposes a note that says that whether each piece of energy related information can be dynamic will be decided during the normative phase.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-66 V0.6.0.

* * * * Start of Changes * * * *

## 8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

Following interim conclusions for KI#3 are proposed:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile is extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

NOTE 1:	What energy related information can be included in the NF profile and the granularity of the included information (e.g. NF Instance granularity, Service Operation granularity, etc.) will be decided during the normative phase.

NOTE 2:	Whether each piece of energy related information can be dynamic will be decided during the normative phase.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

* * * * End of Changes * * * *