3GPP TSG-WG SA2 Meeting #163 	S2-2406924

May 27 – 31, 2024, Jeju Island, Korea	(Revision of S2-2406436)

Source:	Intel (Rapporteur)

Title:	KI#2: Conclusion update based on NWM discussion

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract: This paper proposes to update the conclusion for KI#2 based on NWM discussion.

# 1. Introduction

This contribution proposes conclusion update for KI#2 based on NWM discussion.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

## 8.2	Conclusion for KI#2

The following principles are concluded for normative work:

-	SMF selects local PSA UPF considering N6 delay, when available.

-	Upon receiving the DNS message reporting in step 14 in clause 5.2.3.2.2 of TS 23.548 [5] from EASDF, SMF may take existing NWDAF analytics (i.e. DN performance) into account to select a (L-)PSA UPF supporting lowest N6 delay and/or modify the ECS options in DNS handling rules sent to the EASDF to influence the EAS (re)selection. There will be no impact to DN performance analytics defined in TS 23.288 [10].

NOTE 1: The SMF determines N6 delay using DN analytics based on implementation.

-	SMF collects N6 delay measurement from impacted UPF per pair of 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (e.g. EAS/Designated IP (range)) as following:

- SMF determines the impacted UPF(s) according to the candidate DNAI(s) which can be determined based on the list of DNAI(s) received in AF request.

- SMF determines the measurement protocol based on the protocol supported by UPF and the measurement protocol(s) supported by application side (e.g. Pre-configuration in SMF or via AF request).

- SMF sends the N6 delay monitoring information such as measurement endpoint (e.g. target EAS IP address(es)/Designated IP (range)) per DNAI in the list, the determined measurement protocol and optionally related security information to the impacted UPF.

- UPF(s) performs N6 delay monitoring and reports the N6 delay monitoring result to SMF.

-	5GC is not aware of EAS load.

-	N6 delay between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) is measured by leveraging existing mechanisms (e.g.,  ICMP, TWAMP, OWAMP and other protocols defined by IETF).

-	5GC may enable the N6 delay measurement via AF influence procedure per clause 4.3.6.2 (Processing AF requests to influence traffic routing and/or Service Function Chaining for Sessions not identified by a UE address) in TS 23.502[03] or via EAS Deployment Information Management defined in clause 6.2.3.4 in TS 23.548 [5]. When AF influence procedure is used, in Nnef_TrafficInfluence_Create/Update Request, AF can provide an indication of delay sensitive service, optionally delay requirement, a list of DNAI(s), measurement endpoint (e.g. target EAS IP address(es)/Designated IP (range)) per DNAI in the list, the measurement protocol(s) and optionally the protocol-specific configuration parameters.

NOTE 2: For the enabling of SMF to measure N6 delay aspect, whether one mechanism between AF influence and EDI is enough will be determined in normative phase.

# * * * * End of changes * * * *