3GPP TSG-WG SA2 Meeting #163	S2-2406634

Jeju, Korea, May 27th – 31th, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#8, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_ph2 / Rel-19

Abstract: This paper proposes to add the conclusion section for KI#8 in FS_NG_RTC_Ph2 TR.

# 1. Discussion

Eight solutions were proposed for KI#8: #17, #23, #24, #25, #26, #27, #31, #32.

These solutions have the following similarities:

-	Solutions are based on data channel architecture, a new NF is added to store avatar representation and related data;

-	The avatar representation and related data can be accessed by Avatar-id and user identity;

-	MF/MRF supports avatar representation animation and rending based on audio/video/text or metadata received from UE.

In the following table solutions are classified based on following categories:

-	Which NF connect to DAR;

-	How the UE downloads the avatar representation;

-	Which NF downloads the avatar representation;

-	Which NF controls the avatar service logic;

-	The role of animation/rendering the avatar;

|  | Which NF connect to DAR | How the UE downloads the avatar representation | Which NF downloads the avatar representation | Which NF controls the avatar service logic | The role of animation/rendering the avatar |
| --- | --- | --- | --- | --- | --- |
| Solution #17: Solution for support of IMS avatar communication using data channel | XR AS | over ADC | XR AS | XR AS | UE-A or MF/MRF or UE-B |
| Solution #23: Download avatar representation in bootstrap data channel from avatar repository | DCSF | over BDC | N/A | N/A | N/A |
| Solution #24: Transition Between Avatar and Audio/Video Communication | DCSF | over ADC | DCSF | DCSF | MF/MRF or UE-B |
| Solution #25: Supporting network-based avatar communication by media capability invocation | XR AS | over ADC | MF/MRF | XR AS | MF/MRF |
| Solution #26: Support for Avatars in AR Calls | MF | unclear | unclear | XR AS | MF/MRF or UE-B |
| Solution #27: Early capability negotiation for IMS Avatar Communication | NEF | through NEF | DCSF | DCSF | UE-A or MF/MRF or UE-B |
| Solution #31: Support of IMS DC based avatar communication | DCSF | over BDC | N/A | DCSF | UE-A or MF/MRF or UE-B |
| Solution #32: Solution for support of IMS avatar communication without using data channel | MF | unclear | MF | IMS AS | UE-A or MF/MRF or UE-B |

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-77.

# * * * * First change * * * *

## 8.x Conclusions Related to Key Issue #8

The following conclusions are agreed for supporting IMS Avatar communication:

-	In release 19, avatar communication is only supported based on the data channel architecture.

-	In release 19, network based rendering and UE based rendering is supported, which means:

-	In network based rendering case the rendering of avatar representation and animation stream is executed in the network.

-	In UE based rendering case rendering of avatar representation and animation stream is executed by the local or peer UE.

-	A new Base Avatar Repository (BAR) stores Avatar representation and related user specific data.

-	The Avatar ID list is downloaded to the UE though bootstrap data channel via DCSF. DCSF can retrieve Avatar ID list from BAR.

-	DC AS, MF and/or DCSF interacts with BAR to retrieve the avatar representation by using user identity information and/or Avatar ID. BAR can verify whether UE is allowed to use an avatar representation.

-	The Avatar representation can be animated using audio/video or metadata (such as action and facial expression data) by UE or network. The metadata can be generated by UE or the network.

-	The Avatar representation and metadata are transmitted between the UE and the IMS network using data channel and/or RTP.

NOTE 1:	The metadata media format and the negotiation procedure for Avatar representation animation is further defined in SA4.

NOTE 2:	Whether using RTP to transport avatar representation is feasible will be further aligned with SA4.

-	When network based rendering is used, DC AS sends Avatar communication service control instruction to the MF, the MF should support media transcoding, e.g. ASR/TTS/SLR/SSL.

-	The transition between Avatar communication and MMTel session using audio/video is based on the media negotiation between UEs, or between UE and network.

NOTE 3:	The list of the instruction sent from DC AS to MF will be defined in SA4 or CT1.

NOTE 4:	Whether the authentication and authorization of avatar representation is needed will be decided by SA3.

NOTE 5:	Whether a reference point between UE and BAR is needed will be further align with the conclusions of SA4.

# * * * * End of changes * * * *