SA WG2 Meeting #163	S2-2407012

May 27 – May 31, 2024, Jeju, Korea	(Revision of S2-2406744)

Source:	InterDigital Inc.

Title:	Key Issue #2, New Solution: UE Assisted Paging for AIoT devices

Document for:	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes a solution for Key Issue #2. An Intermediate Node UE handles paging and communication on behalf of AIoT devices.

# 1	Discussion

This paper proposes a new solution for Key Issue #2 - Identification, Subscription, Registration and Connection management.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-13.

* * * * Start of Changes – All New Text * * * *

## 6.x	Solution #x: UE assisted paging for AIoT Devices

### 6.x.1	Functional Description

This solution applies to Key Issue #2 "Identification, Subscription, Registration and Connection management".

This solution proposes that a UE Reader (i.e., intermediate node) handles paging and communication on behalf of AIoT devices using a network-assigned virtual TMSI (VTMSI). The UE receives the VTMSI when registering for AIoT devices paging and communication assistance. The UE then establishes a PDU Session used for the AIoT devices communication which the AMF and UE associates with the VTMSI. The UE monitors paging using the VTMSI to receive downlink data (e.g., command) for the AIoT devices. When paged with the VTMSI, the UE performs the Service Request including the PDU Session ID used for the AIoT service. The UE forwards the data received via the PDU Session to and from the AIoT devices (e.g., using AIoT devices supported RAT).

### 6.x.2	Procedures

The figures below describe the procedures (Registration, PDU Session establishment, network triggered Service Request) with enhancements to support AIoT devices paging and communication assistance by an UE.

Figure 6.x.2-1 shows the UE Registration and PDU Session establishment procedures for an UE.

Figure 6.x.2-1: UE Registration and PDU Session establishment procedures

0. 	During Registration, the UE is authorized in the network (e.g. by the UDM/UDR) to act as a UE for an AIoT service. The authorization information included in the subscription data includes the AIoT Service ID. The UE performs an initial Registration procedure providing its AIoT assistance capabilities. The UE is provisioned by a PCF with a policy authorizing the UE for paging and communication assistance for the AIoT Service ID(s).

1. 	Based on the AIoT authorization policy, the UE sends a Registration Update request message including an AIoT paging and communication assistance request indication and the AIoT Service ID (s) the UE wishes to provide assistance for.

2. 	The AMF checks that the UE is authorized as UE for the provided AIoT Service ID based on subscription information. If the device is authorized the AMF allocates a virtual temporary identifier (VTMSI) for each AIoT service that the UE wishes to provide assistance for. The VTMSI can be in the form of a 5G-S-TMSI to be used in Paging and Service Request procedures. The AMF stores the VTSMI in the UE Context along with the associated AIoT Service ID.

3. 	The UE receives from AMF a Registration Response message including the VTMSI associated with the AIoT Service ID provided by the UE in the Registration Request.

4. 	The UE stores the VTMSI along the AIoT Service ID and starts monitoring for AIoT device paging using the VTMSI.

NOTE: 	The UE performs regular Paging monitoring for its own paging using its 5G-S-TMSI (e.g., for non-AIoT data services). i.e. the PO calculation will still use 5G-S-TIMSI.

5. 	The UE establishes a PDU Session for the AIoT service. The triggering of this PDU Session establishment is not based on uplink data reques. The UE sends the VTMSI to the AMF in the NAS message to the AMF that carries the PDU Session Establishment Request or provides a DNN/S-NSSAI combination used for the AioT service. If VTMSI is provided, the AMF can determine a DNN/S-NSSAI combination for the AIoT service (stored in 2b) based on UE’s subscription information or location configuration and provide the DNN/S-NSSAI combination to the SMF when the PDU Session Establishment Request is sent to the SMF.

Editor's note:	The details of the management (e.g. deactivation) of the PDU Session for AIoT service is FFS.

6. 	The AMF associates the VTMSI with the PDU Session ID (SM Context).

7. 	The UE receives a PDU Session Establishment Accept confirming that the UE is authorized to send and receive data on behalf of AIoT devices. The UE associates the PDU Session ID with the AIoT Service ID. The UE is ready to handle the paging and receipt of downlink data for the AIoT devices.

Figure 6.x.2-2 shows the network-triggered Service Request procedure for the UE

Figure 6.x.2-2: Network triggered service request for UE

0. 	The UE is authorized by the network to act as a UE for an AIoT service. The UE has established a PDU Session for the AIoT service communication. The AIoT application on the UE has established communication with the AIoT AS.

1. 	The AS sends some downlink data to the UE destined for the AIoT devices.

2. 	The UPF detects and notifies the SMF of the arrival of a downlink packet destined for the PDU Session of the IoT Assistant UE.

3. 	The SMF requests AMF transfer of downlink message to UE/RAN providing the PDU Session ID.

4. 	The AMF determines that the message is for a PDU Session used for AIoT device communication based on the PDU Session ID, associates stored AIoT Service ID, and retrieves the associated VTMSI.

5. 	The AMF sends a paging message to RAN to page the UE identified by VTMSI. RAN sends a paging message to the UE including the VTMSI. The paging cycle and the calculation of Paging Occasion uses the UE’s original settings so the UE doesn’t need a different set of Pos for Paging with VTMSI.

6. 	The UE monitors the paging and compares the UE identity in the received paging message with both VTMSI and its own 5G-S-TMSI. If the UE detects the paging for its VTMSI and determines that it needs to receive incoming downlink data for the AIoT devices on the PDU Session established for the AIoT service.

7. 	The UE sends a service request message and includes the PDU Session ID for the AIoT service in the list of PDU Sessions to request the re-establishment of RAN/CN communications resources.

8. 	Following the completion of the Service Request the AMF sends a UE Configuration Command message including optionally a new VTMSI to be used for a future AIoT service paging.

9. 	The UE receives downlink data for the AIoT devices. The UE transmits the data to the devices using the applicable AIoT RAT.

Editor's note:	It is FFS how UE in RRC_INACTIVE state handles the paging.

### 6.x.3	Impacts on existing services, entities, and interfaces

UDM/UDR:

- Stores/Provides: AIoT Assistance Authorization information

PCF:

- Provisions UE with AIoT Assistance Authorization policy

AMF:

- Checks for AIoT Assistance Authorization from UDM

- Allocates and Sends VTMSI to authorized UE (Registration Accept)

- Pages UE using VTMSI

UE (AIoT Assistant):

- Receives from PCF and applies authorization policy to operate as AIoT Assistance UE

- Sends to AMF:

-	AIoT Assistance Capabilities and AIoT Service ID (Registration Request)

-	VTMSI (NAS message for PDU Session Establishment request)

- Receives VTMSI from AMF

- Monitors and receives paging messages for VTMSI.

* * * * End of Changes * * * *