3GPP TSG-WG SA2 Meeting #163	S2-2407074

# Jeju, Korea, May 27 – May 31, 2024

Source: 	China Mobile, China Unicom, NTT DOCOMO, SK Telecom, China Telecom, KDDI, Deutsche Telekom, DISH Network, Orange, KPN, T-Mobile, BT, Rakuten Mobile, Vodafone, vivo,NEC, Huawei, Tencent, CATT, Lenovo,Toyota, ZTE, Samsung, LGE, Meta, Interdigital, OPPO, MediaTek Inc., ETRI

Title: 	Conclusion on KI#1: Network energy related information exposure

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: Propose an update for conclusion on KI#1: Network energy related information exposure.

1	Discussion

This paper provides the update of conclusion of KI#1: Network energy related information exposure.

2	Proposal

It is proposed to include the below changes into TR 23.700-66 v0.6.0.

FIRST CHANGE

8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are for normative work for KI#1:

- A new network functionality (to be specified by SA2) is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator’s policy.

- If the authorized consumer is AF, the granularities include: per application corresponding to the AF, per Network Slice level, per UE, and per UE per AF.

- If the authorized consumer is 5GC NFs, the granularities include: per application, per Network Slice, and per UE, per-UE-per-QoS flow.

NOTE: Whether the per PDU session level is needed will be decided during the normative phase.

- The energy related information for the granularities above include:

- Energy consumption information;

- Renewable energy information;

2)	The consumer NFs including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

3)	The information for the calculation of the Energy Consumption information is obtained from the following sources:

a)	OAM: provides the energy consumption information at the gNB(s) and UPF(s) serving the UE, or Network Slice serving the UE;

b)	OAM: provides the overall data volume for the gNB;

Editor Note: It is FFS whether OAM can provide cell level energy consumption information at the gNB(s).

c)	UPF: provides the overall data volume of the UPF;

d)	UPF: provides the data volume for the QoS flow or the SDF;

4)	The new functionality in the 5GC determines the E2E energy consumption based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF).

5)	In the current release of the specification, only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 1:	The exposure and collection of renewable energy information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

NOTE 3: Depending on TSG RAN decision to work on energy efficiency topic, it can be envisaged to work on procedures where the NG-RAN can detect whether a UE was transmitting in certain energy state (the definition of these energy state rely on RAN WGs to define these) and report this to 5GC.

Editor's note:	Whether in this release the NG-RAN will provide per UE level energy consumption information depending on RAN WGs. SA2 will only start the work based on RAN decision to work on energy efficiency topic.

NOTE 4: This work will coordinate with SA WG5 for OAM and charging part.

End of CHANGES