SA WG2 Meeting #S2-163	S2-2407081

27th-31st  May 2024, Jeju, South Korea	(revision of S2-2406995)

# Title:	TR 23.700-63, Conclusion - KI#3

# Source:	Vodafone, Nokia

# Document for:	Approval

Agenda Item:

Abstract:	This pCR provides conclusion of KI#3

# 1	Proposal

It is proposed to agree the text below for inclusion into TR 23.700-63.

* * * * First Change * * * *

# 8	Conclusions

## 8.x	Conclusion for Key Issue #3

Solution #1 makes the basis of normative work for KI#3. The following principles are extracted from solution #1:

-	Nnef_TrafficInfluence service shall be enhanced to accommodate requests from AFs for handling of Headers/Tags. Nnef_TrafficInfluence_Create and Nnef_TrafficInfluence_Update operations shall be enhanced by using the information described in Table 6.1.2.1-1 as basis.

-	Procedures in clause 4.3.6 of TS 23.502 is reused to provision the Headers/Tags handling information to the SMF/UPF

-	A new data subset in the Application data set is required in the UDR.

-	PCC rule is enhanced by adding the Header/Tags handling information.

-	SMF/UPF are enhanced to support the handling of Headers/Tags based on the Header/Tags handling information received in PCC rule.

-	UPF profile in NRF is enhanced with the new UPF capability to support the UPF selection and discovery

-	Option 2 is used for Headers/Tags direct reporting to NEF/AF.

-	N4 impact for PFCP for Header/Tag handling instructions to UPF and reporting is left for normative phase.

NOTE: 	Reporting can cause UPF to issue lots of reports (e.g. in case of reporting for any UE). Thus, direct reporting from UPF to AF should be used, instead of N4 based reporting that would induce high signaling load.

* * * * End Changes * * * *