3GPP TSG-WG SA2 Meeting #163	S2-2407137

Jeju Island, Korea, May 27th – 31st, 2024	(revision of S2-2406667)

Source:	Huawei, HiSilicon

Title:	Conclusion of key issue 2

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract: this contribution proposes conclusion for key issue#2.

1. Introduction/Discussion

Solution #4, #5, and #6 in TR 23.700-59 address key issue #2 “Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information”.

Following table summarized the information used for NWDAA proposed in three candidate solutions.

|  | Information obtained/collected/derived by network for DAA | How the information helps for DAA |
| --- | --- | --- |
| Solution #4 | Not mentioned | Unclear |
| Solution #5 | Relative distance between UAVs(real-time), by means of R18 LCS/ranging Relative distance between UAVs (predictions), by means of Relative Proximity Analytic from NWDAF | Relative distance, if smaller than a threshold, indicates potential confliction. |
| Solution #6 | Same as solution#5 | Same as solution #5 |

Way forward proposal:

Solution #4 proposed LDS (an application server) to interact with UAV via application layer. For example, LDS may send deconfliction information or warn the UAV of potential conflict, as shown in step 6 of the information flow.

In addition, it is unclear the information obtained/collected/derived by network for DAA.

It is proposed not to consider solution#4 as key issue#2 conclusion.

Solution #5 and solution #6 propose the same solution principles, and address all the technical issues listed in key issue#2 description. Moreover, solution#5 proposal is more comprehensive i.e. covers the solution#6 solution.

It is proposed to select solution#5 as key issue#2 conclusion, and baseline solution for normative work.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-59.

# * * * * First change (all texts new)* * * *

8.X	Conclusion for key issue#2

Following information are agreed to be utilised to enable NWDAA:

- Relative distance of UAVs;

- Absolute location of a UAV;

Relative distance of UAVs is generated/derived in network by using the existing Ranging feature defined in TS 23.586 and TS 23.273. USS may request to GMLC to obtain relative distance for one or multiple pair of UAVs as defined in clause 6.20.3 or clause 6.20.4 in TS 23.273. As an example, USS may use the relative distance of UAVs for e.g. UAV tactical deconfliction, collision avoidance, etc.

Absolute location of a UAV is generated/derived in 5G network by using the existing LCS feature defined in TS 23.273. USS may request to GMLC to obtain one or multiple UAV location as defined in clause 6.2.1, 6.3.1 or 6.20.5 in TS 23.273. As an example, USS may use the absolute location of the UAV(s) provided by the network to e.g. calculate relative distance of the pair of UAVs, and based on that to deal with the UAV tactical deconfliction, collision avoidance etc, which is out of SA2 scope.

NOTE 1: How USS uses the relative distance of UAVs or absolute location of a UAV is out of 3GPP scope.

NOTE 2: Whether to use Relative Proximity Analytics provided by NWDAF to support network-assisted DAA will be decided during normative phase.

# * * * * End of changes * * * *