SA WG2 Meeting #163	S2-2407138

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2405975)

Source:	LG Electronics, Ericsson

Title:	KI#3: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes Evaluation and Conclusion for KI#3.

Discussion

For KI#3, conclusion is needed.

The following solutions for KI#3 are described in TR 23.700-59v0.3.0:

-	Solution #7: Mobility Enhancements for enforcements of NTZ

-	Solution #8: Network support for NTZ management

-	Solution #9: Enabling NTZ support for aerial UEs

-	Solution #15: NTZ restriction Provisioning aspects

-	Solution #16: NTZ Enforcement in 5GC

-	Solution #17: Support of No Transmit Zone

Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

All New Texts

8.3	Interim Conclusion for Key Issue #3: Support of No Transmit Zones

Editor's note:	Feedback from and/or coordination with RAN/RAN WGs is needed to finalize the conclusion for Key Issue #3.

8.3.1	General

The following principles are agreed to support NTZ.

1)	NTZ enforcement applies to the UE that is a UAV UE (aka an aerial UE) with aerial subscription.

2)	NTZs are supported for both LTE and NR.

3)	An NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

4)	No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in three dimensions to consider height restrictions), regardless of the service type.

5)	No UAV UE transmits or attempts to transmit in the restricted frequencies, location and height defined by NTZ. But UAV UEs can transmit/receive normally outside of these restrictions.

6)	UAV UEs supporting NTZ regulations are configured with NTZ information, so that these UAV UEs will be aware of the presence of NTZs.

7)	Reception of downlink data from the network by the UAV UE located in the NTZ is allowed given the regulatory requirements are not violated.

8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information (or NTZ policy) is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and (optional) height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator’s network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE);

.

NOTE 2:	Whether all options are required will be determined in normative phase.

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP release.

The UAV UE supporting NTZ (i.e. from this release forward) performs the following:

c)	enforces NTZ based on the configured/provisioned NTZ assistance information.

d)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase.

NOTE 4:	Whether registration updates by the UAV UE before entering the NTZ and when exiting the NTZ are needed will be decided during normative phase.

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase.

* * * * End of Changes * * * *