3GPP TSG-WG SA2 Meeting #163	S2-2407163

Jeju, KR, May 27th – 31st, 2024	(revision of S2-2406903)

Source:	Huawei, HiSilicon

Title:	KI#3 Conclusion: For UE-SAT-UE communication

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Ph3_ARCH / Rel-19

Abstract: To reuse existing LI architecture, AGW should be onboard. In this release, only when two UEs are under the same satellite, the UE-SAT-UE communication will be enabled.

# 1. Introduction/Discussion

Based on SA3-LI LS on LI considerations for UE-Satellite-UE communications, it is clear the requirement that LI functions need to be present along with their associated network functions on board a satellite.

Based on clause of 7.4.1 IMS High Level LI Architecture in TS 33.127, if only UL CL/L-PSA UPF deployed on satellite, some SA3-LI work needed to support the IMS LI as media traffic will not go through to AGW on ground. SA2 can start normative work with both UL CL/L-PSA UPF and AGW deployed on satellite. Whether SA2 can start normative work with only UL CL/L-PSA UPF on board depends on SA3-LI feedback.

Figure 7.4-1: EPS/5GS-Anchored IMS High Level LI Architecture

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change (all new) * * * *

# 8	Conclusions

## 8.3	Conclusion for KI#3 UE-Satellite-UE Communication

### 8.3.x Ground Fallback Option

When, e.g. due to the change of the satellite serving one of the two UEs involved in a UE-Satellite-UE Communication, the IMS PDU Session and IMS call needs to be reconfigured, as an option:

-	the UE-SAT-UE communication is disabled and ground-based PSA UPF and AGW are used.

-	the P-CSCF is notified to reselect the AGW on the ground  . The P-CSCF also notifies the SMF to remove the UL CL/BP/local PSA UPF onboard for the IMS PDU Session.

NOTE:	How to coordinate the change of serving satellite and AGW relocation (e.g., how to trigger a re-invite message between UE and P-CSCF) will be decided during the normative phase.

# * * * * End of changes * * * *