SA WG2 Meeting #163	S2-2407213

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2407133, S2-2406817)

Source:	Nokia, Nokia Shanghai Bell, Huawei, Ericsson

Title:	KI#2: Update of GMLC Option in Solution#5

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper updates the GMLC based option in solution #5.

# Discussion

For Solution#5 in KI#2, there are following inconsistencies in GMLC based option (Option 1):

- Clause 6.5.3.1, the diagram shows 6.20.1 and 6.20.2 in TS 23.273 to identify the relative position of UAVs but the description doesn’t have any matched/related text.

- The request is “via NEF for Ranging/Sidelink Positioning location results,” which can reply with relative position directly, but the logic shows the two separate UAV absolute location request with GMLC and/or USS to derive the relative position (from the two absolute locations)

# Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

## 6.5	Solution #5: Support Network-assisted DAA with Existing 5GC Services

### 6.5.1	Key Issue mapping

This solution addresses KI#2 aspects.

### 6.5.2	Description

Study which existing information collected and generated in the 5GS can be utilised to enable NWDAA is one of the study aspects in KI#2. This solution proposes to utilize the GMLC service on Ranging/Sidelink Positioning location and Relative Proximity predictions on collision generated at NWDAF to support Network-assisted DAA.

Option 1:

When UAVs have ranging capabilities defined in TS 23.586, USS as AF/LCS client may send the Ranging request to GMLC as defined in TS 29.515 for the ranging result of UAVs (the relative position provided by the ranging service) as defined SL-MT-LR in clause 6.20.3 and/or 6.20.4 in TS 23.273. The result of SL-MT-LR contains the of relative distance  between two or more UAVs.

Option 2:

Alternatively, USS could perform the 5G-MT-LR defined in clause 6.1, 6.3 and/or 6.20.5 of TS 23.273 for one or multiple UAVs and use the absolute location of the UAVs replied by the network to calculate relative distance of the two or more UAVs..

### 6.5.3	Procedures and Parameters

#### 6.5.3.1	Procedure for UAV/UAV-C Triggered Network-assisted DAA

Figure 6.5.3.1-1: Procedure for UAV/UAV-C Triggered Network-assisted DAA

1.	The UAV (or UAV-C) establishes a PDU Session for communication with the USS as described in clause 5.2.3 of TS 23.256 [2].

2.	The DAA service may be triggered by UAV(s) or UAV-C. The UAV(s) (via its paired UAV-C) or the UAV-C requests DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides to subscribe/request to 5GC for GMLC service on Ranging/Sidelink Positioning location and/or Relative Proximity predictions on collision from NWDAF.

NOTE 1:	The other content of DAA service information derived at USS is out of scope.

3.	The USS may request GMLC service via NEF for Ranging/Sidelink Positioning location results, i.e. the relative positioning between UAVs (requested by UAV-C) as described in clause 6.20.3 of TS 23.273 [7] for one notification, or clause 6.20.4 of TS 23.273 [7] for notifications (steps 1-20 for initiation monitoring, steps 21-31 for monitoring periodic).

USS as AF/LCS client may send the positioning request to GMLC as SL-MT-LR or 5G-MT-LR in TS 23.273.

(1)	For option 1:

USS as AF/LCS client trigger SL-MT-LR positioning

request to GMLC. USS as AF/LCS client may checks the relative position in ranging result is lower than the DAA distance threshold. GMLC reports the relative position result to USS within the given time slot..

(2)	For option 2, GMLC replies to USS of the 5G-MT-LR results of UAV 1 and UAV 2.

NOTE 2:	USS may request same or different GMLC for the UAV1 and UAV 2 location, e.g. depending on which PLMN the UAV registers.

4.	The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [6] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [6]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)). The other parameters included in the request are described in clause 6.5.3.3.

5.	After NEF receive the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [6]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

6.	If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [6] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [6].

7.	If receiving the location of UAV1 and UAV2, USS calculates the relative positioning result for UAV UE1 location and UAV UE2 location from step 1 and step 2. The USS estimates the potential collision based on the information received in step 3 and/or the predictions from step 6. The USS informs the UAV-C(s) the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

USS may be replaced by TPAE.

8.	The UAV-C(s) informs its paired UAV(s) the potential collision, and information received from the USS, include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision, and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

9.	UAVs performs operations to avoid collision.

9a.	If both the two UAVs which may collision (e.g. UAV 1 and UAV 2) been informed by their paired UAV-C(s), the UAVs can be steered to avoid collision in accordance with the received information (e.g. trajectory correction information to avoid collision) and using mechanisms that are out of scope for 3GPP.

9b.	If only one of the two UAVs which may collision (e.g. UAV 1) been informed by its paired UAV-C, the UAV 1 triggers conflict resolution procedure with UAV 2 as described in steps 4-7 in clause 5.6.1 of TS 23.256 [2].

#### 6.5.3.2	Procedure for AAM Triggered Network-assisted DAA

Figure 6.5.3.2-1: Procedure for AAM Triggered Network-assisted DAA

1.	As described in clause 5.7.2 steps 1-6 of TS 23.256 [2], the UAV(s) listens for signals on the correspondingly destination Layer-2 ID configured for the used service type. The AAM scans the airspace over the area/arena for UAV(s), retrieves for each detected UAV the corresponding Remote-ID, and establishes a PC5 direct communication link with the discovered UAV. Using the PC5 unicast direct communication link the AAM and the UAV establishes a bidirectional communication channel for exchange of messages.

2.	The AAM establishes a PDU Session for communication with the USS as described in clause 5.2.3 of TS 23.256 [2].

3.	The AAM may request network assist DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides, e.g. to request GMLC service for Ranging/Sidelink Positioning location, to subscribe/request to NWDAF for Relative Proximity predictions on collision.

NOTE:	The other content of DAA service information derived at USS is out of scope.

4.	The USS requests GMLC service for Ranging/Sidelink Positioning location, and/or subscribes/requests notification on Relative Proximity predictions provided by NWDAF via NEF, as described in clause 6.5.3.1 steps 3-6.

5.	The USS estimates the potential collision based on the received information/analytics in step 4. The USS informs the AAM the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

6.	The AAM provides the determined steering policy to the specific UAVs (e.g. UAV 1 and UAV 2) according to the information received in step 5, and the UAVs are steered to avoid collisions in accordance with received policy and using mechanisms that are out of scope for 3GPP, as described in clause 5.7.2 steps 7-8 of TS 23.256 [2].

#### 6.5.3.3	Parameters in Request for Relative Proximity Analytics

The USS acting as an Application Function communicates with the NEF which corresponds to the NF consumer in clause 6.19.4 of TS 23.288 [6].

The USS can either subscribe to notifications from the NEF (i.e. a Subscribe-Notify model) or request a single notification from the NEF (i.e. a Request-Response model). The USS request contains the following parameters:

-	Analytics ID = "Relative Proximity";

-	Target of Analytics Reporting: a UE, a group of UEs;

-	Analytics Filter Information:

-	Area of Interest;

-	An individual or set of direction(s) of interest;

-	Number of UAVs to be accounted for relative proximity (i.e. the number of UAVs for which one UAV may report proximity information);

-	One or several attributes to be accounted for relative proximity (i.e. additional information that can be provided in addition to distance between two UAVs): velocity, average speed, orientation, mobility trajectory;

-	Preferred level of accuracy of the analytics;

-	Maximum number of objects;

-	An Analytics target period indicating the time period over which the predictions are requested.

### 6.5.4	Impacts on services, entities and interfaces

UAS NF/NEF:

-	Enhance Nnef_AnalyticsExposure service, e.g. additional parameters.

NWDAF:

-	Enhance the inputs and the outputs of Relative Proximity Analytics.

Editor's note:	It is FFS if existing data is enough or enhancement is needed.

USS/AF:

-	Enhance for supporting UAV/UAV-C trigger DAA service.

-	Enhance for supporting AAM trigger network assist DAA service.

-	Support of calculate the relative positioning between any two UAV UEs.

Editor's note:	It is FFS other possible impacts related to services, entities and interfaces.

* * * * End of Changes * * * *