SA WG2 Meeting #163	S2-2407282

# Jeju, South Korea, 27 - 31 May 2024                                            revision of S2-2405250

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0436 | rev | 5 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ProSe support for NPNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ProSe_NPN |  |  |  |  |  | Date: |  |  | 2024-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is to address the objectives in the TEI19_ProSe_NPN Public Network Integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. This is still belonging to the concept of PLMN. ProSe case has been supported and only ProSe in conjunction with CAG has not been supported. Standalone NPNs (SNPNs) are currently not supported by 5G ProSe. In this CR, support for SNPNs is added, whereby only the non-roaming architecture is supported for SNPN where the serving SNPN, subscribed SNPN are the same. Additionally the inter-SNPN architecture is not supported. Since the RRC container is already sent to the L2 remote UE, the remote UE can receive the supported CAG information of the gNB from the RRC container. Therefore linkage between the RSC and the CAG is not required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ProSe service in the subscribed SNPN is specified. The ProSe in conjunction with CAG is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of ProSe in SNPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.X (new), 4.2.X.1 (new), 4.2.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

4.2.X	Support for 5G ProSe in NPNs

4.2.X.1	Support for 5G ProSe in SNPN

The proximity based services defined in this specification are applicable to SNPN(s) with the following adaptations:

-	the non-roaming architecure in clause 4.2.1 is applicable also to SNPN, whereby the term PLMN is replaced by subscribed SNPN.

-	the roaming architecture in clause 4.2.2 is not supported.

-	inter-SNPN operation in clause 4.2.3 is not supported.

-	for 5G ProSe Direct Discovery, the procedures in clauses 5.2 and 6, the network function services in clause 7, and the identifiers as specified in clause 5.8.1 are used with the following adaptations:

-	the concept of Local PLMN for direct discovery is not supported.

-	in the ProSe Application ID in clause 5.8.1.1 the value of PLMN ID within the ProSe Application ID shall be combined with a Network Identifier (NID) to identify an SNPN.

-	the ProSe Application Code in clause 5.8.1.4 is extended to allow the value of PLMN ID within the ProSe Application Code to be combined with a Network Identifier (NID) to identify an SNPN.

-	the Announcing PLMN ID in clause 5.8.1.11 is not used in case of SNPNs.

-	the Monitored PLMN ID in clause 5.8.1.17 is ignored in case of SNPNs.

-	in the 5G ProSe Direct Discovery with 5G DDNMF procedures as specified in clause 6.3.1, the term PLMN is to be replaced by SNPN, whereby PLMN IDs shall be combined with a Network Identifier (NID) to identify an SNPN, except if it pertains to PLMN IDs of VPLMNs or Local PLMNs or Announcing PLMN IDs. The term HPLMN is to be replaced by subscribed SNPN.

-	the 5G ProSe UE-to-Network Relay reference architectures for 5G ProSe Layer-2 UE-to-Network relay, 5G ProSe Layer-3 UE-to-Network relay and UE-to-UE relay in clauses 4.2.7 and 4.2.8 are supported, where only the non-roaming architecture as specified for 5G ProSe Layer-3 UE-to-Network relay in clause 4.2.7.1 is supported for SNPNs, and the PLMN A, PLMN B and PLMN C in clauses 4.2.7 are the same in case of SNPNs and are replaced by the SNPN. The term PLMN is replaced by SNPN and the term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs.

-	for Authorization and Provisioning and for the subscription information of the ProSe service in NPNs, the procedures and functionality as specified in clauses 5.1 and 5.7 and 6.2 are used with the following adaptations:

-	the term PLMN is replaced by SNPN. The term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs. Except the subscribed SNPN, no other SNPNs are authorized to perform the 5G ProSe service (i.e. 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay discovery, 5G ProSe UE-to-Network Relay communication, 5G ProSe UE-to-UE Relay discovery and 5G ProSe UE-to-UE Relay communication) over PC5 reference point.

-	for 5G ProSe Layer-2 UE-to-Network relay operation, the procedures as specified in clauses 5.4.2 and 5.4.4.2, and the identifiers as specified in clause 5.8.3 are used with the following adaptations:

-	the term PLMN is replaced by SNPN.

-	the 5G ProSe Layer-2 Remote UE has to operate in SNPN access mode, if it intends to access SNPN for ProSe via 5G ProSe Layer-2 UE-to-Network relay.

-	the 5G ProSe UE-to-Network Relay has to operate in SNPN access mode, if it intends to access SNPN for ProSe via 5G ProSe Layer-2 UE-to-Network relay.

-	the 5G ProSe Layer-2 UE-to-Network relay may participate in the 5G ProSe UE-to-Network Relay Discovery procedure for emergency service if the serving NG-RAN broadcasts ims-EmergencySupport or imsEmergencySupportForSNPN in system information as specified in TS 38.300 [12] and TS 38.331 [16].

-	the NCGI in clause 5.8.3.3 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

-	the 5G ProSe Layer-2 Remote UE only selects the 5G ProSe Layer-2 UE-to-Network Relay for which the NCGI includes the subscribed SNPN of 5G ProSe Layer-2 Remote UE for non-emergency service.

NOTE 1:	To prevent that unauthorized 5G ProSe Remote UEs can gain access to an NPN via 5G ProSe UE-to-Network Relay, the existing User Plane and/or Control Plane authorization verification procedures as specified in clause 6.3.4 of TS 33.503 [29] and restricted 5G ProSe Direct Discovery procedures as specified in clause 6.1.3.2 of TS 33.503 [29] are reused.

-	for 5G ProSe Layer-3 UE-to-Network relay operation, the procedures as specified in clauses 5.4.1, 5.4.4.3 and 6.5, and the identifiers as specified in clause 5.8.3 apply with the following adaptations:

-	the term PLMN is replaced by SNPN. The term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs.

-	the 5G ProSe Layer-3 UE-to-Network relay has to operate in SNPN access mode in order to relay traffic to/from an SNPN for a 5G ProSe Layer-3 Remote UE.

-	the 5G ProSe Layer-3 Remote UE has to operate in SNPN access mode in order to exchange traffic to/from an SNPN via a 5G ProSe Layer-3 UE-to-Network relay

-	the NCGI in clause 5.8.3.2 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

-	the TAI in clause 5.8.3.2 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

NOTE 2:	To prevent that unauthorized 5G ProSe Remote UEs can gain access to an NPN via 5G ProSe UE-to-Network Relay, the existing User Plane and/or Control Plane authorization verification procedures as specified in clause 6.3.3 of TS 33.503 [29] and restricted 5G ProSe Direct Discovery procedures as specified in clause 6.1.3.2 of TS 33.503 [29] are reused.

-	the support for 5G ProSe for UEs in limited service state as specified in clause 5.9 applies whereby the term PLMN is replaced by an SNPN.

The SNPN onboarding procedure defined in clause 5.30.2.10 of TS 23.501 [4] via 5G ProSe UE-to-Network Relay is not supported.

4.2.X.2	Support for 5G ProSe in conjunction with CAG in PNI-NPN

In this release, the CAG based access control is not applicable to the 5G ProSe Layer-3 UE-to-Network Remote UE accessing the network via 5G Layer-3 ProSe Relay.

NOTE 1:	PNI-NPNs by means of one (or more) Network Slice instances or dedicated DNNs are supported for 5G Layer-3 ProSe UE-to-Network Relay.

The 5G ProSe Layer-2 UE-to-Network Relay procedures in conjunction with CAG for PNI-NPN are supported with the following adaptations:

-	the 5G ProSe Layer-2 UE-to-Network relay may participate in the 5G ProSe UE-to-Network Relay Discovery procedure if the 5G ProSe Layer-2 UE-to-Network relay is accessing a CAG cell in the Allowed CAG list configured on the 5G ProSe Layer-2 UE-to-Network relay.

NOTE 2:	5G ProSe Layer-2 Remote UE and 5G ProSe Layer-2 UE-to-Network Relay can access different CAG IDs.

NOTE 3:	In order to prevent that unauthorized 5G ProSe Remote UEs can gain access to a CAG cell operated by a PNI-NPN which a 5G ProSe Layer-2 Remote UE is not allowed to access, the existing access control mechanisms as specified in clause 5.30.3.4 of TS 23.501 [4] and clause 16.7.4 of TS 38.300 [12] are reused.

* * * * End of Changes * * * *