SA WG2 Meeting #163	S2-2407284
Jeju Island, Korea, May 27 – 31, 2024

# Source:	Nokia, Meta

# Title:	WID on 5GS XRM Ph2

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Extended Reality and Media service (XRM) Phase 2

Acronym:	XRM Ph2

Unique identifier:	1010032

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | x | x |  |
| No |  |  |  |  |  |
| Don't know | x |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  | SA1 | 950005 | Study on Localized Mobile Metaverse Services |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

Rel-18 has introduced the support for power saving optimizations based on dynamic End of Data Burst indication in addition to more static periodicity and burst size information. This, as well as the PDU Set based handling, could further be enhanced to support adaptive applications.

In Rel-18, SA2 did not have sufficient time to introduce any potential new 5QIs (as needed), consider Alternative PDU Set based QoS handling and/or PDU Set based QoS handling for DSCP marking over N3/N9. In addition, it is also assumed that the application may not require all the PDUs within a PDU Set thus content ratio could be considered as part of the study.

SA2 specifications do not have a solution for multiplexed data flows. Also, dynamic change in traffic characteristics was not considered sufficiently in the earlier releases.

SA2 also did not consider support for non-3GPP access for XRM features in the earlier releases. SA2 may need to consider additional exposure aspects from 5GS towards application for XRM features.

The work item is to document the conclusion agreed as part of FS_XRM_Ph2 study item and document them as normative specification for 5G System in the respective specifications.

# 4	Objective

FS_XRM_Ph2 conclusion agreed (as in TR 23.700-70) for the following key issues will be specified:

WT#1.1

KI#1 Enhancement for PDU Set based QoS handling.

- Support PDU Set QoS parameters within Alternative QoS.

- Support PDU Set Information marking independent of PDU Set QoS handling.

WT#1.3

KI#3 Enhancement for PDU Set based DSCP marking.

- Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network (i.e. to enable differentiated handling of PDU Sets within QoS Flow).

WT#2.1

KI#4 QoS handling enhancement for XRM services.

- Additional packet filter for traffic detection and QoS Flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

WT#2.2

KI#5 Dynamic traffic characteristics update.

- Support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters) provided by the application in the DN.

WT#3.1

KI#6 Further enhancement to support XR based on non-3GPP access.

- Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF).

- Support L4S in wireline access (e.g. W-AGF).

WT#3.2

KI#7 Extend PDU Set QoS Control mechanisms to non-3GPP access networks.

- Support PDU Set QoS in untrusted/trusted access (e.g. N3IWF, TNGF).

- Support PDU Set QoS in wireline access (e.g. W-AGF).

NOTE 1: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks.

WT#4

KI#9 Network exposure

- GBR Data Rate can be exposed towards the application layer.

- PDU Session AMBR and rate limiting information can be exposed towards the application layer.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  | 2 |  | WT#1 is self-contained |
| WT#1.1 |  | 1.0 | Yes |  |
| WT#1.2 |  | 0.75 | No |  |
| WT#1.3 |  | 0.25 | No |  |
| WT#2 |  | 2.0 |  | WT#2 is self-contained |
| WT#2.1 |  | 1.0 | No |  |
| WT#2.2 |  | 1.0 | Yes |  |
| WT#2.3 |  | 0.0 | No |  |
| WT#3 |  | 1.0 |  | WT#3 is self-contained |
| WT#3.1 |  | 0.5 | No |  |
| WT#3.2 |  | 0.5 | No |  |
| WT#4 |  | 1 | May be | May be dependent on WT#1.1 |

Total TU estimates for the normative phase: 6

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 23.501 | Architectural and functional updates | SA#106 |
| 23.502 | Procedural updates | SA#106 |
| 23.503 | Policy aspects related updates | SA#106 |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Devaki Chandramouli (Nokia), devaki.chandramouli@nokia.com.

Secondary Rapporteur: Curt Wong (Meta USA), curtwong@meta.com.

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN impact to be covered by RAN WGs.

Potential security impact to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

Potential multimedia and codecs aspects to be covered by SA4.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Charter Communications, Inc?

CableLabs

China Mobile

Tencent?

Xiaomi

Broadcom?

Cisco?

Samsung?

Meta USA

CATT?

ZTE?

FirstNet

Telefonica?

AT&T

FutureWei?

Lenovo?

Huawei

HiSilicon?

LG Uplus?

BT Plc?

MATRIXX Software?

DISH Network?

Deutsche Telekom?

Intel

Mediatek?

Verizon?

LG Electronics

Google?

OPPO?

China Telecom?

NTT Docomo?

Ericsson?

InterDigital Inc.?

Apple

Vivo?