3GPP TSG-WG SA2 Meeting #163	S2-2407358

Jeju Island, Korea, May 27th – 31st, 2024	(revision of S2-2407321)

Source:	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell

Title:	Conclusion of key issue 2

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract: this contribution proposes conclusion for key issue#2.

1. Introduction/Discussion

Solution #4, #5, and #6 in TR 23.700-59 address key issue #2 “Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information”.

Following table summarized the information used for NWDAA proposed in three candidate solutions.

|  | Information obtained/collected/derived by network for DAA | How the information helps for DAA |
| --- | --- | --- |
| Solution #4 | Not mentioned | Unclear |
| Solution #5 | Relative distance between UAVs(real-time), by means of R18 LCS/ranging Relative distance between UAVs (predictions), by means of Relative Proximity Analytic from NWDAF | Relative distance, if smaller than a threshold, indicates potential confliction. |
| Solution #6 | Same as solution#5 | Same as solution #5 |

Way forward proposal:

Solution #4 proposed LDS (an application server) to interact with UAV via application layer. For example, LDS may send deconfliction information or warn the UAV of potential conflict, as shown in step 6 of the information flow.

In addition, it is unclear the information obtained/collected/derived by network for DAA.

It is proposed not to consider solution#4 as key issue#2 conclusion.

Solution #5 and solution #6 propose the same solution principles, and address all the technical issues listed in key issue#2 description. Moreover, solution#5 proposal is more comprehensive i.e. covers the solution#6 solution.

It is proposed to select solution#5 as key issue#2 conclusion, and baseline solution for normative work.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-59.

# * * * * First change (all new text)* * * *

8.X	Conclusion for key issue#2

Following information are agreed to be utilised to enable NWDAA:

-	Absolute location of a UAV;

-	Output of Relative Proximity Analytics provided by NWDAF.

Absolute location of a UAV is generated/derived in 5G network by using the existing LCS feature defined in TS 23.273. USS may request to GMLC to obtain one or multiple UAV location as defined in clause 6.2.1, 6.3.1 or 6.20.5 in TS 23.273. As an example, USS may use the absolute location of the UAV(s) provided by the network to e.g. calculate relative distance of the pair of UAVs, and based on that to deal with the UAV tactical deconfliction, collision avoidance etc, which is out of SA2 scope.

USS (as an AF) may request Relative Proximity Analytics provided by NWDAF for the purpose of DAA.

NOTE 1: How USS uses the relative distance of UAVs or absolute location of a UAV is out of 3GPP scope.

NOTE 2: Applicability of the above solutions to support DAA will be evaluated during normative phase.

# * * * * End of changes * * * *