3GPP TSG SA WG2 Meeting #164	S2-2407459

# Maastricht, Netherlands, August 19 – August 23, 2024                  (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1402 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6: Combination of standalone bootstrap DC and standalone application DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#6 conclusion in TR 23.700-77, the standalone IMS DC session needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The standalone bootstrap DC, standalone application DC, and the combination of standalone bootstrap DC and standalone application DC are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The standalone IMS DC session cannot be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new)* * * *

AC.X	Standalone IMS Data Channel Session

AC.X.1	Standalone IMS DC

AC.X.1.1	General

The standalone bootstrap DC session and standalone application DC session are established with the SIP INVITE request which only contains SDP offer for bootstrap DC and/or application DC, without SDP offer for any other media than data channel. The bootstrap DC setsup signalling and application DC setup signalling defined in clause AC.7.1 and AC.7.2 are used with the following changes:

- The standalone bootstrap DC session is used for downloading the application list and applications from the DCSF. The request URI in the INVITE request for standalone bootstrap DC is set to a PSI indicating standalone bootstrap DC which is used by IMS AS to target to the DCSF, and the PSI is configured in the UE. The standalone bootstrap DC session is the local DC session within the originating network.

- The combination of standalone bootstrap DC and standalone application DC session is used to establish application DC for the application that has been downloaded in the UEs. The request URI in the INVITE request for the combination of standalone bootstrap DC and standalone application DC session is set to the IMPU of the terminating UE. The detail setup procedure is described in clause AC.X.1.2.

- During DC routing decision at IMS AS, the IMS AS determines whether to accept the standalone data channel session when the user has subscribed to the standalone IMS DC service.

AC.X.1.2	 Combination of standalone BDC and standalone ADC setup

The combination of standalone BDC and standalone ADC session is used to establish application DC for the application that has been downloaded in the UEs.

The originating UE sends INVITE request which only conatins SDP offers for bootstrap DC and application DC without SDP offer for any other media. The originating UE requests INVITE only for the DC application that has been downloaded in the originating UE, and the application information (e.g. the identifier of the application) is indicated within the application binding information in the application DC SDP offer.

The terminating UE accepts and responds among the DC application that has been downloaded in the terminating UE. If there is any application that has not been downloaded among the DC applications offered by the originating UE, the terminating UE downloads accepts the bootstrap DC offer, and downloads those DC application after the boostrap DC establishment. After receiving the response, the originating UE updates the session with the SDP offer of application DC again and the terminating UE responds the SDP answer of application DC after the application is downloaded.

Figure AC.X.1.2-1 depicts a signalling flow diagram when the remote UE has not downloaded DC Application(s) yet during the IMS DC session setup for a combination of standalone BDC and standalone ADC.

Figure AC.X.1.2-1: IMS DC session setup signalling procedure for a combination of standalone BDC and standalone ADC

The steps in the call flow are as follows:

- UE#1 sends the SIP INVITE request to IMS AS, through originating network P-CSCF and S-CSCF. In the SIP INVITE, the SDP offer for bootstrap DC and application DC are included and SDP offer for any other media is not included. For the SDP offer for application DC, the application bindin information of the DC application(s) that have been already downloaded in UE#1.

- DC routing decision is performed by the IMS AS. The IMS AS determines whether to accept the standalone data channel session when the user has subscribed to the standalone IMS DC service. If determined, the Media resource reservation is proceeded.

3-4. The SIP INVITE request is transferred to the terminating network and the UE#2.

5-6. If the terminating UE has downloaded the application and if the user accepts the session for the application, the terminating UE responds with the SDP answers for both bootstrap DC and application DC. Within the SDP answers of application DC, the application binding information for the accepted applications are included. If there is any application that has not been downloaded in UE#2, the UE#2 downloads the application after the bootstrap DC is established, as described in step 10.

- Session Event Notification and Media resource update are performed based on the response from the terminating side.

- The response from UE#2 is transferred to the UE#1.

- DC QoS procedure is executed.

- Upon reception of response on bootstrap DC answer in step 8, the bootstrap DC is established between the UE#1 and the originating MF, and between the originating MF and the UE#2. The UE#2 downloads the application that has not been downloads which were offered in step 4.

- For the accepted DC applications in step 8, the application DC is established between the UE#1 and UE#2.

- The UE#1 may send SIP re-INVITE request with the standalone ADC offer for the application that were not accepted in step 8. The SDP offer for this standalone ADC includes the application binding information for the DC applications that are offered.

- The same procedures with the step 2-5 are proceeded.

- Afthe completion of downloading the offered DC applications, if the UE#2 accepts the session for the applications, UE#2 responds with the SDP answers for application DC that includes the application binding information for the accepted applications.

15-16. The same procedures with the step 7-8 are proceeded.

- For the accepted DC applications in step 16, the application DC is established between the UE#1 and UE#2.

# * * * * End of changes * * * *