3GPP TSG SA WG2 Meeting #164	S2-2407465

Maastricht, Netherlands, August 19 – August 23, 2024	(Revision of S2-240xxxx)

Source: 	Samsung

Title: 	KI#2: Conclusion update

Document for: 	Approval

Agenda Item: 	19.4.1

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution provides update of conclusion for key issue #2.

1	Discussion

This contribution provides the conclusions for KI#2.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-66.

FIRST CHANGE

8.2	Conclusion for Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

The following principles for enhancements on subscription and policy control are agreed for the normative work:

-	The following information is stored as part of the subscription data in the UDM/UDR:

-	Energy saving subscription information per UE to assist the network to perform energy saving strategies for the UE.

-	The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions on the AM, SM and UE policies (reusing the existing parameters), based on operator policy.

NOTE 1:	Further details and what additional energy related information can be taken into account for policy control will be defined during the normative work.

NOTE 2:	Further details, more parameters and associated procedures will be defined during the normative work.

End of CHANGES