3GPP TSG-WG2 Meeting #164	S2-2407491

19th – 23rd August 2024, Maastricht, Netherlands

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1096 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – VFL training and inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is to cover the following agreed principles in the conclusions of the TR 23.700-84 clause 8.2: - VFL related new functionalities including P#2.1.1, P#2.1.2, P#2.1.3, P#2.1.5 and P#2.1.6. - Registration and discovery of VFL entities including P#2.2.1, P#2.2.2 and P#2.2.3. - P#2.3: For sample alignment for VFL including P#2.3.1, P#2.3.2, P#2.3.3 and P#2.3.4. The following open issues om P#2.3 are addressed: NOTE 2: Whether and how the feature alignment is supported will be determined in normative phase. The proposal is to support optional feature alignment during the preparation phase, e.g. by each VFL participant registers Feature ID(s) into NRF. NOTE 3: The procedure to perform sample alignment is not agreed yet, i.e. it can be done in a standalone procedure or part of the preparation phase, which will be determined in normative phase. The proposal is to support sample alignment during the preparation phase, it can be done extending the existing Nnwdaf_ModelTraining services, with the candidate samples, this requires minor changes compared with having a new procedure. - P#2.4: VFL training, all principles except P#2.4.5, - P#2.5: VFL inference, all principles except P#2.5.2, P#2.5.5: The following open issues in P#2.5 are addressed: NOTE 10: Whether the NWDAF supports both AnLF and MTLF or not during VFL training and inference is to be discussed in normative phase. The proposal is to define a distribute inference where the NWDAF supports both AnLF and MTLF, this will be the initial case that can be later extended in a subsequent release. This proposl is to simplify the procedure for distributed inference, the AnLF to MTLF interface is internal to NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To cover P#2.1, new definitions for VFL Server, VFL Client and VFL procedure are listed in clause 3.1. A VLF process is replaced by a VFL procedure, some additional definitions are listed. To cover P#2.2, NWDAF discovery and selection clause is extended to include the new information that NWDAF provides in the NRF profile or NEF (when the AF is untrusted). It is poposed to reuse the existing FL capability information as is, and add a new optional IE for FL type set to HFL or VFL, when the FL type is not present the FL method is HFL. Some additional IE are also included in the NF profile thay includes the Interoperability information, listing the type of VFL supported, and related information with the type of VFL supported, and the convergence reports. To cover P#2.3, the procedure for preparation for VFL allows the FL Server to trigger sample alignment, the final selection of the samples to us is upto the FL Server. The Feature IDs may be registered to the NRF or locally available at the Server. To cover P#2.4, the general procedure for VFL is added To cover P#2.5, the procedure for distributed inference is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions defined in the TR 23.700-84 are not specified. Some of the open issues left for normative work are not discussed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.2, 5.3, 5.4 (new), 5.4.1 (new), 5.4.2 (new), 6.2C.1, 6.2F.2. 6.2.x (new), 6.2x.1 (new), 6.2x.2 (new), 6.2x.2.1 (new), 6.2x.2.2 (new), 6.2x.2.2 (new), 6.2x.2.3 (new), 6.2x.2.4 (new), 6.2x.2.4.1 (new), 6.2x.2.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4]. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

ML feature: An ML feature are the input data an ML model uses for training or inference.

ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

VFL Client: An NF that holds the local dataset and performs local training and inference as requested by the VFL Server. There can be multiple VFL Clients in the VFL training and the VFL inference procedure.

VFL procedure: It refers to the procedure to train an ML Model or to perform inference of an ML Model using VFL techniques. When the VFL Server is the NWDAF, the VFL procedure is related to an ML Model for an Analytics ID.

VFL Server: An NF that integrates local training results for the local ML model updated in the VFL training procedure. In the VFL training procedure, the VFL Server also performs discovery and selection of VFL Clients. In the VFL inference procedure, the VFL server combines local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer.

* * * Next Change * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DCAF	Data Collection Application Function

HFL	Horizontal Federated Learning

RE-NWDAF	Roaming Exchange Network Data Analytics Function

VFL	Vertical Federated Learning

* * * Next Change * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required, ML Model Interoperability indicator and NF consumer information. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. FL server), may include the supported FL type (i.e. VFL or HFL), and may include Time interval supporting FL both as FL capability information and the supported FL type during the registration in NRF.

NOTE 6:	When the supported FL type is not provided, the FL capability type refers to the NWDAF capability for FL server or FL client for HFL.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include FL capability type (i.e. FL client), may include the supported FL type (i.e. VFL or HFL)  and may include Time interval supporting both FL as FL capability information and the supported FL type during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 7:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID. An NWDAF containing MTLF may indicate to support both HFL and VFL in the supported FL type for a specific Analytics ID.

-	An NWDAF containing MTLF supports FL as client or as server, and supporting FL type as VFL may include the Interoperability information (e.g. type of VFL training method, dimensionality and support for ML model convergence reports), optionally supported Feature IDs as part of the NF profile registered in NRF.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server, the supported FL type (i.e. VFL or HFL) and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as FL client and may provide the supported FL type (i.e. VFL or HFL) and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 8:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF and/or AF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

* * * Next Change * * *

## 5.3	Federated Learning (FL) among multiple NWDAFs

This clause describes Horizontal Federated Learning.

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAF for short), the main functionality includes:

FL server NWDAF:

-	discovers and selects FL client NWDAFs to participant in an FL procedure

-	requests FL client NWDAFs to do local model training and to report local model information.

-	generates global ML Model by aggregating local model information from FL client NWDAFs.

-	sends the global ML Model back to FL client NWDAFs to perform an additional training iteration if needed.

FL client NWDAF:

-	locally trains ML Model as tasked by the FL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other FL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the FL server NWDAF.

-	receives the global ML Model from FL server NWDAF and perform an additional training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML Model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML Model training, then FL procedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model. The FL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating FL server to initiate FL procedure on the FL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as described in clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using its local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

* * * Next Change * * *

## 5.4	Federated Learning (FL)

### 5.4.1	Horizontal Federated Learning

Horizontal Federated Learning is described in clause 5.3, and its related procedures are described in clause 6.2C. A FL Server refers to a NF that has the FL capability type set to FL server and the supported FL type set to HFL or not present.

### 5.4.2	Vertical Federated Learning

Vertical Federated Learning is a machine learning technique that trains an ML Model across multiple NFs. Each NF trains their local ML Model with its local data for the same samples. Only NWDAF containing MTLF and AF are the NFs that train the ML Model using VFL, possibility via NEF. When NEF is involved, the NEF may provide a candidate list of VFL Clients that fulfil certain criteria provided by the AF.

A FL Server refers to a NF that has the FL capability type set to FL server and the supported FL type set to VFL.

Vertical Federated Learning includes the following procedures:

- Registration of the NF profile including a list of VFL related information to NRF. Registration of the NWDAF profile to NRF is described in clause 5.2. For an untrusted AF, the NEF registers based on configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes as part of the information about the AF an VFL capability information. The procedure for registration and discovery of VFL server and VFL client is described in clause 6.2x.2.1.

- Preparation for VFL including sample alignment to ensure that all the VFL participants has common samples when training ML models. as described in clause 6.2x.2.2.

- VFL training as described in clause 6.2x.2.3.

- Distributed inference as described in clause 6.2x.2.5.

* * * Next Change * * *

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Horizontal Federated Learning technique to train an ML Model.

* * * Next Change * * *

### 6.2F.2	Contents of ML Model Training

The consumers of the ML Model training services (i.e. an NWDAF containing MTLF and/or an AF) may provide the input parameters in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operations as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	For HFL, ML Model Interoperability Information as defined in clause 6.2A.2.

-	(Only for Nnwdaf_MLModelTraining_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing MTLF with the subscription.

-	[OPTIONAL] For HFL, ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] For HFL, ML Model file.

NOTE 1:	It is up to NWDAF implementation to determine whether to include ML Model file in input parameters considering ML Model file size, etc.

-	[OPTIONAL] ML Model identifier: For HFL, identifies the provided ML Model. For VFL identifies the server’s ML model.

-	[OPTIONAL] ML Preparation Flag: identifies whether the request is for preparing Federated Learning or executing Federated Learning.

-	[OPTIONAL] For HFL, ML Model Accuracy Check Flag: identifies that the request is for using the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the NWDAF service consumer acting as the FL Server NWDAF.

-	[OPTIONAL] For HFL, ML Correlation ID: identifies the Federated Learning procedure for training the ML Model. This parameter is included when the service is used for Federated Learning.

-	[OPTIONAL] For HFL, Data Availability requirement. This is the requirement on data availability for the ML Model training. e.g. FL Server NWDAF sends the requirement in preparation request to a FL Client NWDAF for selecting the FL Client NWDAF which can meet the data availability requirement. The following may be included:

-	Event ID list to be collected for local model training.

-	Dataset statistical properties as defined in clause 6.1.3.

-	Time window of the data samples.

-	Minimum number of data samples.

-	[OPTIONAL] FL Availability time requirement. This is the requirement on availability time for the ML Model training, e.g. FL Server NWDAF sends the requirement in preparation request to FL Client NWDAF for selecting the FL Client NWDAF which is available in the required time for training ML Model.

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	[OPTIONAL] Target of Training Reporting: indicates the object(s) for which data for ML Model training is requested, i.e. group of UEs identified by a list of Internal-Group-Ids or any UE (i.e. all UEs). This parameter contains the samples for VFL training when samples are UEs, it is also used during the preparation phase for sample alignment.

-	[OPTIONAL] Use case context: indicates the context of use of ML Model.

-	[OPTIONAL] Training Reporting Information with the following parameters:

-	Maximum response time: indicates maximum time for waiting notifications (i.e. model training results).

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of current ML Model training.

-	[OPTIONAL] Expiry time.

-	[OPTIONAL] Indication of skipping the current FL round.

-	[OPTIONAL] During VFL preparation phase, Interoperability information containing the VFL training method (e.g., neural networks, etc).

- [OPTIONAL] During VFL preparation phase, additional interoperability information that depends on the VFL training method. When VFL training is neural networks, the additional interoperability information, suggested by server includes the dimensionality of the intermediate results which defines the number of samples, number of nodes for the intermediate matrix, the type of content in the intermediate matrix container, exchanged between FL Clients and FL Server (e.g., activations, gradients, etc). the supported model convergence reports.

- During VFL training, Intermediate results that carries information between the VFL participants (server or client). The Intermediate results contain when the VFL training is "neural networks":

-	Intermediate matrix indicator may be set to e.g. gradients and/or other agreed content in the preparation phase.

-	Intermediate matrix container which includes the matrix of e.g., gradients per batch of samples sent in the container.

-	[OPTIONAL] model convergence reports which are used to support the converge of a model during training and may contain one or multiple of the below parameters, The server sends it to the clients that may use it to adapt the internal ML model training settings (Seetings are implementation specifics or set by configuration and some examples of possible settings are given below):

-	Expected number of iterations left.

-	Convergence speed such as Fast/slow/almost done converegence. This may trigger the receiver to e.g., set the learning rate.

-	Under or over fitting. This may trigger the receiver to e.g., expand or reduce the input data to be collected.

-	Not converging at all. This may trigger the receiver to e.g., change the ML Model.

-	Stuck in a local minimum or similar. May trigger the receiver to introduce random noise to its local model.

-	Addition of random noise or similar in Intermediate results. This is sent to inform the receiver, that local monitoring of convergence may diverge from expected.

The NWDAF containing MTLF provides to the consumer of the ML Model training service operations as described in clause 7.10 and clause 7.11, the output information in notification or response as listed below:

-	(Only for Nnwdaf_MLModelTraining_Notify) The Notification Correlation Information.

-	[OPTIONAL] ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] ML Model identifier: identifies the provisioned ML Model.

-	[OPTIONAL] Global ML Model Accuracy information: The model metric value of the global ML Model and optionally the used metric, which is calculate by the FL Client NWDAF using the local training data as the testing dataset.

[OPTIONAL] Status report of FL training: Accuracy information of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the FL Client NWDAF during FL procedure.

NOTE 2:	The parameters in Training Input Data Information are up to the implementation.

-	[OPTIONAL] ML Correlation ID. This parameter may be included when the service is used for Federated Learning.

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of ML Model training indicated by the FL Server NWDAF.

-	[OPTIONAL] Delay Event Notification with the following parameters:

-	delay event indication: this parameter indicates that FL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the FL Server NWDAF.

-	[OPTIONAL] cause code (e.g. local ML Model training failure, more time necessary for local ML Model training, etc.).

-	[OPTIONAL] Expected time to complete the training: Indicates to the FL Server NWDAF that expected remaining training time and may be provided with Delay Event Notification.

-	[OPTIONAL] For VFL preparation phase, agreeable Interoperability information.

-	[OPTIONAL] For VFL preparation phase, maximum agreeable dimensionality of the Intermediate results.

-	[OPTIONAL] For VFL preparation phase, the list of samples agreeable by VFL client.

-	[OPTIONAL] For VFL, model convergence reports (during preparation phase the listed parameters indicates which parameters are supported to be received from server. The client shall not send any other parameter than indicated as supported by server in the request).

- [OPTIONAL] For VFL training, Intermediate results that carries information between the VFL participants (server or client). The Intermediate results contain when the VFL training is "neural networks":

-	Intermediate matrix indicator may be set to e.g. activations and/or other agreed content in the preparation phase.

-	Intermediate matrix container which includes a matrix of e.g., activations per batch of sample sent in the container.

-	[OPTIONAL] model convergence reports which are used to support the converge of a model during training and may contain one or multiple of the parameters mentioned above in the request from the server, The reports are only sent if the client perform convergence monitoring. The reports may be used in server to either replace monitoring or support monitoring at server.

NOTE x: Addition of random noise or similar in Intermediate results is only added after server has sent a report that will initiate a setting in client that introduces such action.

* * * Next Change (all new text) * * *

## 6.2X	Vertical Federated Learning

### 6.2X.1	General

This clause specifies how NWDAF(s) containing MTLF and AF(s) together can leverage Vertical Federated Learning technique to train an ML Model.

### 6.2X.2	Procedures

#### 6.2X.2.1	Registration and Discovery procedure for Vertical Federated Learning

This procedure describes Registration of VFL participants and Discovery of the clients before VFL procedure start.

Figure 6.2X.2.1-1: Registration and Discovery procedure for Vertical Federated Learning

The VFL Server can either be an AF or an NWDAF.

If VFL Server is an NWDAF, the client(s) can be either AF(s) or NWDAF(s).

If VFL Server is an AF the client(s) can be NWDAF(s).

Steps 1 to 3 are the registration procedure.

1-3.	FL Server or FL Client(s) registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), FL capability type information (i.e. FL server and/or FL client and/or FL type (i.e. HFL and/or VFL)), Interoperability information, and Time interval supporting FL as described in clause 5.2.

If AFs are untrusted a NEF is used to register information of the AF instead of the AF(s) as described in clause 6.2.2.3.

Editor’s Note: Further extensions are needed to show when any of the VFL participants are an untrusted AF. In this case the procedure below will contain a NEF.

Steps 4 to 6 are the discovery procedure.

4-6.	Server determines ML Model requires FL based on e.g. operator policy (e.g. pre-configured list of ML Models), Analytics ID or data cannot be obtained directly from data producer NF (e.g. due to privacy reasons).

If the VFL Server is an NWDAF containing MTLF and cannot perform as VFL Server NWDAF, the MTLF first discovers and selects VFL Server from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, Model filter information as defined in clause 6.2A.2, FL capability Type (i.e. FL server), Time Period of Interest.

NOTE: The above step may include a request to an AF to become VFL Server.

Once the VFL Server (the requested or the selected one) is determined, the VFL Server discovers and selects client(s).

If VFL Server is an NWDAF, it uses NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, FL capability Type (i.e. FL client), NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local ML Model training, Time Period of Interest.

#### 6.2X.2.2	Preparation procedure for Vertical Federated Learning

Preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) on what of the Interoperability Information shall be used, sample alignment and optionally feature alignment.

NOTE 1: 	Features can be non-specified, privacy protected and therefore feature alignment is a simple alignment using info registered in NRF, e.g. data sources info or proprietary Feature ID).

Figure 6.2X.2.1-1: Registration and Discovery procedure for Federated Learning

If VFL Server is an AF, it may use UE member selection to select candidate UEs before starting below procedure.

Editor’s Note:	Further extensions are needed to show when any of the VFL participants are an untrusted AF. In this case the procedure will contain a NEF.

1.	FL Server sends Federated Learning preparation request to the FL Client(s), using Nnf_MLModelTraining_Subscribe or Nnf_MLModelTrainingInfo_Request service with the ML Preparation Flag. The Server may add Interoperability Information, sample IDs, Analytics ID and optionally Feature ID.

NOTE 2:	Nnf_MLModelTraining_Subscribe can either be Nnwdaf_ MLModelTraining_Subscribe or Naf_ MLModelTraining_Subscribe.

NOTE 3:	Federated Learning preparation procedure (i.e. steps 1-4) can be skipped if the FL Server can decide that the FL Client(s) support the FL procedure to be performed, e.g. based on information acquired from previous FL procedures or from the NRF, or based on local configuration.

2.	FL Client(s) checks if it can meet the ML Model training requirement. If it cannot meet the requirements for any reason, it may decide which requirements it can agree to.

3.	FL Client(s) invokes Nnf_MLModelTraining_Notify or Nnf_MLModelTraining_Subscribe response or Nnf_MLModelTrainingInfo_Request response service operation to indicate to the FL Server whether it accepts the requirements, request new requirements or whether it will not join. The client includes the requirements it can accept or may add a reason if it cannot join the FL process.

If the notify or response includes new requested requirements, procedure from step 7 is repeated.

4.	FL Server NWDAF determines the FL Client(s) to be involved in the FL procedures based on the information received in step 6 in Figure 6.2X.2.1-1 and other information received in step 3 (if available).

#### 6.2X.2.3	General procedure for Vertical Federated Learning

Figure 6.2X.2.2-1: General procedure for Vertical Federated Learning

Editor's Note:	Further extensions are needed to show when any of the VFL participants are untrusted AF(s). In this case the procedure below will contain a NEF, and the service operations going via NEF includes the same service operation but as an Nnef service operation in between.

Editor’s Note: Further extensions are needed to the procedure which only describes the Server being NWDAF and clients are also NWDAFs. Extensions includes when untrusted AF is included either as Server or client, the service operations are changed accordingly to be an NFaf service operation.

1.	FL Server selects clients(s) as described in clause 6.2X.2.1 and clause 6.2X.2.2.

2.	FL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected clients(s), which participate in the Federated learning to perform the local model training. The request includes ML Model ID, sample IDs, and may include Interoperability Information and maximum response time, the FL Client NWDAF has to report the intermediate results to the FL Server NWDAF before the maximum response time elapses.

3.	[Optional] Each FL Client collects its local data.

4.	During Federated Learning training procedure, each FL Client trains the local ML Model and reports the Intermediate result to the FL Server in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response. The Intermediate results includes Intermediate matrix indicator per intermediate Intermediate matrix container and may include model convergence reports.

NOTE 1:	There can be multiple intermediate result containers in one message. The intermediate content indicators indicate what is included in each container.

The Intermediate results, which is sent from the FL Client(s) to the FL Server during the FL training process, is the information needed by the FL Server to generate the intermediate or final output.

NOTE 2:	The output is the output for the Analytics ID. During FL it is intermediate and becomes final when training is done.

A client may indicate in message that it will leave the FL.

NOTE 3:	The FL Server may inform a FL Client to cease the ML Model training by sending termination request and to report back the current Intermediate results.

5.	The FL Server generates the intermediate output. The FL Server may also compute the loss and/or gradients.

6.	[Optional] The FL Server may terminate the current FL training process.

If the FL Server decides to stop the Federated Training process, steps 7 and 8 are skipped.

7.	If the FL procedure continues, FL Server NWDAF determine which FL Client(s) to continue the FL with and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the intermediate result to the FL client(s). The Intermediate results includes Intermediate matrix indicator per intermediate matrix container and may include model convergence reports, sample IDs (if changed).

8.	Each FL Client performs training locally using info received by the FL Server at step 7.

NOTE 4:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached or server requests to pause.

The FL Server requests the FL client(s) to stop the FL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the FL process has finished. If a stop is requested, the FL client(s) terminate the local model training and stores at least the local Model.

NOTE 5:	The local Model is connected inside the client to the Model ID received from the server. The connection is used for inference.

#### 6.2x.2.4	Distributed Inference

##### 6.2x.2.4.1	General

The inference process occurs once the training process has been done, the VFL Server knows that there is an ML Model for an AnalyticsID trained and determines which ones from participants in the training that shall participate in the inference. The VFL Server has kept the client NF IDs from training and can from that initiate inference. The server and clients (if NWDAFs) are a combined NWDAF containing both MTLF and AnLF. The differences between the VFL training and inference are that there is no check of the labels, and as such not intermediate results are sent to the clients to repeat the process.

If the VFL Server has no suitable model available, in the response to the Analytics Subscription it responds to consumer that training of model is needed. Server starts training. If VFL is needed step 1 is performed in Figure 6.2X.2.2-1. After the training process is complete, the VFL Server NWDAF will send Nnwdaf_AnalyticsSubscibe_Notify that includes the output to the consumer.

NOTE 1: The consumer can at any time unsubscribe to the Analytics subscription. Whether the Server decides to stop the FL is up to Server to decide.

##### 6.2x.2.4.2	Procedure for distributed inference

Figure 6.2x.2.4.2-1: General procedure for distributed inference

Editor's note:	Further extensions are needed to show when any of the Consumer, Server and clients are untrusted AF(s). In this case the procedure below will contain a NEF, and the service operations going via NEF includes the same service operation but as an Nnef service operation in between.

Editor’s Note: Further extensions are needed to the procedure which only describes the Server being NWDAF and clients are also NWDAFs. Extensions includes when untrusted AF is included either as Server or client, the service operations are changed accordingly to be an NFaf service operation.

0.	The consumer sends a subscription request to FL server to retrieve Analytics, using Nnwdaf_AnalyticsSubscription including Analytics ID.

The Server knows that there is an ML Model for an Analytics ID trained.

1.	FL Server selects clients(s) using input from earlier training described in 6.2X.2. The server may not select all clients, e.g. depending on their contribution to the training result or if NWDAF decides to do inference independently of AF(s).

2.	FL Server NWDAF sends a Nnwdaf_ AnalyticsSubscription _Subscribe to the selected clients(s). The request includes ML Model ID, samples IDs.

3.	[Optional] Each FL Client collects its local data.

4.	During distributed inference procedure, each Client performs inference using the local ML Model and reports the intermediate result to the FL Server in Nnwdaf_AnalyticsSubscription_Notify.

The Intermediate results, which is sent from the FL Client(s) to the FL Server during the FL training process, is the information needed by the FL Server to generate the final output.

5.	The FL Server generates the final output.

6a.	The FL Server sends a Nnwdaf_AnalyticsSubscription_Notify message to update the consumer with the requested Analytics.

* * * End of Changes * * *