SA WG2 Meeting #164 	S2-2407539

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1106 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment on ML Model update description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-8-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 10.3.2 clearly states that the Nadrf_MLModelManagement_StorageRequest service operation can be used to update the ML model(s) stored in ADRF, but the description in clauses 5B.1 and 6.2B.5 does not mention ML model(s) update related information. In both Nadrf_MLModelManagement_Delete and Nadrf_MLModelManagement_RetrievalRequest service operations, the storage transaction ID is a mandatory input parameter. Therefore, some ADRF products designed can only recognize storage transaction ID. Without the storage transaction ID in the model update request, some ADRF may not be able to locate and update the stored models. It is proposed to add the transaction ID as an optional input parameter in the Nadrf_MLModelManagement_StorageRequest service operation. After receiving the transaction ID, the ADRF can locate and update a batch of ML models by re-writing its memory in a ‘one-time’ operation. Comparing with the ML model update method in the current specification, in which the ADRF has to locate and update ML models multiple times with ML model IDs(i.e. the ADRF needs to locate and update every single ML model), using transaction ID is much more efficient, especially in the case of large numbers of ML models to be updated in one update request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some rewording to make the service operation more understandable. Alignment of ML Model update description in the specification. Add storage transaction identifier as an optional input parameter in Nadrf_MLModelManagement_StorageRequest service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment in the description of ML model update. ML model update procedure can not be executed in some scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5B.1, 6.2B.5,10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5B.1	General

The ADRF offers services that enable a consumer to store, retrieve and delete data and analytics. The analytics are produced by the NWDAF as described in clause 6.1 and data are collected as described in clause 6.2. Multiple instances of ADRF may be deployed in a network, NF consumer selects a target ADRF instance using ADRF discovery mechanism defined in clause 6.3.20 of TS 23.501 [2].

Data may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageRequest containing the data or analytics to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive data or analytics for storage. The ADRF then subscribes to the NWDAF or DCCF for data or analytics, providing ADRF Notification Address (+Notification Correlation ID). Analytics or Data are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_DataManagement Nnwdaf_DataManagement or Nmfaf_3caDataManagement service).

Data may be retrieved from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve data or analytics for a Storage Transaction Identifier or a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify. The ADRF determines the availability of the data or analytics in its repository and sends in the response to the consumer either the data or analytics.

-	a consumer sending an Nadrf_DataManagement_RetrievalSubscribe request to the ADRF to retrieve data or analytics for a specified data or analytics collection time window. If the time window includes the future and the ADRF has subscribed to receive the data or analytics, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the data or analytics and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_DataManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the data or analytics or provide instructions to the endpoint to fetch the data or analytics using an Nadrf_DataManagement_RetrievalRequest.

Data may be deleted from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_Delete request. The ADRF response provides a result indication.

An ADRF may be configured to register the data it is collecting with a DCCF. The registration uses the Ndccf_ContextManagement service specified in clause 8.3.2. The registration may subsequently be used by the DCCF to obtain data from the ADRF as described in clause 6.2.6.3.6.

The ADRF offers services that enable a consumer to store, delete and retrieve ML Models. The ML Models are trained by NWDAF containing MTLF as described in clause 5.1.

ML Model(s) may be retrieved from the ADRF by:

-	a consumer sending the ADRF an Nadrf_MLModelManagement_RetrievalRequest as described in clause 10.3.4, containing one or more tuples of unique ML Model identifier stored in ADRF or Storage Transaction Identifier. The ADRF response provides one or more tuples of unique ML Model identifiers and ML Model file address of ML Model file stored in ADRF.

ML Model(s) may be stored or updated in the ADRF by:

-	a consumer sending the ADRF an Nadrf_MLModelManagement_StorageRequest as described in clause 10.3.2, containing the ML Model or ML Model address to be stored or updated. The ADRF response provides a result indication.

ML Model(s) may be deleted from the ADRF by:

-	a consumer sending an Nadrf_MLModelManagement_Delete request as described in clause 10.3.3. The ADRF response provides a result indication.

When a consumer requests data or analytics to be stored in an ADRF, it may specify Storage Handling information requesting:

-	a lifetime indicating how long the data or analytics should be stored; and/or

-	that a notification alerting the consumer be sent prior to data deletion from the ADRF.

The ADRF, DCCF or NWDAF may be configured with default operator storage policies. The policies specify how long data or analytics are to be stored and conditions when the default policy supersedes Storage Handling Information provided in a request from the Data or Analytics Consumer. Based on the default operator policy and the Storage Handling information, the ADRF, DCCF or NWDAF determines the Storage Approach that will be applied for the data or analytics.

The Storage Approach is comprised of the lifetime for how long the data or analytics will be stored and the notification requirement to be applied when data or analytics are to be deleted from the ADRF.

When more than one consumer requests a storage lifetime for the same data or analytics, the Storage Approach should be based on the longest requested storage lifetime.

The response to the consumer request for data or analytics includes the Storage Approach.

NOTE:	The default operator policy for how long data or analytics are to be stored may be longer or shorter than the lifetime requested by the consumer. A default operator policy may for example accept only consumer requested lifetimes that are shorter or longer than the default policy.

The ADRF, DCCF or NWDAF determines when the data or analytics lifetime has expired. When data or analytics is to be removed from the ADRF, an alert is sent to the Consumer that the data is about to be deleted. The alert contains a Storage Transaction Identifier that can be used by the consumer to retrieve the data or analytics.

* * * Next Change * * *

6.2B.5	ML Model Storage and Update in ADRF

The procedure depicted in figure 6.2B.5-1 is used by NWDAF containing MTLF to store or update ML model(s) in an ADRF.

Figure 6.2B.5-1: ML Model Storage in ADRF

0.	NWDAF containing MTLF determines to store or update ML model(s) in ADRF based on MTLF policy.

1.	NWDAF containing MTLF requests to store or update a (set of) ML model(s) to the ADRF by invoking Nadrf_MLModelManagement_StorageRequest service.

2.	[Optional] If instead of the ML model(s), the ML model address(es) is/are included in request, ADRF downloads the ML model(s) based on the ML model address(es) and locally stores the ML model(s).

3.	The ADRF sends Nadrf_MLModelManagement_StorageRequest Response message to the consumer including the ML Model storage or ML Model Update result indication.

* * * Next Change * * *

10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML Model(s). ML Model(s) or ML Model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and set of:

-	(Updated) ML model(s); or

-	the tuple (unique ML Model identifier and address (e.g. URL or FQDN) of (updated) Model file and (updated) Storage size required for each of the ML Model).

NOTE 1:	The ADRF can download ML Model files based on the ML Model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML Model and locally stores or updates the ML Model is left for implementation.

Inputs, Optional: [Conditional] Storage Transaction Identifier.

NOTE 2: The Storage Transaction Identifier Input is for ML Model(s) update only.

Outputs Required:

-	Result Indication;

-	[Conditional] one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of (updated) ML Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

* * * End of Changes * * *