3GPP TSG-SA2 Meeting #164	S2-2407567

# 19 – 23 August 2024, Maastricht, Netherlands							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1114 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI2: Vertical Federated Learning Registration, Discovery and Sample Alignment where FL Server is NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for Vertical Federated Learning Registration, Discovery, Sample Alignment and Selection of FL clients to participate in vertical federated learning taking into account the agreed Principles P#2.1 through P#2.5 in clause 8.2 of 3GPP TR 23.700-84 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding support of registration and discovery for vertical federated learning in a new sub-clause under clause 6.2C.2.1. For selecting candidate FL clients a new VFL interoperability information field is proposed to be included to allow the FL server to select compatible FL clients. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on Registration, Discovery, Sample Alignment and Selection of FL clients for vertical federated learning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.1, 6.2C.2.1, 6.2C.2.1.x (new), 6.2C.2.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning (Horizontal or Vertical) technique to train an ML model.

### 6.2C.2	Procedures

#### 6.2C.2.1	Registration and Discovery procedure for Federated Learning

##### 6.2c.2.1.x	Support for horizontal federated learning

Figure 6.2C.2.1.x-1: Registration and Discovery procedure for Horizontal Federated Learning

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Steps 4 to 6 are the NWDAF Discovery procedure.

4-6.	NWDAF containing MTLF determines ML model requires FL based on operator policy (e.g. pre-configured list of ML models), Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, the FL Server NWDAF discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local model training, Time Period of Interest, ML Model Interoperability Indicator.

7.	FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML model training requirement (e.g. Analytics ID, ML Model Interoperability information, Available data requirement, Availability time requirement (time span needed for the FL process), etc.). Available data requirement includes a list of Event IDs of the local data for training, and may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples.

NOTE:	Federated Learning preparation procedure (i.e. steps 7-9) can be skipped if the FL Server NWDAF can decide that the FL Client NWDAF(s) supports the FL procedure to be performed, e.g. based on information acquired from previous FL procedures or from the NRF, or based on local configuration.

8.	FL Client NWDAF(s) checks if it can meet the ML model training requirement and/or can successfully download the model if the model information is provided in the request and decides whether to join the Federated Learning process based on operator policy (e.g. pre-configured list of ML models) and/or implementation. Example criteria used by FL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and ML Model Interoperability information.

9.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response service operation or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the FL Server NWDAF whether it will join the FL procedure and may include the reason in the response message if it cannot join the FL process.

10.	FL Server NWDAF determines the final list of FL Client NWDAF(s) to be involved in the FL procedures based on the information received in step 6 and other information received in step 9 (if available).

.

************************************ SECOND CHANGE ****************************************

##### 6.2C.2.1.y	Support for Vertical Federated Learning where FL Server is NWDAF

Figure 6.2C.2.1.y-1: Registration and Discovery procedure for Vertical Federated Learning where FL server is NWDAF

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server or FL client), VFL interoperability information and Time interval supporting FL as described in clause 5.2.

Steps 4 to 6 are AF registration procedure via NEF

4-6.	NEF registers to NRF its NF profiles which includes Analytics ID, Service Area and FL capability type information (i.e. FL server or FL clients).

NOTE X: How the NEF interfaces with AF to obtain FL capability type is out of scope of 3GPP.

Steps 7 to 9 are the NWDAF and/or AF via NEF Discovery procedure.

7-9.	NWDAF containing MTLF determines ML model requires vertical federated learning based on operator policy, Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as VFL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, VFL interoperability information, FL capability Type (i.e. FL server), Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, the FL Server NWDAF discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) and AF acting as an FL client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local model training, Time Period of Interest, VFL Interoperability Indicator.

Steps 10 to 18 are the sample alignment procedure

10.	FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s) and/or AF (via NEF), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML model training requirement (e.g. Analytics ID, Available data requirement, Availability time requirement (time span needed for the FL process), etc.). Available data requirement includes a list of Event IDs of the local data for training.

11.If the FL preparation request is to the AF the request may be received via an NEF. The NEF identifies the AF to forward the request based on the contents of the request

12.	The NEF forwards the FL Preparation Request to the AF

13.	The AF identifies common samples and decides to join or not in the VFL process

14.	The AF responds including the available data and an indication to join or not the VFL process

15.	The NEF forwards the response to the FL Server NWDAF.

16. In parallel to steps 12 to 15 the candidate NWDAF FL clients identify common sampels and decide to join or not in the VFL process. Example criteria used by FL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and VFL Interoperability information.

17.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response service operation or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the FL Server NWDAF whether it will join the FL procedure and may include the reason in the response message if it cannot join the FL process. The FL client NWDAF(s) include in the response the available data (i.e. Event IDs as a list of Features and Samples) if they decide to join the VFL process.

18.	FL Server NWDAF determines the final list of FL Client NWDAF(s) to be involved in the VFL procedures. The FL Server NWDAF assigns a VFL process ID corresponding to the VFL process and determines initial contribution weights from each VFL participant according to the data availability and/or importance of the feature for the VFL process.

************************************ END OF CHANGES ****************************************