3GPP TSG SA WG 2 Meeting 164		S2-2407572

Maastricht, Netherlands, 19 – 24 August 2024

Title:	DraftLS on Data collection for training LMF-side models

Response to:

Release:	Release 19

Work Item:	FS_AIML_CN

Source:	Lenovo (to be SA WG 2)

To:	RAN, RAN WG1

Cc:	RAN WG2

Contact person:			Dimitrios Karampatsis

dkarampatsis AT lenovo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	3GPP TR 23700-84 (v0.2.0)

# 1	Overall description

3GPP SA2 has started defining normative procedures for the support of training LMF-side models based on agreed principles as described in clause 8.1 of 3GPP TR 23.700-84.

3GPP SA2 has agreed that either the LMF, as per Principle #2, or the  NWDAF, as per Principle #3, performs the model training.

If the LMF performs the model training the procedure to collect data for training is implementation specific whereas if the NWDAF performs the training then MTLF obtains data for training from a data repository or other data sources as per Principle #3.3.

For the data used for model training, inference and model performance monitoring Principle #5 states that the related procedure for data collection will be coordinated with RAN WGs in the normative phase.

Principle #5:  Data used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs. The related procedures for data collection will be coordinated with RAN WGs in the normative phase.

3GPP SA2 is currently discussing the the method to support data collection by the NWDAF. There are different options as follows:

- NWDAF collecting training data via GMLC and re-using LPP/NRPPa protocols for data collection from UE/RAN as per Solution#1, 11

- NWDAF collecting training data directly from the LMF by defining a new service and re-using LPP/NRPPa protocol for data collection from UE/RAN as per Solution #4

- NWDAF collecting training data directly from the AMF by defining a new service and new protocol over N2 for data collection from UE and RAN as per Solution 7.

Question to RAN WG1

In order to assist in the decision on the data collection method, 3GPP SA2 kindly ask RAN WG1 to provide feedback whether the LPP/NRPPa protocol can be re-used for data collection from UE/RAN or a new protocol is required.

# 2	Actions

To RAN, RAN WG1

ACTION: 	SA2 respectfully asks RAN & RAN WG1 to provide feedback on the questions raised.

# 3	Dates of next TSG SA WG 2 meetings

SA2#165	14 - 18 October 2024, 		Hyderabad, India

SA2#166	18 - 22 November 2024, 		Orlando, USA