SA WG2 Meeting #164	S2-2407597

# Maastricht, Netherlands, 19 – 23 August, 2024                  (revision of S2-240xxxx)

Source: 	CATT

Title: 	Discussion and conclusion proposals for Topology 1 procedures

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper discussed Topology 1 procedures and made some conclusion proposals.

1	Discussion

Based on the solutions documented in TR 23.700-13 and RAN2 agreements, it can be seen that the following three cases may be supported for AIoT Services:

- Inventory only

- Command only

- Inventory and Command

For Inventory only and Command only, the high level procedure based on existing solutions is illustrated in Figure 1.

Figure 1: High level procedure for Inventory only and Command only

1. AF sends AIoT Service Request message to NEF, and the Request message includes Inventory only or Command only service operation, Device ID info and Area info, etc.

2. NEF authorizes the AF request and determines the AIoTF for AIoT Services.

3. NEF sends AIoT Service Request message to AIoTF, and the Request message includes Inventory only or Command only service operation, Device ID info and Area info, etc.

4. AIoTF determines the A-RAN for AIoT Services.

5. AIoTF sends AIoT Service Request message to A-RAN, and the Request message includes Inventory only or Command only service operation, Device ID info, etc.

6. A-RAN sends Initial Trigger Message (Device ID info) to AIoT Device.

7. AIoT Device performs Random Access procedure.

8. AIoT Device sends AIoT Service Response message to AIoTF via A-RAN. For Inventory only operation, the Response message includes Device ID. For Command only operation, the Response message includes Device ID and Command Result info.

9. AIoTF performs validation and/or authentication of AIoT Device.

10. AIoTF registers AIoT Device in to UDM.

11-12. AIoTF sends AIoT Service Response to AF via NEF.

For Inventory and Command, the high level procedure based on existing solutions is illustrated in Figure 2.

Figure 2: High level procedure for Inventory and Command

1. AF sends AIoT Service Request message to NEF, and the Request message includes Inventory and Command service operation, Device ID info and Area info, etc.

2. NEF authorizes the AF request and determines the AIoTF for AIoT Services.

3. NEF sends AIoT Service Request message to AIoTF, and the Request message includes Inventory and Command service operation, Device ID info and Area info, etc.

4. AIoTF determines the A-RAN for AIoT Services.

5. AIoTF sends AIoT Service Request message to A-RAN, and the Request message includes Inventory and Command service operation, Device ID info, etc.

6. A-RAN sends Initial Trigger Message (Device ID info) to AIoT Device.

7. AIoT Device performs Random Access procedure.

8. AIoT Device sends AIoT Service Response message to AIoTF via A-RAN, and the Response message includes Device ID.

9. AIoTF performs validation and/or authentication of AIoT Device.

10. AIoTF registers AIoT Device in to UDM.

11. AIoTF sends Command Request message to AIoT Device via A-RAN.

12. AIoT Device sends Command Response message to AIoTF via A-RAN, and the Response message includes Command Result info.

13-14. AIoTF sends AIoT Service Response message to AF via NEF.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1	Conclusions for Key Issue #3: Support of Ambient IoT Services

For Key Issue #3, the following principles are concluded:

-	Three AIoT Services operations are supported: Inventory only, Command only, Inventory and Command.

-	AF request one of the AIoT Services operations to AIoTF via NEF.

-	AIoTF requests one of the AIoT Services operations to A-RAN.

-	AIoT Device responds to AIoTF with service operation result.

-	AIoTF responds to AF via NEF with service operation result.

# * * * * End of Changes * * * *