3GPP TSG-WG SA2 Meeting #164 	S2-2407658

Aug. 19 – 23, 2024, Maastricht, NL	(revision of S2-240xxxx)

Source:	Intel

Title:	KI#3: Conclusion update

Document for:	Approval

Agenda Item:	19.9.1

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract: This paper proposes conclusion update for KI#3.

# 1. Discussion

This contribution proposes conclusion update for KI#3 on how to route EC traffic between local DN and central DN.

Currently there are two solutions (i.e. Sol#17 for CAT-A and Sol#21 for CAT-B) for further consideration per the interim conclusion for KI#3.

For Sol#17, Security concern was raised by several companies.

  - Security Aspect

Security concern was raised for Sol#17 on the EAS sending traffic using the UE IP address as Source IP address after handling the traffic with EAS IP address as the Destination IP address and UE IP address as the Source IP address. After being processed at EAS, the EAS replaces the Destination IP address/Port Number of the uplink traffic with the central AS’ and send back to L-PSA UPF. Since the EAS is under the control of AF and SP under SLA, there should be no security issue.

While for the traffic with central AS IP address as the Destination IP address and UE IP address as the Source IP address, if the traffic is not encrypted, after firstly being processed by EAS, it can be forwarded to L-PSA UPF with the IP header unchanged. If the traffic is encrypted, the EAS is unable to process the traffic, it will be simply sent back to the L-PSA UPF with the IP header unchanged. This kind of traffic can be routed through the data path of the UE’s PDU Session without security concern.

  - Charging Aspect

From UE perspective, it has no idea whether it’s communicating with local EAS or central AS. For the uplink traffic with EAS IP address as the Destination IP address, the UE would also expect it to be charged by the operator, it doesn’t matter whether the traffic goes through L-PSA UPF or C-PSA. For roaming scenario, the Charging system in HPLMN should be able to consolidate the Charging Data record by coordinating with the Charging system in VPLMN to ensure there is no over-charging issue.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

## 8.3	Conclusion for KI#3

Conclusion:

-	The traffic routing rule between local DN and central DN should be controlled by SMF, which can be influenced by AF.

-	The requirement of establishing connectivity for traffic routing between local DN and central DN should be provided by AF.

-	QoS should be supported for the traffic routed between local DN and central DN.

-	Charging should be supported for the traffic routed between local DN and central DN, any charging issue should be verified with SA WG5.

-	The UL CL UPF may be enhanced to support classifying and routing uplink traffic from L-PSA UPF to C-PSA UPF based on the Destination IP address/Destination Port number, downlink traffic from C-PSA UPF to L-PSA UPF based on the Source IP address/Source Port number of the downlink traffic as described in Solution #17.

# * * * * End of changes * * * *