3GPP SA WG2 Meeting #164	S2-2407680

19-23 August 2024, Maastricht, NL

Title:	    [DRAFT] LS reply on LS on RAN2 agreements and assumptions for Ambient IoT

Response to:	LS on RAN2 agreements and assumptions for Ambient IoT

Release:	Rel-19

Work Item:	FS_Ambient_IoT_solutions, FS_AmbientIoT

Source:	SA2

To:	RAN2

Cc: 	RAN3, SA3

# Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank RAN2’s LS(S2-2407447/ R2-2406150) on RAN2 agreements and assumptions for Ambient IoT.

- RAN2 will study the following cases for A-IoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

For above agreement, SA2 would like to confirm both AIoT device and Group ID can be used to filter AIoT devices, but there is also some other filter information which is different from per device ID and group ID but captures the information used to filter AIoT devices. Hence, the above three can be categorized as a filter information, Including the one of the following:

  - per AIoT device ID list,

  - Group ID,

  - Match information, which is different from per device ID and group ID but captures the information used to filter AIoT devices.

SA2 also would like RAN2 to feedback whether the following AF provided information is useful for RAN to perform AIoT operation or not

  - The number of target AIoT devices;

  - The characteristics of the target AIoT devices, e.g. whether UL and/ DL amplification are supported by AIoT device, whether the device’s UL transmission is generated internally by the device, or be backscattered on a carrier wave provided externally.

2. Actions:

RAN2:

ACTION: 	SA2 kindly asks RAN2 to provide feedback on above.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #165		14-18 October 2024	Hyderabad, IN

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, FL, US