3GPP TSG-WG SA2 Meeting #164	S2-2407694

Maastricht, Netherlands, August 19 - 23, 2024	(revision of S2-240xxxx)

Source:	China Mobile

Title:	TR 23.700-66 KI#1 Conclusion update based on the way forward proposal

Document for:	Approval

Agenda Item:	19.4.1

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This pCR proposes to update the conclusion of KI#1.

1	Discussion

This paper provides conclusion update of KI#1: Network energy related information exposure.

2	Proposal

It is proposed to include the below changes into TR 23.700-66.

First Changes

8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

1)	A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator's policy:

-	If the authorized consumer is AF, the granularities include: per per UE.

-	If the authorized consumer is 5GC NFs, the granularities include: per UE.

-	The energy related information for the granularities above include:

-	Energy consumption information;

-	Renewable energy information.

2)	The consumer NFs including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

3)	The information for the calculation of the Energy Consumption information is obtained from the following sources:

If RAN cannot report the energy consumption information per UE level:

a)	OAM: provides the energy consumption information at the gNB(s) and UPF(s) serving the UE;

b)	OAM: provides the overall data volume for the gNB;

c)	UPF: provides the overall data volume of the UPF;

d)	UPF: provides the data volume for the specific UE.

If RAN can report the energy consumption information per UE level:

a)	OAM: provides the energy consumption information at the UPF(s) serving the UE;

b)	UPF: provides the overall data volume of the UPF;

c)	UPF: provides the data volume for the specific UE;

d)  RAN: provides the energy consumption information for the specific UE.

Editor's note: Whether in this release the NG-RAN will provide per UE level energy consumption information and state depending on RAN WGs. SA WG2 will only start the work based on RAN decision to work on energy efficiency topic.

4)	The new functionality determines the E2E energy consumption based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF).

5)	In the current release of the specification, only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 1:	The exposure and collection of renewable energy information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

Editor's note: Whether in this release the NG-RAN will provide per UE level energy consumption information and state depending on RAN WGs. SA WG2 will only start the work based on RAN decision to work on energy efficiency topic.

NOTE 3:	This work will coordinate with SA WG5 for OAM and charging part.

End of Changes