3GPP TSG-WG2 Meeting #164	 S2-2407827

# Maastricht, Netherlands, 19 August – 23 August, 2024                   (revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0238 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I-SMF based Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR intends to support I-SMF based local offloading mangement based on the conclusion of clause 8.1 in TR 23.700-49. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the reference architecture for I-SMF based local offloading mangement supporting Edge Computing with UL CL/BP. - Add the descriptions/procedures for support of I-SMF based local offloading mangement. - Add related information/description for EASDF service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new functionality of I-SMF based local offloading mangement can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.x(new), 7.1.1, 7.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR S2-2407828 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.502 CR S2-2407829 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> First change <<<<

4.2	Reference Architecture for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501 [2]. The following reference architectures for non-roaming, LBO roaming and HR with Session Breakout (HR-SBO) roaming scenarios further depict the relationship between the 5GS and a DN where Edge Application Servers (EASs) are deployed in an EHE.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

Figure 4.2-5 depicts 5GS architecture for HR-SBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-5: 5GS providing access to EAS with UL CL/BP for HR-SBO roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

NOTE 3:	For the HR-SBO roaming scenario, there can be other UPF(s) located in VPLMN between the UPF acting UL CL/BP and the UPF acting as remote PSA in HPLMN.

Figure 4.2-6 depicts 5GS architecture for I-SMF based local offloading mangement supporting Edge Computing with UL CL/BP.

Figure 4.2-6: 5GS providing access to EAS with UL CL/BP for local offloading mangement scenario

>>>> Second change <<<<

6.X 	Support of I-SMF based Local Offloading Mangement

6.x.1	General

In order to reduce the impact on central 5GC NFs (e.g. central SMF) for the purpose of EAS (re)discovery and local UPF (re)selection, the I-SMF can be inserted to perform local offloading mangement, including:

-	EASDF discovery and selection.

-	EASDF configuration and DNS message handling.

-	EAS Deployment Information management.

This local offloading mangement mechanism is only applied for Session Breakout model. For Distributed Anchor Point and Multiple PDU Session models, the existing mechanisms shall be used.

The reference architecture supporting this scenario is depicted in Figure 4.2-6 in clause 4.2.

6.x.2	Procedure for PDU Session Establishment and EAS discovery supporting Local Offloading Mangement

Figure 6.x.2-1: PDU Session Establishment and EAS discovery supporting Local Offloading Management

1.	During the Registration procedure, the AMF receives the Local Offloading Management allowed indication from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure for non-roaming and roaming with local breakout as in clause 4.3.2.2.1 of TS 23.502 [3], if the AMF receives the Local Offloading Management allowed indication in the step 1, the AMF selects I-SMF for local offloading management as described in clause 5.34.3 of TS 23.501[x]. Even if the service area of serving SMF can include the current UE location, but AMF determines the UE location is in the service area of the local offloading mangement (e.g. the service area of local offloading mangement is pre-configured), and the AMF may insert the I-SMF.

3.	The AMF sends the Local Offloading Mangement allowed indication to the selected I-SMF via Nsmf_PDUSession_CreateSMContext Request. AMF receives Nsmf_PDUSession_CreateSMContext Response.

4.  The I-SMF may select UL CL/BP UPF and L-PSA UPF based on UE location and Local Offloading Mangement Allowed Indication.

5.  The I-SMF may be pre-configured with the local offloading policy, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN. Also, the I-SMF may request the local offloading policy from PCF via SMF by sending the Local Offloading Mangement request indication via Nsmf_PDUSession_Create Request to the SMF.

6.	If the SMF receives the Local Offloading Mangement request indication, the SMF may request and retrieve the Local Offloading Policy from PCF by SM Policy Association Establishment/Modification with the Local Offloading Mangement request indication as indicated in clause 5.2.5.4.2 of TS 23.502 [3]. If local offloading mangement is authorized successfully via retrieving subscription information from UDR, the PCF provides the local offloading policy.

7.	The SMF sends the local offloading policy to I-SMF via Nsmf_PDUSession_Create Response.

8.  The steps 2-4 of clause 6.2.3.2.2 are performed by replacing SMF with I-SMF with following modifications:

In step 2, I-SMF selects EASDF for local offloading mangement, and sends N4 Session Modification to configure the local UPF for local offloading mangement based on the local offloading policy.

9.  I-SMF retrieves EAS Deployment Information as defined in clause 6.2.3.4.

10. The steps 5-19 of clause 6.2.3.2.2 are performed by replacing SMF with I-SMF with following modifications:

In step 5, I-SMF configures EASDF for DNS message handling based on retrieved EAS Deployment Information and local offloading policy.

>>>> Third change <<<<

7.1.1	General

The following table illustrates the EASDF Services and Service Operations.

Table 7.1.1-1: NF services provided by the EASDF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neasdf_DNSContext | Create | Request/Response | SMF, I-SMF |
|  | Update | Request/Response | SMF, I-SMF |
|  | Delete | Request/Response | SMF, I-SMF |
|  | Notify | Subscribe/Notify | SMF, I-SMF |
| Neasdf_BaselineDNSPattern | Create | Request/Response | SMF, I-SMF |
|  | Update | Request/Response | SMF, I-SMF |
|  | Delete | Request/Response | SMF, I-SMF |

>>>> Fourth change <<<<

7.1.2.1	General

Service description: This service enables the consumer to create, update, or delete DNS context in EASDF and to Subscribe to DNS message related reporting from EASDF.

DNS contexts in EASDF include rules on how EASDF is to handle DNS messages.

This service also can be supported by V-EASDF in VPLMN for HR scenario supporting HR-SBO.

For the case of local offloading mangement, this service can be invoked by I-SMF.

>>>> End of Changes <<<<