3GPP TSG-WG2 Meeting #164	S2-2407871

19th – 23rd August 2024, Maastricht, Netherlands

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0126 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Pre-Mission Planning and In-Mission Monitoring Flight |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding support for Pre-Mission Planning and In-Mission Monitoring Flight based on the conclusions agreed in TR 23.700-59 clause 8.1.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added new clause 5.X for Pre-Mission Planning and In-Mission Monitoring Flight. - Added high-level functional description. - Added procedure for NEF Assist Pre-mission Flight Planning. - Added procedure for NEF Assist In-mission Flight Monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed Rel-19 feature is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.X(new), 5.X.1(new), 5.X.2(new), 5.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[16]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[17]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[18]	3GPP TS 26.502: "5G multicast-broadcast services; User Service architecture".

[19]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[Y]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

>>>> Next Change <<<<

## 5.X	Pre-Mission Planning and In-Mission Monitoring Flight

### 5.X.1	Functional Description

This clause provides pre-mission planning and in-mission monitoring flight mechanism. It is based on NEF services to support service exposure and interactions between MNOs and UTM functions for supporting pre-mission flight planning and in-mission flight monitoring for UAVs. The NEF services enable USS/UTM to request NEF assistance for pre-mission flight planning and in-mission flight monitoring.

For NEF Assist Pre-mission Flight Planning, the Movement Behaviour Analytics and QoS Sustainability Analytics are used with enhancement by including height information of UAV for the analytics generation. The NEF service is enhanced to provide assistance information for pre-mission flight planning, e.g. the assistance information may be the best matched route among the ones provided from the USS/UTM, or potential flight route(s) if candidate flight route(s) is not provided from the USS/UTM. One time request from the USS/UTM to NEF for the assistance information, i.e. request/response mechanism, is used.

For NEF Assist In-mission Flight Monitoring, USS/UTM subscribes to the network for UAV location for periodical notification and based on the collected information to determine e.g. whether UAV in flight is in right route/any deviation happened. USS/UTM requests QoS Sustainability Analytics containing the UAV flight path information defined as 3D location waypoints.

>>>> Next Change <<<<

### 5.X.2	Procedure for NEF Assist Pre-mission Flight Planning

Figure 5.X.2-1: Procedure for NEF Assist Pre-mission Flight Planning

1.	The UAV establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3.

2.	The UAV (or via its paired UAV-C) requests Pre-mission flight planning service from USS/UTM via Nnef_Assist_Request service operation. The request message includes identifier of the UAV (e.g. GPSI, CAA-Level UAV ID), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), and may include candidate flight route(s) if available.

3.	The USS/UTM derives information for the Pre-mission flight planning request and decides to request assistance information from NEF.

NOTE 1:	The content of Pre-mission flight planning service information derived at USS is out of scope.

4.	The USS/UTM sends Pre-mission flight planning assist request to NEF. The request message includes identifier of the UAV (e.g. GPSI), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), candidate flight route(s) (received from the UAV or local derived at the USS/UTM), and accuracy level of predictions relevant to the flight planning.

5.	The NEF maps parameters included in the request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (Movement Behaviour analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	If the NEF cannot satisfy the requirements on flight planning (e.g. cannot get predictions relevant to the flight planning from NWDAF with the required accuracy level), the NEF responses to the USS/UTM to reject the Pre-mission flight planning assist request, and may include the detail reason.

7.	If being rejected in step 6, the USS/UTM may response to the UAV (or its paired UAV-C) to reject the Pre-mission flight planning request, may include the detail reason if available.

8.	The NEF performs steps 2-5 in clause 5.3.4, and filter list of UAVs in the areas of interest.

9.	The NEF subscribes/requests for notification on Movement Behaviour Analytics provided by NWDAF as defined in clause 6.21.4 of TS 23.288 [Y] and QoS Sustainability Analytics provided by NWDAF as defined in clause 6.9.4 of TS 23.288 [Y]. The subscribe/request message include identifier of the UAV (e.g. GPSI) obtained in step 4. The other parameters included in the request are described in TS 23.288 [Y]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the NWDAF on Movement Behaviour Analytics and QoS Sustainability Analytics.

10.	The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as defined in clause 6.20.3 of TS 23.273 [8]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the GLMC service on Ranging/Sidelink Positioning location.

11.	The NEF generates assistance information for pre-mission flight planning based on the information and analytics from steps 8-10. The assistance information may be the best matched route among the ones provided from the USS/UTM in step 4, or potential flight route(s) if candidate flight route(s) is not provided in step 4.

12.	The NEF responses to the USS/UTM via Nnef_Assist_Response service operation with the assistance information.

13.	The USS/UTM decides flight planning by using the assistance information and flight planning mechanism which is out of scope.

14.	The USS/UTM sends response to the UAV (or its paired UAV-C) with the planned flight route.

NOTE 2:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 5.X.1-1, the consumer can also be other entities, e.g. TPAE.

>>>> Next Change <<<<

#### 5.X.3	Procedure for NEF Assist In-mission Flight Monitoring

Figure 5.X.3-1: Procedure for NEF Assist In-mission Flight Monitoring

NOTE 1:	The NFs in Figure 5.X.3-1 represents all the other NFs beside GMLC and NEF/UAS NF involved in the procedure.

1.	The USS/UTM sends In-mission flight monitoring assist request to NEF via Nnef_Assist_Subscribe service operation. The request message includes identifier of the UAV (e.g. GPSI) and information of the flight route, monitoring mode (e.g. UAV location monitoring, fight route monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring), report requirements (e.g. report format (event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results)), and may include planned fight route if fight route monitoring, list of UAVs in the areas of interest if flight environment monitoring, preset value for distance if Ranging/Sidelink positioning location monitoring.

2.	NEF maps the parameters in the request from the USS/UTM to information used by the 3GPP system. The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location).

3.	If the NEF cannot provide the requested assistance information for In-mission flight monitoring, the NEF responses to the USS/UTM to reject the In-mission flight monitoring assistance request, may include the detail reason if available.

4.	For UAV location monitoring, the NEF executes steps 2-5 in clause 5.3.2 to obtain the location of the target UAV.

5.	For fight route monitoring, the NEF executes steps 2-4 in clause 5.3.3. Upon to the request in step 1 for reporting, the NEF may compare the monitored flight route with the planned fight route (if provided in step 1) and generate assistance information on whether the UAV is flight in right route.

6.	For flight environment monitoring, the NEF executes steps 2-5 in clause 5.3.4 to obtain the information of aerial UEs in the same geographic area of the target UAV.

7.	For Ranging/Sidelink positioning location monitoring, the NEF requests GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [8], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation the monitoring, steps 21-31 for monitoring periodic). The list of UAVs in the areas of interest output from step 6 can be used as inputs to step 7 for NEF to request GLMC service for Ranging/Sidelink Positioning location, if the UAVs is unknown in the areas of interest.

8.	The NEF subscribes/requests for notification on QoS Sustainability Analytics provided by NWDAF as defined in clause 6.9.4 of TS 23.288 [Y].

9.	The NEF notifies the USS/UTM via Nnef_Assist_Notify service operation the monitoring results (e.g. the location of the target UAV, indication on whether the UAV is flight in right route, the target UAV presence in areas of interest, the information of aerial UEs in the same geographic area of the target UAV, the distance of other UAVs and the target UAV and whether the distance is smaller than a preset value, the outputs of the QoS Sustainability Analyitcs).

Steps 4-9 may be repeated for report monitoring results periodically.

NOTE 2:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 5.X.2-1, the consumer can also be other entities, e.g. TPAE.

>>>> End of Changes <<<<