3GPP SA WG2 Meeting #164	S2-2407964

Maastricht, Netherlands, August 19-23, 2024

Source:	Ericsson

Title:	KI#1.X: Conclusions on DualSteer

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution:  This is a proposal for conclusions for DS study with focus on network aspects. Two options are considered: With and without AMF impacts.

# Discussion

The following observations are used to derive the proposed conclusions for DualSteer.

Observation 1: A common SMF and anchor UPF should be used for both legs of the “DualSteer session”, which simplifies the management of the session and user plane and avoids un-necessary inter-NF signalling.

Observation 2: There is a need for the network to be able to link the two PDU Sessions for the two SUPIs. This is needed for different reasons and in different NFs. For example, when the second PDU Session is established (for SUPI2), the AMF2 needs to be able to select the same SMF as was selected for the first PDU Session. Furthermore, the SMF needs to be able to associate the CreateSMContext request (with a PDU Session Establishment request) from AMF2/SUPI2 with the already existing PDU Session that was established for SUPI1 from AMF1 without further interactions with other NFs (to avoid un-necessary signalling and speed up the session management for DS). There are different options for how this can be done, with different impacts to UE, AMF and SMF. Below we summarize a few options that fulfil the requirements:

Option A: With no AMF impact:

- 	Option A1: Linked SUPI, common PDU Session ID and SMF double registration in UDM

After receiving the first PDU Session Establishment request, the SMF registers the PDU Session in UDM for both SUPI1 and SUPI2 (i.e. two separate UECM registrations).

When receiving the PDU Session Establishment request for SUPI2, the AMF2 can select the same SMF as per current standards. This option requires that the UE uses the same PDU session ID for both requests (PDU Session ID is a required input of Nudm_UECM_Registration service and cannot be updated later).

The UE includes the Linked SUPI with all PDU Session Establishment Requests in case DS is used (it is assumed that the DS device is aware the two SUPIs that are used for DualSteering). The Linked SUPI allows the SMF to use a common identifier to link the sessions for both PDU Sessions of SUPI1 and SUPI2, and independently of whether the UE registers or establishes the PDU Session first with SUPI1 or SUPI2.

- 	Option A2: SMF-generated Correlation ID, common PDU Session ID and SMF double registration in UDM

SMF selection takes place in the same way as described for Option A1, i.e. with a common PDU Session ID.

A Correlation ID is generated by SMF and provided to the UE in the first PDU Session Establishment procedure. The UE then includes this Correlation ID in any subsequent PDU Session Establishment request messages that is associated to the two linked PDU Sessions, allowing the SMF to associate the PDU Session Establishment request to the existing PDU Session in SMF.

Option B: With AMF impacts:

- 	Option B1: UE-generated Correlation ID provided to the AMF at PDU Session Establishment Request

This option allows different PDU Session IDs for the two SUPIs.

The DS device generates a Correlation ID and includes it in the MM part of the NAS message for the first PDU Session establishment request. The AMF provides Correlation ID and the linked SUPI to the SMF in the CreateSMContext, allowing it to make the session context accessible for all SM requests from both SUPIs. The SMF includes the UE-generated Correlation ID in registration of the first PDU Session in UDM.

In the second PDU Session Establishment request, the UE includes the Correlation ID in the MM part of the NAS message. The AMF retrieves the UE context from UDM to find the already selected SMF using linked SUPI and UE-generated Correlation ID. The AMF then includes the linked SUPI and UE-generated Correlation ID in the CreateSMContext to SMF, allowing SMF to identify the existing PDU Session Context.

- 	Option B2: SMF-generated Correlation ID, stored in UDM, and provided by UE

This option allows different PDU Session IDs for the two SUPIs.

When the first PDU Session is established (SUPI1), the SMF generates a Correlation ID and provides it to the UE in the PDU Session Establishment Accept. SMF also provides the Correlation ID in a new parameter in the PDU Session registration towards UDM. This new parameter is provided by UDM to AMF as part of the UE context in SMF data.

The UE provides the SMF-generated Correlation ID in the MM NAS part of the second PDU Session Establishment request (SUPI2) and the AMF2 obtains the linked SUPI (SUPI1) from the SUPI2 AM subscription data, and can then get SMF ID from UDM using SUPI1 and the SMF-generated Correlation ID.

The AMF then includes the linked SUPI and SMF-generated Correlation ID in the CreateSMContext to SMF, allowing SMF to identify the existing PDU Session Context

- 	Option B3: SMF-generated Correlation ID, stored in UDM, and provided by AMF, common PDU Session ID

This option requires common PDU Session IDs for the two SUPIs.

When the first PDU Session is established (SUPI1), the SMF generates a Correlation ID and provides it in a new parameter in the PDU Session registration towards UDM. This new parameter is provided by UDM to AMF1 as part of the UE context in SMF data.

After receiving the second PDU Session Request (SUPI2), the AMF2 can select the same SMF based on the PDU Session ID and the linked SUPI.

The AMF then includes the linked SUPI and the Correlation ID in the CreateSMContext to SMF, allowing SMF to identify the existing PDU Session Context

The following table summarises the options and the new parameters or requirements for each option.

Table 1: Summery of all options

| Options Characteristics | A1 | A2 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- | --- |
| UE AMF | - | - | UE-generated Correlation ID | SMF-generated Correlation ID | - |
| UE SMF | Linked SUPI | SMF-generated Correlation ID | - | - | - |
| SMF UE | - | SMF-generated Correlation ID | - | SMF-generated Correlation ID | - |
| AMF SMF | - | - | UE-generated Correlation ID And Linked SUPI | SMF-generated Correlation ID And Linked SUPI | SMF-generated Correlation ID And Linked SUPI |
| AM subscription data | - | - | Linked SUPI | Linked SUPI | Linked SUPI |
| SM Subscription data | Linked SUPI | Linked SUPI | - | - | - |
| UE context in SMF data | - | - | UE-generated Correlation ID | SMF-generated Correlation ID | SMF-generated Correlation ID |
| Requires double registration of the PDU session | Yes | Yes | No | No | No |
| Requires common PDU Session ID | Yes | Yes | No | No | Yes |

Observation 3: The DS device indicates its capability for Dual Steering and Simultaneous transmission to the network in both registration requests. The following two options are considered:

- 	Option A:  With no AMF impact: the UE adds the above capabilities to the UE Policy Containers

- 	Option B: With AMF impact: the UE adds the above capabilities to the registration request and AMF sends these parameters to UE PCF if it (or the VPMLN) supports DS.

The above UE capabilities are taken into account by UE PCF in generating the UE policies.

Observation 4. To provide seamless traffic switching, IP address handling and avoid UPF impacts, a DS session should be managed by a single N4 session.

Observation 5: To provide a consistent set of policy rules for DS session during switching, a DS session should be associated by a single Policy Association session.

Observation 6: To enable detecting the linked Subscription profiles in the 5GC, the subscription data for SUPI1 in UDM shall include a new field that contains SUPI2 (and vice versa).

Observation 7: Compared to switching traffic on PDU session level, maintaining the UP connectivity for both 3GPP accesses at the same time with possibility to switch traffic flows between the two 3GPP accesses has the following advantages: 1) it enables seamless switching between the accesses and avoids interruption of services, 2) it requires less control plane signalling and management for switching, 3) it leverages the established principles of ATSSS, 4) it reduces the number of PDU sessions needed, since a single PDU Session can carry traffic flows with different switching policies.

Observation 8: As a consequence of observation 7, the steering/switching is done withing the same PDU session and should be controlled by the network via SM policies/rules, which is provided to the UE after creating the DS session. Based on URSP rules, the DS device initiates PDU Session Establishment in both 3GPP accesses (if the DS device is registered in both accesses) in case an application is to be using DualSteer. If the UE is only registered in one 3GPP access, the UE establishes a PDU Session in that access.

Observation 9: Since both of the SUPIs used in DS have the same HPLMN and serve the same subscriber, differentiation between the two legs in steering/switching rules should be done based on how they connect to the network. Using SUPIs as identifiers in steering/switching rules is possible but requires constant monitoring updating the rules based on changes with each leg’s connection. Therefore, it is proposed to use the combination of PLMN + RAT type for constructing the steering/switching rules.

Observation 10: To simplify the process of generating and interpreting the URSP rules and avoiding coordination between PCFs, each AMF (each access) will establish a separate UE policy association and the same set of URSPs should be provided by the network to both SUPIs. The DS device should use the latest delivered URSP rules and resolve the possible inconsistencies. If there is a need for differentiation between the URSP rules based on the PLMN and Access/RAT type, relevant validity constraint can be applied by the PCF.

Observation 11: For non-simultaneous transmission, to ensure that the DL and UL transmissions are done via the same access, the UE provides PMF message to UPF, which is used by UPF for making switching decisions.

# 2. Text Proposal

Although all of the options provided in the discussion section are feasible, based on their impact level and the limitations, two alternatives are selected:

- 	A1: No AMF impact, common PDU session ID, double registration of the PDU session, and Linked SUPI provided by UE.

- 	B2: Correlation ID generated by SMF, communicated to the UE and included by the UE in the MM part of the NAS message.

Both options are included below (with corresponding colours), but only one of the options are to be selected for normative phase.

It is proposed to introduce the following changes in TR 23.700-54, where changes that apply to only one of the options are identified by using highlighted text and also mentioning the option in the parentheses.

* * * Start Changes  * * * *

# 8	Conclusions

## 8.1	Conclusions for DualSteer

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities for DualSteer.

#### 8.1.x1	 Conclusions for KI#1.1: Subscription aspects to support DualSteer

The following bullets are the conclusions agreed for KI#1.1:

- 	The AM subscription data is enhanced by including the linked SUPI (option B2)

- 	The SM subscription data is enhanced by including the linked SUPI (option A1)

- 	SMF registration information and UE context in SMF data is enhanced by including the SMF-generated Correlation ID (option B2)

#### 8.1.x.2	 Conclusions for KI#1.2: Registration and mobility management for DualSteer

The following bullets are the conclusions agreed for KI#1.2:

-	The UE provides its capability for DS and simultaneous transmission

- 	in the UE policy container of the Registration request (option A1)

- 	to AMF in the Registration request and AMF provides these capabilities to UE PCF if it supports DS (option B2)

#### 8.1.x3	 Conclusions for KI#1.3: Session management aspects for DualSteer

The following bullets are the conclusions agreed for KI#1.3:

- 	A common SMF, UPF and PCF is used for session management and session anchoring for the two linked PDU Sessions.

- 	Based on the URSP rules, the DS device requests PDU Session establishment in both 3GPP accesses if DS is to be used (if the UE is registered in both 3GPP accesses). If the DS device is registered in a single access, the DS device requests PDU Session establishment in that access, based on URSP rules.

-	A single N4 session is used to serve the two linked PDU Sessions.

- 	A single SM Policy Association session is used to serve the two linked PDU Sessions.

-	The UP connection (including N3) is active for both accesses at the same time with possibility to switch traffic flows between the two 3GPP accesses. Steering/Switching is controlled by the network via policies/rules provided to the UE via SM NAS signalling.

-	For non-simultaneous transmission, the UE provides PMF reports to UPF which indicates to the UPF when switching should occur.

-	During the establishment of the first PDU Session, the UE receives via the SM NAS signalling the SMF-Generated Correlation ID in the N1 SM container (option B2)

-	The NAS message for PDU Session Establishment Request from UE is enhanced to include

o 	The linked SUPI in the N1 SM container (option A1)

o 	The SMF-Generated Correlation ID in the MM part of the NAS message (option B2)

-	The UE provides the same PDU Session ID in both accesses (option A1)

- 	AMF selects the same SMF for PDU Session Requests from both SUPIs via

o 	current mechanisms (option A1 )

o 	obtaining the SMF ID from the UE context in SMF data using the provided linked SUPI field in the AM subscription data and the SMF-generated correlation ID provided by the UE (option B2)

-	SMCreateContext request from AMF to SMF is enhanced to include

o 	SMF-generated Correlation ID and Linked SUPI (option B2)

-	For registering the PDU session in UDM, the SMF

o 	registers the PDU session for the linked SUPI in addition to the SUPI that sent the request (option A1).

o 	includes the SMF-generated Correlation ID as a new parameter (option B2)

#### 8.1.x4	 Conclusions for KI#1.4: Policy enhancements for DualSteer

The following bullets are the conclusions agreed for KI#1.4:

- 	The same set of URSP rules are provided for both SUPIs.

- 	The URSP rule is enhanced with a new value in the Access Type preference in Route Selection Descriptor to indicate DualSteer.

- 	DualSteer steering and switching policy is provided by SM PCF as part of PCC rule. The information is provided to the UE(s) and UPF by SMF. The DualSteer steering and switching rule is provided to the UE(s) during PDU Session establishment and modification as part of SM NAS signaling.

- 	Combination of PLMN+RAT type is used by PCF for constructing the steering/switching rules.

-	The DS device should indicate its capability for simultaneous transmission in the N1 SM Container during PDU session establishment, and the SMF should provide it to the PCF during SM policy association.

- 	The steering/switching rules contain conditions for when traffic can be switched from one 3GPP access to another 3GPP access. The rules are created by the SMF based on the PCC rules. The PCF takes into account the UE capability for simultaneous transmission for creating the PCC rules. The switching conditions may contain below information:

- Switching mode: Active—Standby or Shortest Delay (only for cases with simultaneous transmission)

- Access descriptor that describes the Access 1 and Access 2 based on the PLMN (home or visited) and RAT type.

* * * End of Change * * * *