SA WG2 Meeting #164	S2-2407965

August 19 – 23, 2024, Maastricht, The Netherlands

Source:	Ericsson

Title:	KI 2.2: Evaluation and analysis

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution analyses the principles of the solutions for KI#2.2 and proposes a way forward.

# 1. 	Introduction

The main goal of KI#2.2 as agreed in the SID is to find options to simplify ATSSS over non-3GPP access, e.g. as stated in the SID:

WT#3: Study whether and how to define a functional architecture and procedures for steering, switching and splitting of traffic not based on current TNGF/N3IWF to simplify the operation over non-3GPP access without compromising the security of the 5G network.

It is also clear in the justification that the intent never was to define a solution that support all capabilities of rel-18 ATSSS, and that the solution would not assume N3IWF/TNGF, as stated in the Justification part of the SID:

At the same time, many network deployments do not have such nodes and it is therefore beneficial to study how to support a limited set of access traffic aggregation and steering features applicable to non-integrated non-3GPP access not based on TNGF/N3IWF.

This paper evaluates different solutions proposed for KI#2.2 and proposes a way forward.

In the discussion below, we divide the solutions in the TR into the following categories:

- 	Category 1: Solutions using MPQUIC between UE and UPF, without N3IWF/TNGF, without IKEv2/IPSec and without NAS via non-3GPP (Solutions 2.7 and 2.8)

- 	Category 2:  Solutions using IKEv2/IPSec between UE and UPF, without N3IWF/TNGF and without NAS via non-3GPP (Solutions 2.2 and 2.6)

- 	Category 3: Solutions based on existing N3IWF/TNGF (solutions 2.12 and 2.13)

# 2. 	Analysis of Category 1 solutions

##### 2.1 	Extension of the PCC rules and ATSSS/N4 rules

An open issue in Sol 2.7/2.8 is whether and how to extend the policy rules provided to the UE. To allow the CN to indicate to the UE and to the UPF how to handle the traffic over simplified ATSSS over non-3GPP access, the URSP rules sent by the PCF to the UE would not need to be extended, while the PCC rules as well as the ATSSS and MAR/N4 rules would need to be extended to explicitly include the access combinations for the relevant steering modes. An example of how the ATSSS rules can be extended were provided in S2-2406029.

Observation 1: ATSSS rules should be extended to indicate the use of simplified ATSSS over non-3GPP access.

##### 2.2 	MA PDU Session Establishment in 3GPP access

One concern that was expressed at SA2#163 was that sol 2.7/2.8 has a dependency that MA PDU Session shall be first established over the 3GPP access and cannot operate when 3GPP access is not available. Another concern that was raised is that the solution space for non-3GPP access is already fragmented and that we should not add a new architecture option.

It should however be noted that Sol 2.7/2.8 do not introduce a "parallel track" that is orthogonal to other non-3GPP solutions. Sol 2.7/2.8 instead builds on existing MA PDU Session functionality (MPQUIC), provides an option to streamline the existing MA PDU Session solution for certain scenarios where this is suitable, and it can also co-exist with deployments with N3IWF/TNGF. Also, as stated in the FS_MASSS SID, a justification for the study is that "many network deployments do not have such nodes and it is therefore beneficial to study how to support a limited set of access traffic aggregation and steering features applicable to non-integrated non-3GPP access not based on TNGF/N3IWF". Using Sol 2.7/2.8 can thus be a steppingstone for many operators to bring MA PDU Sessions to the market.

For scenarios where there is a need to operator non-3GPP access standalone, e.g. with single-access PDU Sessions over non-3GPP access, the existing rel-18 solutions are available and can co-exist with sol 2.7/2.8.

Observation 2: Sol 2.7/2.8 provides clear benefits for specific scenarios. For scenarios where UEs need to connect to the 5GC without an additional 3GPP leg, solutions based on N3IWF/TNGF exist already and can co-exist with Sol 2.7/2.8. Category 1 solutions are not a replacement of existing N3IWF-based solutions.

##### 2.3 	Handling of MA PDU Session when UE is no longer reachable via 3GPP Access.

ATSSS-Lite is defined with the assumption that 3GPP NAS signalling can be exchanged only via 3GPP access. This means that no 3GPP NAS signalling is transported over non-3GPP access. The assumption is thus that the 3GPP access is at least available when the MA PDU Session with simplified ATSSS is to be established. If the SMF becomes aware that the UE is not reachable over 3GPP access (e.g., based on notification from the AMF), it notifies the UPF of this event. The SMF can also start a timer to decide for how long to keep the MA PDU Session, in case the UE does no return to 3GPP access coverage. Similarly, if the UE becomes aware of the unavailability of the 3GPP access based on implementation specific mechanisms, the UE can decide whether to maintain or release the related MA PDU Session) (QUIC connection with UPF).

Observation 3: Loss of 3GPP access can be handled using suitable timers.

# 3. 	Analysis of Category 2 solutions

##### 3.1 	Support of all Steering Functionalities

Sol 2.7/2.8 is based on MPQUIC and therefore does not support ATSSS-LL.

However, ATSSS-LL is already supported by rel-18 ATSSS and Sol 2.7/2.8 do not claim to replace rel-18, it is just an optimization for certain scenarios. The SID also clearly states that the goal is to study how a limited set of access traffic aggregation and steering features can be supported in a simplified way. The argument that Sol 2.7/2.8 do not support ATSSS-LL are therefore unfounded.

It may however still be interesting to find solutions that simplifies ATSSS over non-3GPP also for ATSSS-LL. Solutions 2.2 and 2.6 achieve clear simplification (avoiding NAS via non-3GPP access) but at the same time introduce new requirements in UPF and also new interfaces. Also, the existing rel-18 solution for ATSSS with ePDG can already fulfil similar functionality as Solutions 2.2 and 2.6.  These solutions therefore do not introduce any substantial simplifications compared to a pre-rel-19 system.

It can however be possible to collocate the ePDG with UPF as described in Sol 2.2 as a deployment option.

Observation 4: Sol 2.2, collocating ePDG with UPF, can be supported as a deployment option with minor enhancements to the standard (delivering ePDG IP address via SM NAS to the UE).

##### 3.2 	Variants for IPSec tunnel establishment

Solutions 2.6 proposes a variant where IPSec is used between UE and UPF but is not relying on ePDG procedures. Instead, new authentication signalling transport is introduced between UPF-SMF and SMF-AUSF. This adds new requirements on UPF, N4, SMF and AUSF, and introduces a new interface between SMF and AUSF. This option thus has significant systems impact and can hardly qualify as a “simplification”. At the same time, SA3 has started to study other solutions for how to use IPSec between UE and UPF based on deriving a new key during primary authentication in 3GPP access and providing this key to UPF for use in authenticating IKEv2/IPSec. Such solutions are based on existing procedures and interfaces and would thus be much simpler than Sol. 2.6. Such solutions could be accommodated by SA2 assuming that they are agreed in SA3.

Observation 5: In case there is a desire to support IPSec between UE and UPF for ATSSS-LL traffic but without relying on ePDG (e.g. to avoid 4G interfaces), solutions can be discussed together with SA3 on how to do that with small impacts to the system. IPSec should however not be required for MPQUIC traffic.

# 4. 	Analysis of Category 3 solutions

##### 4.1 	Level of simplification

Based on the comparison in Annex A, Sol 2.12/2.13 provides none or very minor simplification compared to existing rel-18. Sol. 2.13 is also technically unclear since the routing between UE and N3IWF/TNGF is not described. Sol 2.12 makes it possible to avoid double encryption in case MPQUIC is used, but the IPSec packet encapsulation overhead is still maintained, and IPSec integrity protection cannot be turned off. It can be questioned whether avoiding encryption but still requiring integrity protection brings any real processing improvements in the UE and the network.

Observation 6: The option to support NULL encryption between UE and N3IWF does not provide any real improvements since the packet encapsulation overhead is still maintained, and IPSec integrity protection cannot be turned off.

# 5. 	Security related aspects

There have been arguments in SA2 about security properties of different solutions.

For category 1 solutions, it has been commented that there may be security issues with exposing a MPQUIC proxy in UPF via the Internet.

For category 2 solutions, the security aspects may relate to having a direct UE-UPF connection via non-3GPP access without a separate security entity.

For both categories, it should however then be noted that the UPF is already exposed via the Internet (via N6). Also, operators typically deploy Firewalls (either separately or integrated in UPF) but those Firewall aspects are out of 3GPP scope.

For category 3 solutions, the concerns may relate to using NULL encryption in IPSec or remove the IPSec SA altogether.

It can thus be noted that the security aspects of all KI#2.2 solutions need to be studied, but this is within SA3 remit, and a corresponding study already work-in-progress in SA3.

Observation 7: No concrete technical arguments regarding security of any solution in TR 23.700-54 have been brought up in SA2. This is a topic that is in SA3 scope (applies equally to all solutions for KI#2.2).

# 6. Summary

A summary of the evaluation is provided in Annex A

# 7. 	Proposal

Conclude KI#2.2 by

- 	Supporting the possibility to use MPQUIC directly between UE and UPF by establishing the MA PDU Session via 3GPP access (Sol 2.7/2.8).

- 	Enabling implementation options with ePDG collocated with UPF, by providing ePDG IP address to the UE in SM NAS signalling at MA PDU Session establishment (Sol 2.2).

- 	In addition, the use of IPSec directly between UE and UPF by establishing the MA PDU Session via 3GPP access can also be supported pending SA3 discussions (i.e. SA3 could define optimizations that allows a more efficient IPSec setup between UE and UPF).

Text proposal for updating TR 23.700-54 is available in S2-2407966.

# A. 	Annex A

The solutions proposed for KI#2.2 are summarized in Table A.1 (AMF impacts to support capability negotiation and SMF selection has not been included for simplicity since it is similar in all solutions).

Table A.1: Comparison of solutions for KI#2.2

|  | Category 1: Solutions using MPQUIC between UE and UPF, without N3IWF/TNGF, without IKEv2/IPSec and without NAS via non-3GPP |  | Category 2: Solutions using IKEv2/IPSec between UE and UPF, without N3IWF/TNGF and without NAS via non-3GPP |  | Category 3: Solutions based on existing N3IWF/TNGF |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.7 | 2.8 | 2.2 | 2.6 | 2.12 | 2.13 |
| Simplification | Eliminates NAS and N2 via non-3GPP. Eliminates IKEv2 / IPSec via non-3GPP | Eliminates NAS and N2 via non-3GPP. Eliminates IKEv2 / IPSec via non-3GPP | Eliminates NAS and N2 via non-3GPP. Allows IPSec SA to use NULL encryption | Eliminates NAS and N2 via non-3GPP. | Allows IPSec SA to use NULL encryption | Allows to send traffic without IPSec SA |
| 5GC impacts | SMF, UPF. | SMF, UPF. | SMF, UPF, (ePDG). New interfaces (re-using EPC interfaces for ePDG) | SMF, UPF, AUSF. New interface SMF-AUSF. PFCP impacts to carry EAP payload | N3IWF, SMF, UPF | N3IWF/TNGF, SMF, UPF |
| Supported Steering Functionalities | MPTCP, MPQUIC | MPQUIC | ATSSS-LL, MPTCP, MPQUIC | ATSSS-LL, MPTCP, MPQUIC | ATSSS-LL, MPTCP, MPQUIC | Not stated |
| Double encryption over non-3GPP | No | No | Yes for MPQUIC. No for ATSSS-LL, MPTCP. | Yes for MPQUIC. No for ATSSS-LL, MPTCP. | No | Not clear |
| Considerations | Significant simplification: avoids IKEv2/IPSec and NAS via non.3GPP. | Significant simplification: avoids IKEv2/IPSec and NAS via non.3GPP. | Does not simplify the protocol stack since the existing ePDG solution is re-used. Removes crypto overhead, but packet encapsulation is maintained, and IPSec integrity protection is required. Also maintains 4G/EPC protocols and entities. | Partial simplification: IKEv2/IPSec maintained, but avoids NAS via non-3GPP Unclear how the UPF performs traffic aggregation/splitting: if based on MPQUIC/TCP the benefit is unclear. Significant system impact (new interface and impacts to existing interfaces). | Insignificant simplification: Removes crypto overhead, but IPSec packet encapsulation is maintained, and IPSec integrity protection is required. NAS needs to be used via non-3GPP access. | In addition to the considerations for sol. 2.12, the solution is unclear since the traffic forward/routing between UE and N3IWF/TNGF is not described. It is therefore not easy to evaluate. |