SA WG2 Meeting #164	S2-2407967

August 19 – 23, 2024, Maastricht, The Netherlands

Source:	Ericsson

Title:	Access restrictions for satellite access

Document for:	Discussion

Agenda Item:	8.27

Work Item / Release:	IoT_SAT_ARCH_EPS / Rel-18

Abstract of the contribution: This contribution discusses access restrictions for satellite access in relation to IWK between EPC and 5GC.

# History

Work has recently been done in CT4, SA2 and RAN3 to enhance the possibilities to handle subscription-based access restrictions for satellite access in relation to IWK between EPC and 5GC. This has included:

In Dec 2023 (CT4#119, Chicago), CT4:

- 	added support for NR satellite access types to the GTPv2-C Access Restriction Data (ARD), to be able to prevent handovers from LTE to NR satellite access when the subscriber does not have a subscription that permits NR NTN access [1]. The following values were added to the ARD in TS 29.272:

NR (LEO) Not Allowed

NR (MEO) Not Allowed

NR (GEO) Not Allowed

NR (OTHERSAT) Not Allowed

- 	added LTE satellite access types to the 5G RAT Type IE, to be able to prevent handovers from NR to LTE satellite access when the subscriber does not have a subscription that permits LTE-NTN access [2]. The follwing values were added to the RAT Type IE in TS 29.571:

"WB-E-UTRAN(LEO)"

"WB-E-UTRAN(MEO)"

"WB-E-UTRAN(GEO)"

"WB-E-UTRAN(OTHERSAT)"

"NB-IoT(LEO)"

"NB-IoT(MEO)"

"NB-IoT(GEO)"

"NB-IoT(OTHERSAT)"

"LTE-M(LEO)"

"LTE-M(MEO)"

"LTE-M(GEO)"

"LTE-M(OTHERSAT)"

In January 2024, (SA2#160 e-meeting), SA2 made the corresponding updates to SA2 5G specifications:

- 	added LTE satellite access types to the subscribed access restriction parameter in UDM [3]. The following values were added to the access restrictions in 23.501:

-	WB-E-UTRAN (LEO) satellite access not allowed as primary access.

-	WB-E-UTRAN (MEO) satellite access not allowed as primary access.

-	WB-E-UTRAN (GEO) satellite access not allowed as primary access.

-	WB-E-UTRAN (OTHERSAT) satellite access not allowed as primary access.

-	NB-IoT (LEO) satellite access not allowed as primary access.

-	NB-IoT (MEO) satellite access not allowed as primary access.

-	NB-IoT (GEO) satellite access not allowed as primary access.

-	NB-IoT (OTHERSAT) satellite access not allowed as primary access.

-	LTE-M (LEO) satellite access not allowed as primary access.

-	LTE-M (MEO) satellite access not allowed as primary access.

-	LTE-M (GEO) satellite access not allowed as primary access.

-	LTE-M (OTHERSAT) satellite access not allowed as primary access.

There was no need to enhance SA2 EPC specifications, since they do not explicitly list the RAT types that can be restricted in EPC.

SA2 also sent an LS to RAN3 informing them about the CRs and asking RAN3 to check if there is a need to update RAN3 specifications [4].

In February (RAN3#123), RAN3 updated their specifications:

- 	S1AP was enhanced with NR satellite access types [5]. The following values were added to the RAT Restriction parameter (in Handover Restriction List): NR-LEO (4), NR-MEO (5), NR-GEO (6), NR-OTHERSAT (7).

- 	NGAP was enhanced with LTE satellite access types [6]. The following values were added to the RAT Restriction parameter (in Mobility Restriction List): e-UTRA-LEO (7), e-UTRA-MEO (8), e-UTRA-GEO (9), e-UTRA-OTHERSAT (10)

RAN3 also sent an LS reply to SA2 [7] where they informed about the RAN3 CRs and that they have added codepoints for e-UTRA-LEO/MEO/GEO/OTHERSAT to NGAP and XnAP, and codepoints for NR-LEO/MEO/GEO/OTHERSAT to S1AP and X2AP.

Observation 1: RAN3 only added a codepoints for e-UTRA (LEO/MEO/GEO/OTHERSAT) on NGAP. RAN3 did not add the codepoints corresponding to NB-IoT (LEO/MEO/GEO/OTHERSAT) or LTE-M (LEO/MEO/GEO/OTHERSAT). There is thus a difference between what subscription-based restrictions can be defined in UDM and provided to AMF and what can be sent from AMF to NG-RAN in the Mobility Restriction List.

At SA2#162 (Changsha) it was proposed to either introduce a mapping “algorithm” in AMF between the set of values that can be received from UDM and the set of values that can be provided to NG-RAN [8] or send an LS reply to RAN3 to ask them to re-consider and add the codepoints for NB-IoT and LTE-M satellite access to NGAP [9]. The discussion, including the RAN3 LS, was however postponed.

At SA2#163 (Jeju), there was no discussion and the LS reply from RAN3 was again postponed.

# Discussion

It may be useful to compare what type of access restrictions and mobility restrictions can be supported in terrestrial networks, and how they are handled in relation to what can be provided to RAN from AMF/MME.

The following (terrestrial/general) access restrictions are listed in 23.501:

-	For NR:

-	NR not allowed as primary access.

-	NR not allowed as secondary access.

-	NR in unlicensed bands not allowed as primary access.

-	NR in unlicensed bands not allowed as secondary access.

-	NR RedCap not allowed as primary access.

-	For E-UTRA:

-	E-UTRA not allowed as primary access.

-	E-UTRA not allowed as secondary access.

-	E-UTRA in unlicensed bands not allowed as secondary access.

-	NB-IoT not allowed as primary access.

-	LTE-M not allowed as primary access.

Out of these, only the following values can be provided to NG-RAN: e-UTRA (0), nR (1), nR-unlicensed (2).

As can be noted, it is not possible to provide the values “NB-IoT”, “LTE-M” or “RedCap” in the NGAP Mobility Restriction List (MRL) to NG-RAN. It is also not possible to provide the values “NB-IoT” or “LTE-M” in the S1AP Handover Restriction List to E-UTRAN. This is similar to the difference that was mentioned in Observation 1.

Observation 2: The access restrictions for NB-IoT and LTE-M cannot be provided to NG-RAN or to E-UTRAN. This applies to both terrestrial and satellite access.

Access restrictions for NB-IoT and LTE-M are instead handled in the core network, e.g. due to the following reasons:

- 	LTE-M is not a proper RAT type or a property of the radio cell. LTE-M is a UE category. AMF creates the RAT Type “LTE-M” based on the UE category indication from NG-RAN (similar in MME). The “RAT Type” LTE-M is thus only used in the Core Network as a convenient and combined way to indicate that a CAT-M UE is using E-UTRAN access. Since “LTE-M” is not a RAT type per se, providing LTE-M as a “RAT restriction” to NG-RAN would not make much sense.

Furthermore, RAN handover handling is today related to properties of RATs/cells, not UE capabilities. Measurements etc are reported based on available RATs. It would be a new concept to enforce mobility restrictions in RAN related to potential UE capabilities in a target RAT/cell.

- 	NB-IoT is different from LTE-M in the sense that NB-IoT is a proper RAT type, i.e. a property of the radio cell. However, NB-IoT does not support handovers and there is thus no need to provide “NB-IoT” in a mobility/handover restriction list. Also, as stated in TS 36.300, a NB-IoT UE is not assumed to support IWK with NR:

"Interworking with NR is not supported by BL UE (e.g. functions like NR measurement reporting, reselection to NR, handover to NR, redirection to NR are not supported)."

Possibly there could have been a reason to prevent release with redirect (RwR) from NR to NB-IoT, but that does not seem possible for terrestrial access, and it can be questioned why it should then be an issue for satellite access. It may also not help much since it will anyway not be possible to prevent a UE losing coverage in e.g. NR access from trying to connect in NB-IoT access.

Observation 3: Access restrictions for NB-IoT and LTE-M are handled in the core network.

Based on observation 3, it may be argued that the “mismatch” described in observation 1 and 2 are not an issue – it is just the way it already works in terrestrial access. However, TS 23.501 may still need to be updated since applying access restriction for NB-IoT and LTE-M satellite access in AMF is currently not possible (NB-IoT and LTE-M satellite accesses to connect to 5GC is not supported in existing releases). Therefore, providing such access restriction values from UDM to AMF is not useful.

Observation 4: Providing access restrictions for NB-IoT and LTE-M satellite accesses from UDM to AMF is not useful since AMF cannot enforce them.

# Proposal:

Based on the above analysis, it is proposed to remove the access restrictions for NB-IoT and LTE-M satellite accesses from TS 23.501 and note the LS from RAN3. A corresponding CR is provided in S2-2407968.

# References

[1] 	https://www.3gpp.org/ftp/tsg_ct/WG4_protocollars_ex-CN4/TSGCT4_119_Chicago/Docs/C4-235568.zip

[2] 	https://www.3gpp.org/ftp/tsg_ct/WG4_protocollars_ex-CN4/TSGCT4_119_Chicago/Docs/C4-235570.zip

[3] 	https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_160AHE_Electronic_2024-01/Docs/S2-2401630.zip

[4] 	https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_160AHE_Electronic_2024-01/Docs/S2-2401650.zip

[5] 	https://www.3gpp.org/ftp/tsg_ran/WG3_Iu/TSGR3_123/Docs/R3-241185.zip

[6] 	https://www.3gpp.org/ftp/tsg_ran/WG3_Iu/TSGR3_123/Docs/R3-241058.zip

[7] 	https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_163_Jeju_2024-05/Docs/S2-2405846.zip

[8] 	https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_162_Changsha_2024-04/Docs/S2-2404441.zip

[9] 	https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_162_Changsha_2024-04/Docs/S2-2404443.zip