3GPP TSG-WG SA2 Meeting #164	S2-2407998

Maastricht, Netherlands, August 19 - 23, 2024	(revision of S2-2407196)

Source:	Google

Title:	KI#2: Evaluation and Conclusion for Encrypted XRM Traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract: This pCR provides evaluation and proposes to conclude the KI#2.

# 1	Discussion

In that last meeting, we propose to support all three options in KI#2 conclusion because these options introduce requirements to the application implementation which should not be restricted by the 3GPP. Our position is the same for concluding this Key Issue#2. It is recommend to support these options:

- Option1, MoQ: solution#9, #10

- Option2, UDP-Option based RoQ: solution#11, #12, #27

- Option3, UDP tunel (Proxy-UDP-in-HTTP/3 + QUIC-Aware proxying): solution#24, #26

Further, we provide our technical views on each options.

All three options propose to deliver in-band metadata containing PDU Set information for XRM traffic over N6 by using different protocols. The impacts on the AF, PCF, SMF are similar regarding provisioning related information, e.g. protocol description, configuration, and QoS requirements, from the AF to the UPF. The differences in these options are highlighted in the followings:

For configuration information and UPF impacts consideration:

- Option1 and Option 3: require AS address information for media transport over N6 tunnel.

  - Option1: UPF/MoQ relay needs to maintain two MoQ transport connections (between UE and the UPF, between UPF and AS), which implies double encryption and decrption. The exposure of UPF address information to the UE is needed. The interaction among MoQ client, MoQ relay, and AS is achieved by a MoQ subscription procedure for Track Namespace of UE interest.

  - Option3: UPF needs to maintain UDP tunel over N6 using UDP-Connect upgrade token in forwarded mode.

- Option2: require QUIC session correlation ID, and security parameters for encryption/integrity protection on metadata.

For UE impacts consideration:

- Option1: require UE to initiate/maintain MoQ transport connection with UPF.

- Option 3 (connect-UDP in forwarded mode): require UE/http client to initiate/maintain UDP tunnel with AS/http server.

- Option2 and Option 3 (connect-UDP in tunnel mode): there is no UE impacts on media transport over N6.

For IETF impacts consideration:

- Option1: Repurpose MoQ Object message for metadata with specific information mapping between Track and PDU Set Information. This option is not future proof. If the PDU Set information needs to be extended in 3GPP, IETF needs to work on the MoQ metadata for XRM traffic that is 3GPP specific (not usual IETF biz). It is still not yet clear whether MoQ Object format is in the scope of IANA registration.

- Option2: reference [21] IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP" provides the UDP-Option framework. Currently, Kind value in the range 194-253 are reserved and assignable by IANA for PDU Set information.

- Option3: reference [40] IETF draft-ietf-masque-quic-proxy-01: "QUIC-Aware Proxying Using HTTP". (published in 8 July: Forwarded mode) is added. The IANA registration of the new Connection ID for the proposed http datagram format is needed. The encryption/integrity protection of the XRM metadata may be negotiated over via AF request or via UDP tunnel between UPF and AS. The latter may need a Connection ID which requires IANA registration.

Other Considerations:

- Option 1 and Option 3: the N6 media transport tunnel would need to handle the following disconnection occasions:

  - UPF changes. Option 1 would additionally impacts on the UE if any MoQ transport connection is changed.

  - AS changes. The load balancing mechanism can result in disconnection of the N6 media transport tunnel.

Summary:

Based on above consideration on UE/UPF/IETF impacts, the followings are our general observations:

- Option 1 (MoQ) comes with significat impacts on UPF, UE, and unclear IETF impacts as well as potentially more disconnection occassions.

- Option 1 (MoQ) and Option 3 (UDP tunnel over N6) have potential media transport disconnection issues due to AS or UPF relocation.

- Option 2 (UDP option based RoQ) and Option 3 (UDP tunnel over N6) do not have UE impacts.

- Option 2 (UDP option based RoQ) is free of media transport disconnection issue and no UE impacts.

All in all, these options introduce requirements to the application implementation which should not be restricted by the 3GPP. Our recommendation is to conclude this Key Issue#2 by enabling the support for all options in normative work.

# 2	Proposal

It is proposed to include the below changes into TR 23.700-70.

First Changes

### 8. Conclusions

## 8.X	Conclusions for Key Issue #2

The following principles are recommended to be included in the conclusions:

The following options should be supported in normative work.

-	For MoQ, the PDU set information is accommodated via Metadata in MoQ. UPF supports the MoQ relay functionality and identifies the PDU Set information from the metadata in MoQ header. The detailed information include:

-	AF provides the Protocol Description with information related to MoQ based XRM traffic for PDU Set Information in order to support identification of PDU set for encrypted traffic by following the AF session with QoS procedure. The request may include AS address.

-	PCF determines PCC rules based on AF request, to instruct SMF/UPF to identify PDU Set information for MoQ traffic.

-	SMF provides N4 rule to UPF to activate the MoQ relay functionality.

-	UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata. For DL, UPF includes the identified PDU Set information into GTP-U header as specified in Rel-18.

-	UE: operating as MoQ client needs to establish MoQ transport connection with MoQ relay/UPF based on UPF address information exposed by the UPF.

-	For UDP-Option based RoQ, the PDU set information is accommodated via Metadata in UDP-Option. UPF supports the UDP-Option based RoQ functionality and identifies the PDU Set information from the metadata in UDP-Option. The detailed information include:

-	AF provides the Protocol Description with information related to UDP-Option based RoQ traffic for PDU Set Information in order to support identification of PDU Sets for encrypted traffic by following the AF session with QoS procedure. The request includes QUIC session correlation ID (random number) and security keys for metadata encryption/integrity protection.

-	PCF determines PCC rules based on AF request, to instruct SMF/UPF to identify PDU Set information for UDP option based RoQ traffic.

-	SMF provides N4 rule to UPF to activate the UDP-Option based RoQ functionality.

-	UPF supports the UDP-Option based RoQ functionality and identifies the PDU Set information from the metadata in UDP-Option. For DL, UPF includes the identified PDU Set information into GTP-U header as specified in Rel-18.

-	UE: no UE impacts on media transport over N6.

-	For Proxy-UDP-in-HTTP/3 + QUIC-Aware proxying, PDU set information is included in HTTP datagram via a new datagram mode or via packet transforms in QUIC-Aware proxying. UPF supports the HTTP/3 Client functionality. The detailed information include:

-	AF provides the Protocol Description with information related to HTTP datagrams for PDU Set Information in order to support identification of PDU Sets for encrypted traffic, and the AS address to enable encapsulation protocol connection between the UPF and AS following the AF session with QoS procedure, e.g. establishment of a UDP tunnel with the connect-UDP upgrade token in forward mode.

-	PCF generates a PCC rule including the indication of UDP tunnel establishment and AS address.

-  SMF provides N4 rules to the UPF including the indication of UDP tunnel establishment and AS address.

-	UPF supports the HTTP/3 Client functionality, which establishes a UDP tunnel to the AS (where HTTP/3 Proxy operates) with the connect-UDP upgrade token in forwarded mode. For DL, UPF identifies the PDU Set Information from the HTTP datagrams, and includes the PDU Set Information into GTP-U header as in Rel-18. The UPF shall maintain UDP tunel over N6 for media transport for UE(s).

-	UE:

  - connect-UDP in forwarded mode: require UE/http client to initiate/maintain UDP tunnel with AS/http server.

  - connect-UDP in tunnel mode: there is no UE impacts on media transport over N6.

End of Changes