3GPP SA WG2 Meeting #164	 S2-2408050

# Maastricht, NL, 19-23 August 2024											                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1420 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: Support of interworking feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of features to support " IMS data channel interworking with MTSI UE " based on the conclusions for KI#3 in TR 23.700-77 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution add a new chapter to TS 23.228 based on the conclusions for KI#3 in TR 23.700-77. Add a procedure to depict how to perform the interworking based on DC AS when the DCMTSI UE request to provide DC application specific information to MTSI UE during the call |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed " IMS data channel interworking with MTSI UE " is not in the TS 23.228. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.X(new), AC.7.X.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes* * *

AC.7.X	IMS data channel interworking with MTSI UE Procedure

AC.7.X.Y	interworking with MTSI UE based on DC AS

Figure AC.7.X.Y-1 depicts a signalling flow diagram for establishing an application data channel from DCMTSI UE to MTSI UE use case .

In the call flow the UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

Figure AC.7.2.2-1: Person-to-Application (P2A) Application Data Channel set up Signalling Procedure

Precondition:

UE#1 is a DCMTSI UE, UE#2 is a MTSI UE.

UE#1 wants to provide DC application specific information to UE#2 during the call, such as transfer a file (e.g. a photo) to UE#2.

The steps in the call flow are as follows:

1. DC AS subscribes to IMS AS the event that application DC for an DC application that requires interworking is established.

2. IMS session and bootstrap data channel have been established. Selected DC application(s) have been downloaded to UE#1.

The IMS AS and DCSF can know the UE#2 is a MTSI UE based on the port of bootstrap data channel between the DCSF and UE #2 is set to zero in the SDP answer.

NOTE 1:	The UE#2 can be seen as an MTSI UE if UE#2 does not support DC or the network of UE#2 does not support DC.

3-5. Steps 2-3 of clause AC.7.2.1 applies.

The UE#1 may initiate a P2P application DC if the UE#1 does not know the UE#2 is a MTSI UE.

6. After receiving the session event notification, the DCSF determines that IMS DC interworking for the requested DC application is required and perform the P2A application DC establishment procedure.

If the UE#1 initiates a P2P application DC request in step 3, the DCSF may change to P2A application DC.

NOTE 2:	How the DCSF can determine that IMS DC interworking for the requested DC application is required is up to implementation. DCSF can for example determine the interworking is required based on the UE#2 is a MTSI UE, and DC application corresponding to the binding information supports interworking.

7-16. Steps 5-14 of clause AC.7.2.2 applies.

The DCSF may inform the IMS AS the P2A application DC procedure is performed.

17. IMS AS sends a notification to indicate the DC AS that the interworking should be perform for UE#2.

The IMS AS may sends the notification based on the UE#2 is a MTSI UE, the DC application corresponding to the binding information supports interworking, and a P2A application DC is established for the DC application.

18-19. IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, which does not include bootstrap data channel information and application data channel information in the SDP for the application data channel.

20-29. Steps 17-26 of clause AC.7.2.2 applies.

The difference is that in step 25, the IMS AS may inform the UE#1 that the P2A application data channel is established as defined in TS 26.114 [76].

30.	UE#1 can use the established application data channel to provide DC application specific information to the DC AS, such as, sends the file (e.g. a photo) which will be sent to UE#2 to DC AS.

31.	DC AS performs interworking actions based on the notification received in step 17 and the data received in step 30.

NOTE 3:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS may for example generate a hyperlink associated with the file received in step 30 and sends the hyperlink to UE#2 through a SMS, the user of UE#2 can download the file using the hyperlink if he/she wants.

* * * End of Changes * * *