SA WG2 Meeting #164	S2-2408058

Maastricht, Netherlands, 19 August – 23 August, 2024

Title:	LS reply on Subscription of UPF event via I-SMF

Response to:	S2-2407446

Release:	Rel-18

Work Item:	UPEAS

Source:	SA2

To:	CT4

Cc:	SA, CT3

Contact Person:

Name:	Dan Wang

Tel. Number:

E-mail Address:	wangdanyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2408056, S2-2408057

1. Overall Description:

SA2 thanks CT4 for the questions. SA2 gives the answers below of each question:

- whether they considered both approaches above, and if so, on which criteria should the anchor SMF invoke the Nsmf_PDUSession API vs. the Nsmf_EventExposure to subscribe to UPF events via the I-SMF?

Answers by SA2: When the SMF receive the subscription request from consumers and the event filter includes an DNAI and the I-SMF serving the DNAI is inserted for PDU Session, the SMF generates Nsmf_EventExposure_Subscribe request including UPF event consumer address, Notification Correlation Information and Event Filter Information to I-SMF.

- whether the anchor SMF should be able to determine if the requested UPF events/service are supported by the I-SMF/I-UPF, before triggering a subscription towards the I-SMF, and if so, how.

Answers by SA2: Yes, according to DNAI in the event filter from UPF event subscription consumer, if the DNAI can not be served by the anchor SMF, the SMF determines that the requested UPF events/service are supported by I-SMF/I-UPF.

- whether the anchor SMF may need to subscribe to UPF events at the ULCL/BP and/or at the local PSA UPF;

Answers by SA2: It depends on the event that NF consumers subscribed. If needed, the SMF may need to subscribe to UPF events at the ULCL/BP and/or at the local PSA UPF.

- how to support subscriptions at the anchor SMF targeting ANY UE, e.g. ANY UE in a specific area, e.g. whether the anchor SMF should determine by itself the UEs present in the specific area and trigger then subscriptions, per PDU session, for the corresponding UEs?

Answers by SA2: For any UE subscription, the anchor SMF should determine by itself the UEs present in the specific area, and several SMFs are discovered. If DNAI is included in the subscription request and the subscribed UPF events needs the SMF(s) to do a third-party subscription onto UPF, and if the several SMFs determines the target DNAI is not served, the several SMFs sends the Nsmf_EventExposure_Subscribe including the UPF event consumer address, Notification Correlation Information, Event Filter Information, Reporting suggestion information, Target of Event Reporting and Target Subscription Information to several I-SMFs respectively on behalf of the NF consumer. And the I-SMFs send the subscription request to several UPFs respectively on behalf of SMFs.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 group to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP SA2#165	14-18 October 2024		Hyderabad, India

3GPP SA2#166	18-22 November 2024		Orlando, America