3GPP TSG-SA WG2 Meeting #164	S2-2409109

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	CATT

Title:	Update the Conclusion for ATSSS_Ph4 KI#1.2

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes to update the conclusion for ATSSS_Ph4 KI#1.2.

1.	Introduction

This contribution proposes to update the conclusion for ATSSS_Ph4 KI#1.2.

In TS23.501, the Filters used in ATSSS rules related with a MA PDU Session of type Ethernet can refer to IP level parameters such as IP addresses and TCP/UDP ports as excerpt below and the text is highlighted.

----------------------------------------Start of Excerpt----------------------------------------

5.32.6	Support of Steering Functionalities

5.32.6.1	General

The functionality in an ATSSS-capable UE that can steer, switch and split the MA PDU Session traffic across 3GPP access and non-3GPP access, is called a "steering functionality". An ATSSS-capable UE may support one or more of the following types of steering functionalities:

-	High-layer steering functionalities, which operate above the IP layer:

-	In this release of the specification, two high-layer steering functionalities are specified:

-	The first applies the MPTCP protocol (IETF RFC 8684 [81]) and is called "MPTCP functionality" (see clause 5.32.6.2.1). This steering functionality can be applied to steer, switch and split the TCP traffic flows identified in the ATSSS/N4 rules. The MPTCP functionality in the UE may communicate with an associated MPTCP Proxy functionality in the UPF, by using the MPTCP protocol over the 3GPP and/or the non-3GPP user plane.

-	The second applies the QUIC protocol (RFC 9000 [166], RFC 9001 [167], RFC 9002 [168]) and its multipath extensions (draft-ietf-quic-multipath [174]), and it is called "MPQUIC functionality" (see clause 5.32.6.2.2). This steering functionality can be applied to steer, switch and split the UDP traffic flows identified in the ATSSS/N4 rules. The MPQUIC functionality in the UE may communicate with an associated MPQUIC Proxy functionality in the UPF, by using the QUIC protocol and its multipath extensions over the 3GPP and/or the non-3GPP user plane.

-	Low-layer steering functionalities, which operate below the IP layer:

-	One type of low-layer steering functionality defined in the present document is called "ATSSS Low-Layer functionality", or ATSSS-LL functionality (see clause 5.32.6.3.1). This steering functionality can be applied to steer, switch and split all types of traffic, including TCP traffic, UDP traffic, Ethernet traffic, etc. ATSSS-LL functionality is mandatory for MA PDU Session of type Ethernet. In the network, there shall be in the data path of the MA PDU session one UPF supporting ATSSS-LL.

NOTE:	Filters used in ATSSS rules related with a MA PDU Session of type Ethernet can refer to IP level parameters such as IP addresses and TCP/UDP ports.

----------------------------------------End of Excerpt----------------------------------------

Based on the above highlighted texts, if the ATSSS rules for the MPQUIC connect-ip are defined, the ATSSS rules for the MPQUIC connect-ip can also be used for the Ethernet MA PDU Session type. If both MPQUIC-IP and MPQUIC-E are used with the same Ethernet MA PDU Session type, the MPQUIC-IP can be used with the IP payload over the Ethernet, and MPQUIC-E can be used with the non-IP payload over the Ethernet.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First Change * * * *

## 8.2	Conclusions for ATSSS_Ph4

### 8.2.1	Conclusion for Key Issue #2.1

a)	New MPQUIC steering functionalities are defined:

-	Multipath QUIC-IP (MPQUIC-IP) steering functionality based on CONNECT-IP [7] to tunnel IP packets between a client (UE) and a proxy (UPF) using HTTP/3 over MPQUIC;

-	Multipath QUIC-Ethernet (MPQUIC-E) steering functionality based on CONNECT-Ethernet [8] to proxy Ethernet frames between a client (UE) and a proxy (UPF) using the HTTP/3 over MPQUIC.

NOTE 1:	How to address the issue with Ethernet header compression in RAN, without any RAN impact, when MPQUIC-E is used is to be discussed during normative phase.

NOTE 2:	Whether to support CONNECT-Ethernet in Rel-19 is dependent on progress in IETF. It is assumed that IETF last call for the CONNECT-Ethernet draft is needed at least during 1H 2025 to allow the draft to be stable enough for Rel-19.

NOTE 3:	The MPQUIC steering functionality defined in Rel-18 TS 23.501 [3] refers to a Multipath QUIC-UDP (MPQUIC-UDP) steering functionality based on CONNECT-UDP [6].

NOTE 4:	The MPQUIC steering functionalities can be selected if additional features, which are not supported by the ATSSS-LL steering functionality and PMF, are required for the traffic steering/switching/splitting of a service data flow.

b)	During MA PDU Session Establishment with an IP-based PDU Session type, the UE indicates to SMF what MPQUIC steering functionalities it supports. In a single MA PDU Session, different proxy protocols may be used for different service data flows. If the UE supports multiple MPQUIC steering functionalities, it shall use the provisioned ATSSS rules to decide which steering functionality to apply for a service data flow.

c)	Additional parameters associated with a connect protocol, e.g. "ipproto" and "target" parameters associated with CONNECT-IP [7] is derived by the UE based on the detected application.

d)	MPQUIC-IP steering functionality may be used with Ethernet MA PDU Session type.

NOTE 5:	For MPQUIC-IP with Ethernet MA PDU Session type, the IP payload over the Ethernet is used.

e)	MPQUIC-E steering functionality shall be used with Ethernet MA PDU session type.

NOTE 6:	If both MPQUIC-IP and MPQUIC-E are used with the same Ethernet MA PDU Session type, the MPQUIC-IP can be used with the IP payload over the Ethernet, and MPQUIC-E can be used with the Ethernet with non-IP payload.

# * * * * End of change* * * *