3GPP TSG-SA WG2#164	S2-2408131

19-23 August 2024, Maastricht, Netherlands	(was S2-2406824)

Source:	Apple

Title:	Conclusion for KI#1.2

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.2.

# 1.	Introduction

This contribution proposes conclusion for KI#1.2 based on NWM discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.2

The following principles are concluded for the normative work:

-	The serving PLMN needs to support DualSteer feature, the impacts are on SMF/PCF selection and UE capabilities handling.

-	The UE includes in the Registration request whether it is DualSteer capable.

-	A DualSteer Device that supports simultaneous transmission performs two separate registration procedures in a sequential manner for each of the SUPIs of the DualSteer Device.

- 	If the DualSteer Device sends DualSteer capabilities as part of Registration Request for the first SUPI, the serving network sends that information to the UDM. The UDM performs DualSteer authorization based on that and sends the authorisation result to the AMF. The AMF indicates to the DualSteer Device whether the network authorizes the DualSteer feature during registration.

-	The HPLMN provides a Registration Correlation Information to the DualSteer Device during the first SUPI’s registration. The Registration Correlation Information is allocated by UDM. Details are to be specified in the normative phase.

-	When the DualSteer Device performs the registration for the second SUPI, it provides the Registration Correlation Information to the HPLMN so that HPLMN can determine whether the two SUPIs are used by the same DualSteer Device based on the received Registration Correlation Information.

-	When the registration for the first SUPI is accepted, the AMF indicates to the DualSteer Device whether DualSteer is authorized and may provide a registration-related policy when to register the second SUPI.

-	Based on the registration-related policy, the DualSteer Device can trigger the second registration for the second SUPI.

Editor’s Note: Whether and how to impact the second SUPI’s PLMN selection / RAT or Access Type selection is pending on SA1's response to SA2 LS.

# >>>>END OF CHANGES<<<<