3GPP TSG-WG SA2 Meeting #164	S2-2408245

Maastricht, Aug 19 – 23, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	A new solution: AIoT data forwarding over UE reader in topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: a new solution to support information transfer from the Ambient IoT devices to the network

# 1. Introduction/Discussion

This contribution is prepared to propose a new solution for Ambient IoT.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v0.4.0.

FIRST CHANGE (All text is new)

## 6.x	Solution #x: AIoT data forwarding over UE reader in topology 2

### 6.x.1	Description

This solution address aspects of key issue #3 on Support of Ambient IoT Services.

This solution can be applied for only Topology 2.

There are multiple methods that a UE reader forward data received from the AIoT devices to the network.

- Over user plane

- Over control plane where the UE reader in RRC CONNECT state (UL NAS)

- Over control plane where the UE in RRC IDLE state (SDT with CIoT optimisation)

Each mechanism has its own advantages and disadvantages depending on specific factors as described below:

- Latency Requirements:

  - User Plane: Suitable for applications requiring low latency and high data rates, as it allows for direct data transfer.

  - UL NAS: Typically has higher latency compared to the User Plane but might be adequate for less time-sensitive data.

  - CIoT optimisation: Designed for small data bursts and can provide lower latency without full RRC connection setup, making it efficient for small, infrequent data transmissions.

- Data Volume:

  - User Plane: Ideal for large data volumes since it supports higher throughput.

  - UL NAS: More appropriate for moderate data volumes, where full control signalling is acceptable.

  - CIoT optimisation: Best for small data payloads, as it avoids the overhead of establishing and maintaining an RRC connection.

- Energy Efficiency:

  - User Plane: Generally, consumes more power due to continuous connection and data transfer.

  - UL NAS: Might offer a balance between power consumption and connectivity requirements.

  - CIoT optimisation: Optimized for low power consumption, making it suitable for battery-operated devices with small data needs.

- QoS Requirements:

  - User Plane: Offers better support for QoS management, ensuring data prioritization and reliability.

  - UL NAS: May not provide the same level of QoS granularity as the User Plane but can still ensure reliable delivery.

  - CIoT optimisation: Limited QoS capabilities but can be sufficient for non-critical small data transmissions.

Considering the diversity of AIoT deployment scenarios, it is envisaged that all three mechanisms mentioned above would need to be supported and selectively used, depending on several factors including the application requirements and/or the network conditions.

This solution proposes a mechanism to select and indicate which forwarding mechanism should be used by the UE reader.

### 6.x.2	Procedures

Figure 6.x.3-1: Information Flow for AIoT communication optimization

- When a UE registers with the 5GC, it provides its capability, indicating which AIoT data forwarding (UL) methods (e.g., UP, UL NAS and/or CIoT optimisation) it supports.

- An AF sends an AIoT service (e.g., inventory or command) request message to the 5GC. This message may contain the AIoT service requirements such as maximum latency, reliability level, energy level etc.

- The AIoTF may select the AIoT data forwarding method to be used be the UE reader based on the AIoT service requirements given in step 2 as well as  the UE capability indicated in step 1.

4a.	The AIoTF may send a AIoT session request message to the gNB over N2 interface, indicating the selected AIoT data forwarding method.

4b.	The AIoTF may send a AIoT session request message to the UE over N1 interface, indicating the selected AIoT data forwarding method.

5. 	As an alternative to step 3, the AIoTF may not select the AIoT data forwarding method, but only provide the necessary information, including the UE capability and the AIoT service requirements, to the gNB.

6. 	The gNB selects the AIoT data forwarding method to be used be the UE reader based on the information given in step 5.

7. 	The gNB may send a AIoT session request message to the UE, indicating the selected AIoT data forwarding method.

8a.	When the UP method is selected, the UE establishes a PDU session targeting to the AIoTF (or directly DN), using the existing mechanisms and forwards the AIoT data.

8b. When the UL NAS method is selected, the UE forwards the AIoT data to the AMF, which then deliver the message to the AIoTF.

8c. When the SDT with CIoT optimisation method is selected, the UE forwards the AIoT data to the gNB in the RRC RESUME request message. The gNB will then further forward the message to the AIoTF over AMF.

### 6.x.3	Impacts on services, entities and interfaces

Entity in 5GC, responsible for AIoT service handling (e.g., AIoTF) :

- Supports selecting the AIoT data forwarding methods or providing the relevant information to the gNB.

AIoT UE reader:

- Supports indicating its capability for AIoT data forwarding methods

RAN node:

- Supports selecting the AIoT data forwarding methods and indicating the result to the AIoT UE reader

END OF CHANGES