3GPP TSG-SA WG2 Meeting #16z4 	S2-2408255

# Maastricht, NL, August 19 – 23, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1159 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical Federated Learning Support if AF is VFL Server with multiple NWDAFs in TS 23.288. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on several principles listed in conclusion part in clause 8.2 of TR 23.700-84, the proposal specifies the procedure of Vertical Federated Learning among NWDAF(s) and/or AF in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the general procedure of Vertical Federated Learning among NWDAF(s) and/or AF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for Vertical Federated Learning will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE(All new text)

## 6.X	Vertical Federated Learning

### 6.X.1	Description

This clause specifies how NWDAF and AF can leverage Vertical Federated Learning technique.

### 6.X.2	Procedure

#### 6.X.2.3	General procedure for Vertical Federated Learning when AF is VFL Server with multiple NWDAFs

Figure 6.x.2.1-1 VFL when AF is VFL Server with multiple NWDAFs

This clause describes an example that VFL Server AF involves multiple VFL Client NWDAFs by combining VFL procedures of NWDAF as VFL server and that of AF as VFL Server. The VFL Server AF and a VFL Client NWDAF are involved in a VFL process, and the VFL Client NWDAF triggers another VFL process involving multiple NWDAFs.

Steps 1-7 are the NWDAF Discovery and sample alignment procedure.

1-2.  VFL Server AF determines to trigger VFL model training, discovers a VFL Client NWDAF, and sends sample alignment query to the VFL Client NWDAF, including a VFL correlation ID (VFL correlation ID#1).

3.	The VFL Client NWDAF determines to initiate another VFL process based on operator's configuration.

4-6. The VFL Client NWDAF discovers NWDAF (s) as VFL Client (s) and performs sample alignment, where the VFL Client NWDAF assigns another VFL correlation ID (VFL correlation ID#2).

6-7.  The VFL Server AF receives the sample availability response and determines the final sample as in clause 6.X.2.4.

Steps 8-15 are the VFL model training procedure.

8. VFL Server AF sends VFL training requests to the VFL Client NWDAF, including VFL correlation ID (i.e., VFL correlation ID#1).

9-10. The VFL Client NWDAF initiates another VFL model training procedure and sends VFL model training requests to the NWDAF (s) selected in Step 3, including VFL correlation ID (VFL correlation ID#2).

11-12. The VFL Server AF receives intermediate results from the VFL Client NWDAF and replies other intermediate results to the VFL Client NWDAF.

13. The VFL Client NWDAF sends other intermediate results to the NWDAF(s).

Steps 16-22 are the VFL inference procedure.

16-19. The VFL Server AF triggers the VFL inference phase based on the internal logic and sends a sample availability query to the VFL Client NWDAF, including the VFL correlation ID (i.e., VFL correlation ID #1). The VFL Client NWDAF triggers the sample alignment procedure, which is associated with another VFL correlation ID (VFL correlation ID#2). Then, the VFL Client NWDAF sends the available sample to VFL Server AF.

20. The VFL Server AF sends the VFL inference request to the VFL Client NWDAF, including the VFL correlation ID (i.e., VFL correlation ID #1).

21-22. The VFL Client NWDAF gathers intermediate results via another VFL inference process associated with VFL correlation ID (VFL correlation ID#2) and reports intermediate results to the VFL Server AF.

## END of CHANGES